| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G185 |
| --- | --- |

| Title: | Non-CE6: On intra chroma mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toru Kumakura Shigeru FukushimaJVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-5798 kumakura.toru@ jvckenwood.com fukushima.shigeru@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In the 6th JCT-VC meeting, a technique to improve the chroma intra prediction mode coding is proposed in JCTVC-F459. Orthogonal direction and neighboring directions for intra prediction mode are candidates of chroma intra prediction mode instead of vertical mode, horizontal mode and DC mode in the technique. This proposal reports the two techniques to improve coding efficiency, the one is partition shape based DM/LM switching, the other is exclusive vertical/horizontal mode. The simulation results report that the proposed technique provides 0.5-0.7% BD-rate gain with no complexity increase in all intra settings.

# Introduction

In the current HM4.0, intra chroma prediction modes are derived by using intra luma prediction modes of the same position. In the 6th JCT-VC meeting, a technique to improve the chroma intra prediction mode coding is proposed in [1]. Orthogonal direction and neighboring directions for intra prediction mode are candidates of chroma intra prediction mode instead of vertical mode, horizontal mode and DC mode in the technique. This proposal reports the two techniques to improve coding efficiency, the one is partition shape based DM/LM switching, the other is exclusive vertical/horizontal mode.

# Algorithm Description

## Current algorithm (HM4.0)

Table 1 presents intra_chroma_pred_mode and the bit string in the HM4.0, in case that chroma_pred_from_luma_enabled_flag is equal to 1. DM mode, that is mapped to 1 bit string, is 1st prior and LM mode, that is mapped to 2 bit string, is 2nd prior. The other 4 modes are same with first 4 intra prediction modes for luma.

Table 1 Code mapping of chroma mode in current HM 4.0

| intra_chroma_pred_mode (meaning) | Bit(bin) string | IntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | X ( 0 <= X < 35 ) |
| 5 (DM) | "0" | 0 | 1 | 2 | 3 | X |
| 4 (LM) | "10" | LM | LM | LM | LM | LM |
| 0 (planar) | "110" | 7 | 0 | 0 | 0 | 0 |
| 1 (vertical) | "1110" | 1 | 7 | 0 | 0 | 1 |
| 2 (horizontal) | "11110" | 2 | 2 | 7 | 1 | 2 |
| 3 (DC) | "11111" | 3 | 3 | 3 | 7 | 3 |

DM mode means that the chroma intra prediction mode is inherited by the luma intra prediction mode at the same position. One chroma_intra_pred_mode is sent by one luma intra prediction mode for coding unit with Part_2Nx2N (Figure 1 (a)), on the other hand one chroma_intra_pred_mode is sent by four luma intra prediction modes for coding unit with Part_NxN (Figure 1 (b)). For coding unit with Part_NxN, the referred luma block to decode chroma_intra_pred_mode is top-left one in the coding unit.

Figure 1 Connection between luma block and chroma block

## Proposed algorithm

### Proposal 1: Partition shape based DM/LM switching

One intra_chroma_pred_mode is sent for 4 luma blocks and top-left luma block is always referred in coding unit with Part_NxN as in the HM4.0. The effectiveness of DM mode for Part_NxN is lower than for Part_2Nx2N.

It is proposed to switch the order of DM mode and LM mode according to the partition shape. Table 2 presents the mode mapping of proposed algorithm and Figure 2 illustrates the flowchart of proposed algorithm.

Table 2 Mode mapping of proposed algorithm

| intra_chroma_pred_mode(meaning) | Bit(bin) string |  |
| --- | --- | --- |
|  | NxN | 2Nx2N |
| 5 (DM) | "10" | "0" |
| 4 (LM) | "0" | "10" |

Figure 2 Flowchart of proposal 1

The partition size checking is added from HM4.0 to switchDM/LM mode. It is invoked in decoding process not but pursing process. Thus decoder throughput is not increased.

### Proposal 2: Exclusive vertical/horizontal mode

In the 6th JCT-VC meeting, a technique to improve the chroma intra prediction mode coding is proposed [1]. The technique is based on inheritance from luma intra prediction mode. Orthogonal direction and neighboring directions for intra prediction mode are candidates of chroma intra prediction mode instead of vertical mode, horizontal mode and DC mode. Orthogonal direction and neighboring directions for intra prediction mode are derived by referring additional tables. It brings chroma BD-rate gain.

Exclusive vertical/horizontal mode, which is replaced with the orthogonal direction, is proposed in thiscontribution.

Deriving exclusive vertical/horizontal mode, following applies:

- If IntraPredMode == 0 (planar), VerHorMode = DC

- Otherwise, if IntraPredMode == 3 (DC), VerHorMode = 7

- Otherwise, if intraPredOrder[IntraPredMode] < 18, VerHorMode = 2 (horizontal)

- Otherwise(intraPredOrder[IntraPredMode] >=18),  VerHorMode = 1 (vertical)

## Implementation

### Software

This proposed technique is implemented into HM4.0 software.

### Syntax structure

The syntax of this proposed technique is not revised from HM4.0.

# Experiments

## Test conditions

The common test conditions [3] are used.

## Experimental environment

The experimental environment used is presented in Table 3.

Table 3 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 4, Table 5 and Table 6 present the simulation results of proposal 1, proposal 2 and proposal3 for intra only, respectively.

Table 4 Simulation results of proposal 1 for intra only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | 0.0% | 0.0% | -0.2% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.2% | -0.1% | 0.0% | -0.2% | -0.1% |
| Class E | 0.0% | -0.4% | -0.7% | 0.0% | -0.2% | -0.1% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 5 Simulation results of proposal 2 for intra only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class B | 0.0% | -0.5% | -0.6% | 0.0% | -0.5% | -0.6% |
| Class C | 0.0% | -0.6% | -0.6% | -0.1% | -0.7% | -0.6% |
| Class D | 0.0% | -0.5% | -0.7% | -0.1% | -0.5% | -0.6% |
| Class E | 0.0% | -0.4% | -0.6% | 0.0% | -0.6% | -0.6% |
| Overall | 0.0% | -0.4% | -0.5% | 0.0% | -0.5% | -0.5% |
|  | 0.0% | -0.4% | -0.5% | 0.0% | -0.5% | -0.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 6 Simulation results of combination of proposal 1 and proposal 2 for intra only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.2% | -0.1% | 0.0% | -0.4% | -0.3% |
| Class B | 0.0% | -0.4% | -0.6% | 0.0% | -0.7% | -0.7% |
| Class C | -0.1% | -0.6% | -0.6% | -0.1% | -0.9% | -0.8% |
| Class D | -0.1% | -0.7% | -0.7% | -0.1% | -0.7% | -0.8% |
| Class E | 0.1% | -0.9% | -1.3% | 0.0% | -0.9% | -0.8% |
| Overall | 0.0% | -0.5% | -0.6% | -0.1% | -0.7% | -0.7% |
|  | 0.0% | -0.5% | -0.6% | -0.1% | -0.7% | -0.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

The simulation results report that the proposed technique provides 0.5-0.7% BD-rate [3] gain with no complexity increase. (See the attached excel sheet)

# Conclusions

Therefore, we suggest that the proposed technique be adopted to the next WD and HM.

# References

[1] “Parsing friendly intra mode coding”,

JCTVC-F459, Torino, July 2011.

[2] “WD4: Working Draft 4 of High-Efficiency Video Coding”,

JCTVC-F803, Torino, July 2011

[3] “Common test conditions and software reference configurations”,

JCTVC-F900, Torino, July 2011

[4] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. or JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

8.3.2.	Derivation process for chroma intra prediction mode

Input to this process is a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture.

Output of this process is the variable IntraPredModeC.

The chroma intra prediction mode IntraPredModeC is derived as specifed in Table 8-7 or Table 8-7 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 1 with intra_chroma_pred_mode, IntraPredMode[ xB ][ yB ] and chroma_pred_from_luma_enabled_flag as inputs.

Table 8-7 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 1

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 0 <= X < 35 ) |
| 0 | 7 | 0 | 0 | 0 | 0 |
| 1 | 1 | 7 | 0 | 0 | 1 |
| 2 | 2 | 2 | 7 | 1 | 2 |
| 3 | 3 | 3 | 3 | 7 | 3 |
| 4 | LM | LM | LM | LM | LM |
| 5 | 0 | 1 | 2 | 3 | X |

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 0 <= X < 35 ) |
| 0 | 7 | 0 | 0 | 0 | 0 |
| 1 | Invoked sub clause 8.3.2.1 |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 | Invoked sub clause 8.3.2.2 |  |  |  |  |
| 4 | Invoked sub clause 8.3.2.3 |  |  |  |  |
| 5 | Invoked sub clause 8.3.2.4 |  |  |  |  |

8.3.2.1 IntraPredModeC derivation process for intra_chroma_pred_mode is equal to 1 or 2

If chroma_intra_pred_mode is equal to 1 or 2, IntraPredModeC is derived as follows:

If partSize == Part_NxN, IntraPredModeC = Neighbor18[chroma_intra_pred_mode-1][IntraPredMode]

else, IntraPredModeC = Neighbor35[chroma_intra_pred_mode-1][IntraPredMode]

Table 8-5 Specification of Neighbor18

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 12 | 17 | 1 | 15 | 11 | 13 | 14 | 16 |
| 1 | 2 | 13 | 16 | 2 | 11 | 12 | 14 | 10 | 15 |

| IntraPredMode | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 10 | 7 | 4 | 5 | 1 | 6 | 8 | 2 | 9 |
| 1 | 17 | 9 | 5 | 1 | 6 | 7 | 4 | 8 | 2 |

Table 8-6 Specification of Neighbor35

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 22 | 31 | 1 | 27 | 20 | 24 | 26 | 29 |
| 1 | 2 | 23 | 30 | 2 | 19 | 21 | 25 | 10 | 28 |

| IntraPredMode | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 33 | 7 | 19 | 21 | 23 | 25 | 28 | 30 | 32 |
| 1 | 32 | 34 | 20 | 22 | 24 | 26 | 27 | 29 | 31 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 34 | 4 | 11 | 5 | 12 | 1 | 13 | 6 | 14 |
| 1 | 33 | 11 | 5 | 12 | 1 | 13 | 6 | 14 | 7 |

| IntraPredMode | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 15 | 8 | 16 | 2 | 17 | 9 | 18 | 10 |
| 1 | 4 | 15 | 8 | 16 | 2 | 17 | 9 | 18 |

8.3.2.2 IntraPredModeC derivation process for intra_chroma_pred_mode is equal to 3

If chroma_intra_pred_mode is equal to 3, IntraPredModeC is derived as follows:

- If IntraPredMode == 0 (planar), IntraPredModeC  = DC

- Otherwise, if IntraPredMode == 3 (DC), IntraPredModeC = 7

- Otherwise, if intraPredOrder[IntraPredMode] < 18, IntraPredModeC = 2

- Otherwise, (intraPredOrder[IntraPredMode] >=18) IntraPredModeC = 1

8.3.2.3 IntraPredModeC derivation process for intra_chroma_pred_mode is equal to 4

If chroma_intra_pred_mode is equal to 4, IntraPredModeC is derived as follows:

- If partSize == Part_NxN, IntraPredModeC = intraPredMode

- Otherwise, IntraPredModeC = LMmode

8.3.2.4 IntraPredModeC derivation process for intra_chroma_pred_mode is equal to 5

If chroma_intra_pred_mode is equal to 5, IntraPredModeC is derived as follows:

- If partSize == Part_NxN, IntraPredModeC = LMmode

- Otherwise, IntraPredModeC = intraPredMode