| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G186 |
| --- | --- |

| Title: | CE6: Cross-verification of MediaTek and Qualcomm's proposal (JCTVC-G203) by JVC KENWOOD |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Toru Kumakura Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-5798 kumakura.toru@ jvckenwood.com fukushima.shigeru@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of performance of CE6.b JCTVC-G203 by MediaTek and Qualcomm. It is confirmed that the BD-rate results match those provided by the proponent. The simulation results report that the proposed technique provides 0.4% BD-rate gain for intra only settings.

# Introduction

This contribution reports cross-verification results of performance of CE6.b JCTVC-G203 by MediaTek and Qualcomm [1].

# Algorithm Description

MPRM

Four most probable modes are defined. First two modes are same as MPM in HM4.0. Remaining two modes (MPRMs) are constructed from neighboring directions of referring intra prediction mode and sent as 2nd level most probable mode. Look-up tables, which are used to derive neighboring directions, are added.

MPRM_RDO

RD optimization for MPRM is enabled in the encoder.

Chroma mode coding

Code mapping of intra_chroma_pred_mode is modified from HM4.0. Orthogonal direction and neighboring directions for intra prediction mode are candidates of chroma intra prediction mode instead of vertical mode, horizontal mode and DC mode. Look-up tables, which are used to derive orthogonal direction, are added. Look-up tables to derive neighboring directions are same as the table which is defined for MPRM.

19 mode candidates in 4x4 PU

Mode 18 is enabled in 4x4 PU.

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2 presents the simulation results for intra only.

Table 2 Simulation results for intra only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.35% | -0.01% | -0.05% | -0.29% | -0.28% | -0.25% |
| Class B | -0.34% | -0.38% | -0.46% | -0.37% | -0.69% | -0.69% |
| Class C | -0.38% | -0.66% | -0.73% | -0.47% | -0.97% | -0.92% |
| Class D | -0.34% | -0.57% | -0.64% | -0.41% | -0.65% | -0.78% |
| Class E | -0.63% | -0.30% | -0.46% | -0.60% | -0.79% | -0.83% |
| Overall | -0.39% | -0.39% | -0.47% | -0.42% | -0.67% | -0.69% |
|  | -0.39% | -0.39% | -0.48% | -0.42% | -0.67% | -0.69% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

The simulation results report that the proposed technique provides 0.4% BD-rate [3] gain for intra only settings. (See the attached excel sheet)

# Conclusions

It is confirmed that substantial coding gain can be obtained by MPRM and proposed chroma mode coding. Neighboring directions of referring intra prediction mode for MPRM can be obtained by look-up tables. Furthermore, identical look-up tables can also be used for deriving chroma prediction modes.

We think that this proposed technique has better trade-off point of coding gain and complexity.

# References

[1] “CE6b: Intra prediction mode coding”,

JCTVC-G203, Geneva, November 2011

[2] “Common test conditions and software reference configurations”,

JCTVC-F900, Torino, July 2011

[3] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. or JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).