| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G188 |
| --- | --- |

| Title: | Enhancement on operation of coded picture buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Satoshi Shimada, Junpei Koyama and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com sshimada@jp.fujitsu.com koyama.junpei@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution proposes to enhance the operation of the coded picture buffer (CPB).

The specification of CPB operation in WD4, inherited provisionally from the AVC specification, only defines the picture based timing of bitstream operation.

The proposed operation is the sub-picture based bitstream removal from CPB in order to realize interoperable very low delay coding less than 1 picture period.

# Background

There are huge demands for real-time video transmission. In the broadcasting industry, very low delay operation less than 30ms - 100ms is required for transmission of professional video materials via a satellite, a terrestrial, or an IP channel[3].

The existing standards such as AVC, however, do not provide the method for very low delay operation in an interoperable way. That is, only the picture based removal time is defined in the specification of the hypothetical reference decoder (HRD).

AVC-based commercial video transmission apparatus with very low delay operation use vender's proprietary protocol between an encoder and a decoder so that the encoder and the decoder work jointly for lowering codec delay less than 1 picture period. Apparently interoperable very low delay operation cannot be possible with AVC.

The authors have been proposing new operation of CPB in the following past contributions in order to realize interoperable very low delay video transmission. However, no action nor discussion has been taken regarding this proposal due to the fact that the issue of HRD has lower priority at this standardization stage.

- JCTVC-C021 [1]

- JCTVC-D053 [2]

The authors think JCT-VC should accelerate the discussion on high-level syntax such as HRD toward publishing CD text at the 8th meeting in February 2012. In this regard, the authors contribute the proposal again in this meeting in order to draw the participants' attention to the HRD specification in HEVC.

In the following, the proposed operation of CPB is described.

# Proposal

## Introduction

The hypothetical reference decoder (HRD) of MPEG-4 AVC/H.264 defines the picture based operation of the coded picture buffer (CPB).

In the specification of the HRD, the arrival time of the last bit of each treeblock is unknown. The unknown arrival time of the last bit of each treeblock is not a problem so long as the HRD is used in the ideal world because the HRD decodes a picture instantaneously. But in the real world, a decoder normally decodes a picture within one frame period. In the worst case, the last bit of the first treeblock arrives at the removal time of a picture. In this case, the display timing of the picture should be delayed the decoding duration of the picture (hereafter referred to as dt).

When the frame rate is constant, the presentation timing of each picture should be equal to that of the picture in the worst case. As the result, the coding delay exceeds the decoding duration of a picture which is almost equal to one frame period.

Figure 1  Buffer model in conventional standards

Very low delay coding realized by the vertical intra line scheme [2] can average the number of bits of each treeblock line. In another word, the interval of the arrival of the bits of each treeblock line is averaged. Therefore the coding delay of less than one frame period is realized by introducing the “set of treeblocks” based operation of the CPB in the HRD specification of HEVC.

Specifically, a picture is divided into M groups. The unit of the division is, for example, one treeblock line. Each group contains the same number of treeblocks. The value M is signaled in Buffering period SEI.

The new decoder model is also introduced in the specification of the hypothetical decoder. In the new decoder model, the decoding process is invoked at each group and the removal time of each group from the CPB is defined. An encoder is required to set the arrival time of the last bit of each group earlier than the removal time of each group.

The removal time of the i-th group in the n-th picture, tr( n, i ) is defined as

tr( n, i ) = tr ( n - 1 ) + ( tr ( n ) - tr ( n - 1 ) ) * i / M

, where tr( n ) is the removal time of the n-th picture. tr( n, i ) is expressed in the unit of time_scale.

Figure 2  New buffer model for very low delay coding

The decoding period of each group becomes (dt / M). Thus the new buffer model reduces the delay caused by the decoding process to 1/M of that in the conventional buffer model. For example, the coding delay of a HDTV sequence (1920x1080/60Hz) with 64x64 treeblock is about 1 millisecond.

## Syntax and semantics

### Buffering period SEI message syntax

| buffering_period( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_id | 5 | ue(v) |
| if( NalHrdBpPresentFlag ) { |  |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | 5 | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | 5 | u(v) |
| } |  |  |
| } |  |  |
| if( VclHrdBpPresentFlag ) { |  |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | 5 | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | 5 | u(v) |
| } |  |  |
| } |  |  |
| if( NalHrdBpPresentFlag ¦¦ VclHrdBpPresentFlag) { |  |  |
| num_groups_in_picture_minus1 | 5 | ue(v) |
| } |  |  |
| } |  |  |

### Buffering period SEI message semantics

num_groups_in_picture_minus1 plus 1 specifies the number of the group in a picture in the buffer model of the hypothetical reference decoder. num_groups_in_picture_minus1 shall satisfies the following equation.

(PicWidthInLCUs * PicHeightInLCUs) %  (num_groups_in_picture_minus1 + 1) = 0

## Description of the new buffer model

### Timing of coded group of treeblocks removal

A group of treeblocks g( m ) is the m-th group in a coded picture. The number of groups of treeblocks is equal to ( num_groups_in_picture_minus1 + 1 ).

Each group of treeblocks contains equal number of treeblocks NumLCUs. NumLCUs is equal to ( PicSizeInLCUs / ( num_groups_in_picture_minus1 + 1 ) ). g( m ) contains treeblocks whose addresses are from (NumLCUs * m) to ( (NumLCUs * (m + 1) - 1 )).

The nominal removal time of the m-th group of treeblocks in the n-th access unit, tr,n( n, m ), is defined as follows.

tr,n ( n, m ) = tr,n ( n - 1 ) + (tr,n ( n ) - tr,n ( n - 1 ) ) * m / ( num_groups_in_picture_minus1 + 1 )

The last bit of coded g( m ) shall arrive at the CPB of a decoder before tr,n ( n, m ).

### Definition of last bit of coded group of treeblocks

The last bit of a coded group of treeblocks is not obvious because the boundary of a group of treeblocks does not always coincide with the boundary of a slice. The last bit of a coded group of treeblocks is defined in the following subclauses.

#### CAVLC case

When entropy_coding_mode_flag is equal to 0, the last bit of a coded group of treeblocks is defined as follows.

- When a coded group of treeblocks is the last coded group of treeblocks in a picture, the first bit of rbps_slice_trailing_bits() which follows the coded group of treeblocks is the last bit of the coded group of treeblocks.

- Otherwise, when there is a slice_header() syntax structure between a coded group of treeblocks and the next coded group of treeblocks, the first bit of rbps_slice_trailing_bits() which follows the coded group of treeblocks is the last bit of the coded group of treeblocks.

- Otherwise, the last bit of the last coded treeblock in a coded group of treeblocks is the last bit of the coded group of treeblocks.

Note: bits belongs to a coded treeblock in a bitstream are from the bit corresponding to split_coding_unit_flag of depth 0 of the treeblock, to the bit just before the bit corresponding to split_coding_unit_flag of depth 0 of the next treeblock.

#### CABAC case

When entropy_coding_mode_flag is equal to 1, the last bit of a coded group of treeblocks is defined as follows.

- The last bit of bit sequence which corresponds to end_of_slice_flag of the last coded treeblock in a coded group of treeblocks is the last bit of the coded group of treeblocks.

In the case of CABAC, there is no one-to-one mapping between a bitstream and a sequence of bins as the output of the CABAC decoding process. One single bit in a bitstream may correspond to more than two bins, and vice versa.

The last bit in a bitstream which corresponds to bin X is defined as follows.

- When bin X is reconstructed through the arithmetic decoding process of bit A in a bitstream, then bit A is the last bit in a bitstream which corresponds to bin X.

The figure below illustratively explains the definition the last bit in a bitstream which corresponds to a bin X.

A black circle in the figure show the relationship between a bit in a bitstream and a bin as the output of the arithmetic decoding process of the bitstream. For example, the first bin is generated as the result of the arithmetic decoding process of the first bit and the second bit in the bitstream.

As shown in the figure, the last bin (i.e. a bin corresponding to end_of_slice_flag ) of the 0-th treeblock is generated as the result of the arithmetic decoding process of the third bit in the bitstream. That is, the last bit of the 0-th coded treeblock is the third bit in the bitstream. Similarly, the last bins of the first treeblock, the second treeblock, and the third treeblock are generated as the result of the arithmetic decoding process of the 11th bit in the bitstream. That is, the last bit of the first coded treeblock, the last bit of the second coded treeblock, and the last bit of the third coded treeblock are the 11th bit in the bitstream.

Fig. X-X Illustrated explanation of the relationship between last bin of a treeblock and corresponding bits in a bitstream (CABAC case)

# Conclusion

Interoperable very low delay operation less than 30ms is required from various industries such as broadcasting. The authors believes JCT-VC should establish an AHG in order accelerate the discussion on HRD specification that meets industries' requirement. The authors also think the proposed scheme is a good starting point for further discussion.

# References

[1] K.Kazui, J.Koyama, and A.Nakagawa, "Evaluation result of JCTVC-B031", JCTVC-C021, October 2010, Guangzhou CN

[2] K.Kazui, J.Koyama, and A.Nakagawa, "Draft description of proposed syntax and semantics for very low delay coding", JCTVC-D053, January 2011, Daegu KR

[3] K.Kazui, J.Koyama, and A.Nakagawa, "Industry needs of very low delay coding", JCTVC-F147, July 2011, Torino IT

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).