| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G190 |
| --- | --- |

| Title: | CE9: Cross-verification result of MRG_MVD01 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This document reports cross-verification result of MRG_MVD01 (originally proposed in JCTVC-F373 [1], the report from the proponent is JCTVC-G052) in core experiment 9 [2].

The result is identical to the result reported by the proponent.

# Verified scheme

The purpose of MRG_MVD01 is to evaluate the scheme described in JCTVC-F373 proposed by JVC KENWOOD. Specifically, merge mode is extended so that MV difference is explicitly signaled.

In order to signal MV difference, new syntax merge_mvd_flag is introduced. When merge_mvd_flag is equal to 1, one MV difference is signaled even when inter_pred_mode is equal to Bi-pred.

The difference between MRG_MVD01 and other two tests, MRG_MVD02 and MRG_MVD03, is the following.

- No additional motion estimation is performed in extended merge mode.

- Up to five MV candidates in merge mode are evaluated for extended merge mode.

We confirmed that the algorithm implemented in the source code for MRG_MVD01 is identical to that in the core experiment document.

# Result

Table 1summarizes the coding gain in BD-Rate of the proposed scheme. The test condition accords with the condition specified in JCTVC-F800 [4].

Table 1: Summary results of BD-Rate comparing proposal to HM4.0

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.5% | -0.4% | -0.2% | 0.1% | -0.1% |
| Class B | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
| Class C | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | 0.0% | -0.1% | -0.1% | -0.2% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 105% |  |  | 107% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.3% | 0.2% | -0.3% | -0.1% | -0.2% |
| Class C | -0.3% | -0.3% | -0.4% | -0.3% | -0.1% | -0.4% |
| Class D | -0.3% | 0.0% | 0.4% | -0.3% | 0.2% | -0.3% |
| Class E | -0.2% | -0.1% | 0.2% | -0.4% | 0.3% | -0.2% |
| Overall | -0.2% | -0.2% | 0.1% | -0.3% | 0.0% | -0.3% |
|  | -0.2% | -0.2% | 0.0% | -0.3% | 0.0% | -0.3% |
| Enc Time[%] | 106% |  |  | 106% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

The result is identical to the result reported from the proponent (JVC KENWOOD).

# References

[1] S.Fukushima, M.Ueda, K.Arakage and S.Sakazume, “Merge based mvd transmission”, JCTVC-F373, July 2011, Torino IT

[2] "Core Experiment 9: MV Coding and Skip/Merge Operations", JCTVC-E909, July 2011, Torino IT

[3] "CE9: Results of MRG_MVD series", JCTVC-G052, November 2011, Geneva CH

[4] "Common test conditions and software reference configurations", JCTVC-F900, July 2011, Torino IT