| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G196 |
| --- | --- |

| Title: | Modification of derivation process of motion vector information for interlace format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jumpei Koyama, Akihiro Yamori, Kimihiko Kazui, Satoshi Shimada and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 koyama.junpei@jp.fujitsu.com kazui.kimihiko@jp.fujitsu.com sshimada@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution proposes modification of the derivation process of motion vector information for interlace format. Specifically, the adjustment of the vertical component of a chroma MV for 4:2:0 format adopted in AVC and the adjustment of the vertical component of a luma MV predictor are proposed in order to remove inconsistency between the current derivation process of motion vector information and the nature of interlace format. Proposed modification requires few changes of semantics and no changes of block-level syntax. The proposed modification achieves -1.6% to -3.4% bit-rate reduction in luma component and -11.3% to -24.4% bit-rate reductions in chroma components. In particular, the maximum gain of BD-PSNR in chroma component reaches + 1.0 dB.

# Introduction

HEVC standardization is currently being processed focusing on progressive format only. Nevertheless there still remains a strong need for supporting interlace format since broadcasters hold a vast amount of video contents captured in interlace format. Therefore the authors welcome the disposition of m21411[1] at the last meeting that substantially allows the support of interlace format in HEVC.

Since the standardization process of HEVC is reaching the converging phase, modification to the current draft should be essential to the support of interlace format and be simple enough not to delay the standardization process.

Interlace format has two unique characteristics. One is that the vertical sampling position of chroma component for 4:2:0 is different from that in progressive format. The other is that the vertical position of each line in a top-field picture is not aligned spatially with that of the corresponding line in a bottom-field picture. There is inconsistency between the current derivation process of motion vector information and the nature of interlace format.

# Problem

## Chroma MV misalignment

There exists chroma vector misalignment caused by pixel displacement between top and bottom fields in 4:2:0 format. Figure 1 depicts the inconsistency. When a luma motion vector of a prediction unit in a top field picture points to somewhere in a bottom field picture, the corresponding chroma motion vector points to 0.25 pixel below the correct position. Similarly, when a luma motion vector of a prediction unit

Figure 1 Inconsistency on chroma motion vector in 4:2:0 format

in a bottom field picture points to somewhere in a top field picture, the corresponding chroma motion vector points to 0.25 pixel above the correct position.

## Luma MV misalignment in Merge and AMVP mode

Another misalignment happens in deriving the vertical component of a predicted motion vector in Merge and AMVP mode due to pixel displacement between a top field and a bottom field.

Figure 2 shows an example of the inconsistency. In Figure 2, it is assumed that the reference picture of the current prediction unit in the top-field picture C is the bottom-field picture B, and the reference picture of the collocated prediction unit in the bottom-field picture D is the top-field picture A. When the vertical component of the motion vector of the collocated prediction unit, MVPyCol, is equal to -1, the vertical component of the predicted motion vector for the current prediction unit, MVPyscaled, is derived as -1/3. Since the transition of an object is assumed linear in the scaling process, MVPyscaled should be parallel to MVPyCol. However, as depicted in Figure 2, derived MVPyscaled is not parallel to MVPyCol. Thus it results in loss of coding efficiency.

Figure 2 Inconsistency on derivation process of motion vector prediction

# Proposed modification

## Modification on derivation process of chroma motion vector in interlace format

The same chroma MV adjustment as in MPEG-4 AVC/H.264[6] is introduced. When a luma motion vector points to a bottom field from a top field, the vertical component of its corresponding chroma motion vector is shifted -2 in units of octet-pixel so that it points to the correct position. Similarly, when a luma motion vector points to a top field from a bottom field, the vertical component of its corresponding chroma motion is shifted 2 in units of octet-pixel.

MVychroma=MVyLuma2−14 when the luma motion vector points to a bottom field from a top field.

MVychroma=MVyLuma2+14 when the luma motion vector points to a top field from a bottom field.

Figure 3 shows an example. The shifting adjusts the vertical component of the chroma motion vectors to the correct position.

Figure 3 Adjustment of chroma vector in 4:2:0 format

## Modification on derivation process of motion vector prediction in interlace format

The derivation process of a predicted motion vector is modified according to parity of pictures. The detail is the following.

- Firstly, the vertical component of a motion vector in a spatially or temporally adjacent prediction unit (selected according to the current draft), MVPy, is "normalized" as MVPyshifted, which is the corresponding vertical component when the origin and the destination of the motion vector have the same parity.
MVPyshifted=MVPy+2∗isBottomNeighRef−isBottomNeigh
The variables isBottomNeigh and isBottomNeighRef are equal to 1 when the origin of MVPy is a bottom-field picture, and when the destination of MPVy is a bottom-field picture, respectively.

- Then the "normalized" motion vector MVP' = (MVPx, MVPyshifted) is scaled according to the derivation process as defined in WD4. The resulting scaled motion vector is MVPscaled = (MVPxscaled, MVPyscaled).

- Finally the vertical component of the scaled motion vector MVPscaled is inversely normalized according to the parity of the encoding picture and the reference picture. The resulting motion vector MVPcorrect = (MVPxscaled, MVPycorrect) is added to the AMVP candidates list or to the merge candidates list.
MVPycorrect=MVPyscaled+2∗isBottomCurr−isBottomCurrRef
The variables isBottomCurr and isBottomCurrRef are equal to 1 when the current picture is a  bottom-field picture, and when the reference picture is a bottom-field picture, respectively.

This modification enables a predicted motion vector to point to a correct position and compensates loss of coding efficiency. Figure 4 shows an example.

Figure 4 Proposed modification on derivation process of motion vector prediction

# Test condition

Experiments are conducted on HM-4.0 software. One field picture in interlaced sequences is considered as one frame and therefore interlaced sequences are encoded as sequences with half height and doubled frame rate on the software. Field parity is automatically derived by POC, an even POC number represents a top field and an odd POC number represents a bottom field.

We use four SD (720x480/60i, hereafter referred to as Class C') sequences (#15 "Flower garden", #29 "Table tennis", #30 "Mobile and calendar" and #38 "Football") and four HD (1920x1080/60i, hereafter referred to as Class B') sequences (#8 "Walk through the square", #13 "Church", #16 "Whale show" and #22 "Marching in") specified in ITU-R BT.1120[5] and encode those sequences under the common condition described in F900[3]. "Random access" configurations and "Low delay" configurations are applied.

# Results

Table 1 and Table 2 summarize coding gain of the proposed scheme over HM4 in BD-Rate and in BD-PSNR, respectively. Coding gains of each sequence are described in the attached Excel sheets.

Table 1 Results of BD-Rate comparing Proposal to HM4.0

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class B' | -0.8 | -13.6 | -18.0 | -0.8 | -13.2 | -17.2 |
| Class C' | -1.6 | -24.4 | -17.8 | -1.8 | -24.0 | -18.6 |
| Overall | -1.1 | -18.5 | -17.9 | -1.3 | -18.1 | -17.8 |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class B' | -2.2 | -9.8 | -12.0 | -0.3 | -8.6 | -10.5 |
| Class C' | -3.1 | -14.1 | -11.3 | -3.4 | -13.8 | -11.3 |
| Overall | -2.6 | -11.8 | -11.7 | -2.9 | -10.2 | -9.8 |

Table 2 Results of BD-PSNR comparing Proposal to HM4.0

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class B' | 0.020 | 0.250 | 0.383 | 0.021 | 0.229 | 0.340 |
| Class C' | 0.068 | 0.697 | 0.501 | 0.075 | 0.641 | 0.479 |
| Overall | 0.042 | 0.454 | 0.437 | 0.045 | 0.416 | 0.404 |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class B' | 0.064 | 0.166 | 0.231 | 0.071 | 0.121 | 0.171 |
| Class C' | 0.140 | 0.407 | 0.317 | 0.149 | 0.376 | 0.297 |
| Overall | 0.099 | 0.276 | 0.270 | 0.106 | 0.237 | 0.229 |

The average coding gains for Class B' and Class C' sequences (1080 60i) are -1.1% (RA_HE), -1.3% (RA_LC), -2.6% (LD_HE) and -2.9% (LD_LC) for luma component, and for chroma (Cb) component the average coding gains for Class B' and Class C' sequences (1080 60i) are -18.5% (RA_HE), -18.1% (RA_LC), -11.8% (LD_HE) and -10.2% (LD_LC). Particularly, the chroma coding gains of the proposed scheme for "Flower garden" sequence and "Mobile and calendar" sequence are up to 1.0 dB in "Random access high efficiency" configuration.

# Conclusion

This contribution proposes modification on the derivation process of a motion vector information in interlace format. From -1.6 to -3.4% bit-rate reduction in luma component and from -11.3% to -24.4% bit-rate reductions in chroma components are achieved with the proposed MV adjustments. In particular, the maximum improvement in chroma component reaches +1.0 dB. Proposed modification requires few changes of semantics and no changes of block-level syntax.

We propose to establish an AHG with mandates of collecting interlace sequences for further evaluation and analyzing proposed schemes related to interlace format at the 7th meeting.

# References

[1] S.Sekiguchi at el, "Joint proposal of HEVC requirement on the support of interlace format", ISO/IEC JTC1/SC29/WG11/M21411, July 2011, Torino IT

[2] "WD4: Working Draft of High-Efficiency Video Coding", JCTVC-F803, July 2011, Torino IT

[3] "Common test conditions and software reference configurations", JCTVC-F900, July 2011, Torino IT

[4] S. Sekiguchi, K. Sugimoto and H. Sakate, "Proposal on the support of interlace format in HEVC ", JCTVC-F194, July 2011, Torino IT

[5] Rec. ITU-R BT.1210-3, "Test materials to be used in subjective assessment"

[6] A.Nakagawa and H.Miyoshi, "Definition of chroma motion vector in field prediction between different parities", JVT-E088, October 2002, Geneva CH

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).