| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G0197 |
| --- | --- |

| Title: | AHG4: Low latency CABAC initialization for dependent tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, chiayang.tsai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Dependent tiles are processed using tile structure scan order and are allowed to use data from neighboring tiles. The CABAC inheritance also follows that order, which prevents low latency processing. In this contribution, low latency CABAC initialization for dependent tiles is proposed, where the CABAC probabilities of the first LCU in each tile are inherited from those of the adjacent left LCU. The proposed method is compared with dependent tiles of HM-4.0-dev. Simulation results reportedly show that the proposed method causes 0-0.2% and 0-0.7% bit rate increases when the number of tile columns is two and four, respectively. The processing latency for non-first column tiles is reduced from almost one picture to at most one LCU row.

# Introduction

In last 6th JCT-VC meeting, JCTVC-F335[1] for tiles was adopted and added into HM-4.1. Figure 1 shows an example of partitioning one picture into multiple tiles. Inside each tile, LCUs are raster scanned. Inside each picture, tiles are raster scanned. Tile boundaries must be LCU boundaries. Slices and tiles are orthogonal. Tiles can be regarded as a change of LCU scan order, and slices have to follow the modified LCU scan order. Therefore, one slice may cross a tile boundary, and one tile may contain multiple slices.

There are two kinds of tiles: independent tiles and dependent tiles. For independent tiles, encoding and decoding LCUs (e.g. MV prediction, intra prediction, entropy coding) of one tile do not need data from any other tile. Moreover, when CABAC is used as entropy coding method, CABAC probabilities will be re-initialized at the first LCU of each tile to ensure that tiles can be processed independently. For example, as shown in Figure 1, CABAC probabilities for LCU #13, #31, #40 will be re-initialized.

For dependent tiles, tiles can be regarded as a change of LCU scan order. Encoding and decoding a dependent tile can use data from neighboring tiles as long as they have been processed and are available. Moreover, when CABAC is used as the entropy coding method, CABAC probabilities will be updated following the tile structure scan order and will not be re-initialized. For example, as indicated by arrows in Figure 2, CABAC probabilities for LCU #13 will inherit from those of LCU #12, CABAC probabilities for LCU #31 will inherit from those of LCU #30, and so on.

For practical applications, the CABAC inheritance property for dependent tiles described in the previous section will introduce some problems. First, for low latency applications, where the input source for encoder and the display output for decoder always follow the picture based raster scan order, it is desirable to have LCUs in one row to be processed as soon as possible. With tile structure scan order and the CABAC inheritance property for dependent tiles, LCUs in one row cannot be finished before the next LCUs row comes, which prevents low latency processing. Second, for single-core applications, where LCUs are processed one by one following the scan order with neighboring LCU coded data buffered for predictions, extra columns of LCU buffers in addition to row buffers are needed to store the coded information for the LCUs not yet processed. For example, in Figure 2, the LCU coded data of LCU #4, #8, and #12 are needed to be buffered for LCU #13, #19, and #25, respectively. Third, for multi-core applications, where processing of different tiles can be assigned to different cores to achieve parallel processing, processing of each core has to wait for its previous tile, which prevent the parallel processing.

In this contribution, low latency CABAC initialization for dependent tiles is proposed to solve the above mentioned problems. The rest of this document is organized as follows. Details of the proposed method will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

Figure 1.  Example of tile partitioning using three columns and three rows of tiles

Figure 2.  CABAC inheritance for dependent tiles

# Proposed method

For dependent tiles to achieve low latency processing, a simple way is to re-initialize the CABAC probabilities with predefined values at the first LCU of each tile. However, for better coding efficiency, CABAC probabilities from neighboring LCUs can be still used as long as it does not introduce extra processing latency compared to the traditional raster scan order. Therefore, in our proposed method, the CABAC probabilities of the first LCU in each tile are inherited from those of the adjacent left LCU as indicated by the arrows as shown in Figure 3. In Figure 3, the CABAC probabilities of LCU #13 are inherited from those of LCU #4 instead of LCU #12, LCU #31 are inherited from LCU #18 instead of LCU #30.

With the proposed CABAC initialization method for each tile, processing of the first LCU in different tiles can start as long as the left LCU has been processed. The low latency requirement can be satisfied easily because the LCUs in one row can be processed right after the original pixels of the same LCU row are ready. For single core applications, it is not necessary to follow the tile structure scan order. Then the extra columns of LCU buffers to store the coded information for the LCUs not yet processed can be saved. For multi core applications, it allows parallel processing of each core to start as long as the left LCU has been processed.

For the proposed low latency CABAC initialization, there are extra buffers required to store the CABAC probabilities of the left LCU. The number of buffers is equal to the number of tile columns minus one, which is two in Figure 3. Since tiles are raster scanned in one picture, buffers for the previous row of tiles can be reused for the next row of tiles to be processed.

Figure 3.  Low latency CABAC initialization

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-F900 [2] and only high efficiency cases using CABAC were tested. The software is based on HM-4.0-dev, rev1422, where tiles are implemented, and the anchor is HM-4.0.

Simulation results of dependent tiles in HM-4.0-dev with the number of tile columns set as 2 and 4 are shown in Table 1 and Table 2, respectively. Simulation results of the proposed method with the number of tile columns set as 2 and 4 are shown in Table 3 and Table 4, respectively. Tile columns in all simulations are uniformly spaced. The proposed low latency CABAC initialization causes 0-0.2% bit rate increases for 2-column tile partitioning and 0-0.7% bit rate increases for 4-column tile partitioning, while the processing latency is the same as no tile partitioning.

Table 1.  Results of HM-4.0-dev dependent tiles with tile columns set as 2

Table 2.  Results of HM-4.0-dev dependent tiles with tile columns set as 4

Table 3.  Results of the proposed dependent tiles with tile columns set as 2

Table 4.  Results of the proposed dependent tiles with tile columns set as 4

# Conclusions

MediaTek proposed a low latency CABAC initialization method for dependent tiles. The proposed method was compared with dependent tiles of HM-4.0-dev. Simulation results reportedly show that the proposed method causes 0-0.2% bit rate increase when the number of tile columns is set to 2 and 0-0.7% bit rate increase when the number of tile columns is set to 4. The processing latency for non-first column tiles is reduced from almost one picture to at most one LCU row.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- A. Fuldseth, M. Horowitz, S. Xu A. Segall, M. Zhou, “Tiles,” Document of Joint Collaborative Team on Video Coding, JCTVC-F335, July 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.