#### Sequence parameter set RBSP semantics

tile_boundary_independence_idc equal to 1 indicates that all neighbouring treeblocks not contained within the same tile as the treeblock being decoded are marked as not available. Otherwise, tile_boundary_independence_idc equal to 0 indicates that availability of treeblocks is not affected by tile boundaries. Further, tile_boundary_independence_idc equal to 1 indicates that the initialisation process for the entropy decoder is invoked when decoding the first treeblock in a tile. Otherwise, tile_boundary_independence_idc equal to 0 indicates that synchronization process for the entropy decoder is invoked when decoding the first treeblock in a tile.

## Tile decoding process

### Decoding process for entropy coding reset

When tile_boundary_independence_idc is equal to 1, the initialisation process for the entropy decoder is invoked when decoding the first treeblock in a tile. Following the decoding of the last treeblock in the previous tile, the following ordered steps are performed:

First, the entropy decoding process is terminated.

Second, the decoder advances to the first bit of the next byte aligned byte.

After the order steps, the decoder initialises the entropy decoder.  When entropy_coding_mode is equal to 1, the decoder initialisation process corresponds to Error: Reference source not foundError: Reference source not found and Error: Reference source not found with cabac_init_idc set equal to the value transmitted in the slice header of the slice containing the first treeblock of the tile.  Otherwise, the decoder initialisation process described in subclause Error: Reference source not found is invoked.

When tile_boundary_independence_idc is equal to 0, the synchronization process for the entropy decoder is invoked when decoding the first treeblock in a tile. Following the decoding of the last treeblock in the previous tile, the following ordered steps are performed:

First, the entropy decoding process is terminated.

Second, the decoder advances to the first bit of the next byte aligned byte.

After the order steps, the decoder synchronize the entropy decoder.  When entropy_coding_mode is equal to 1, the decoder synchronization process corresponds to Error: Reference source not found2 and Error: Reference source not found4Error: Reference source not found. Otherwise, the decoder initialisation process described in subclause Error: Reference source not found is invoked.

## CABAC parsing process for slice data

In case syntax element tile_boundary_independence_idc is equal to 0, when starting the parsing of a coding tree as specified in subclause 7.3.5, if tbAddr is equal to firstTbInTileAddr the synchronization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found, otherwise if tbAddr-firstTbInTileAddr is equal to tileWidthInLCUs the memorization  process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found.