| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G201 |
| --- | --- |

| Title: | Non-CE1: Codeword reordering for last_significant_coeff_x and last_significant_coeff_y |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In CABAC, bypass bins are coded without context and can be processed in parallel easily when they appear in group. In this contribution, codeword reordering is proposed for last_significant_coeff_x and last_significant_coeff_y to group bypass bins together. It is reported the reordering has no impact on coding efficiency and run time.

# Introduction

The binarization for last_significant_coeff_x and last_significant_coeff_y in JCTVC-F375 [1] was adopted. For transform units (TUs) larger than 4x4, if last_significant_coeff_x or last_significant_coeff_y is smaller than W/2, where W is the TU width, the codeword is binarized with unary codes and coded in the regular mode; otherwise, the codeword is divided into a prefix of W/2 bits of zeros coded in the regular mode and a suffix of log2(W/2) bits coded in the bypass mode. Table 1 shows the 8x8 TU case.

Table 1.  Binarization for last_significant_coeff_x and last_significant_coeff_y for 8x8 TUs

| Value | Binarized code word | Prefix unary code part | Suffix fixed-length code part |
| --- | --- | --- | --- |
| 0 | 1 | 1 |  |
| 1 | 01 | 01 |  |
| 2 | 001 | 001 |  |
| 3 | 0001 | 0001 |  |
| 4 | 000000 | 0000 | 00 |
| 5 | 000001 | 0000 | 01 |
| 6 | 000010 | 0000 | 10 |
| 7 | 000011 | 0000 | 11 |

In CABAC hardware, since bypass bins are coded without context, the data path for bypass bins coding is simple. The CABAC hardware can be easily optimized to improve coding throughput for bypass bins with a parallel architecture. Therefore, grouping bypass bins together can increase hardware throughput. In this contribution, codeword reordering is proposed to group the bypass bins of last_significant_coeff_x and last_significant_coeff_y. The concept is similar to JCTVC-F130 [2] and JCTVC-F423 [3] applied for other syntax elements. The rest of this contribution is organized as follows. The proposed algorithm will be described in Section 2, simulation results will be shown in Section 3, and finally conclusions will be given in Section 4.

# Algorithm description

After binarization of last_significant_coeff_x and last_significant_coeff_y, we reorder the binarization results of last_significant_coeff_x and last_significant_coeff_y to put the bypass bins (suffix parts) of these two syntax elements together. As shown in Figure 1, for an 8x8 TU, the binarization result of last_significant_coeff_x equal to 7 is 000011. The prefix part, 0000, is coded in the regular mode, and the suffix part, 11, is coded in the bypass mode. The binarization result of last_significant_coeff_y equal to 6 is 000010. The prefix part is 0000, and the suffix part is 10. In the proposed method, the bypass bins of last_significant_coeff_x and last_significant_coeff_y are grouped together, and the coding order is shown in Figure 2. The regular parts of last_significant_coeff_x and last_significant_coeff_y are coded first, and then the bypass parts of last_significant_coeff_x and last_significant_coeff_y are coded.

Figure 1.  HM-4.0 binarization for last_significant_coeff_x and last_significant_coeff_y

Figure 2.  Reordering for bins of last_significant_coeff_x and last_significant_coeff_y

# Simulation results

The proposed method is implemented on HM-4.0. All results were simulated according to the common test conditions defined in JCTVC-F900 [4]. The tested conditions are high efficiency all intra (HE-AI), high efficiency random access (HE-RA), and high efficiency low delay (HE-LD). Table 2 shows the results of the proposed reordering for bins of last_significant_coeff_x and last_significant_coeff_y. No bit rate increase is observed. The encoding time and decoding time are roughly unchanged.

Table 2.  Results of reordering for bins of last_significant_coeff_x and last_significant_coeff_y

# Conclusions

MediaTek proposed a codeword reordering method to group the bypass bins of last_significant_coeff_x and last_significant_coeff_y together for increasing hardware throughput. Simulation results reportedly show the proposed method has no impact on coding efficiency and run time.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- V. Seregin, I.-K. Kim, “Binarisation modification for last position coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-F375, July 2011.

- V. Sze, M. Budagavi, “Parallel context processing of coefficient level,” Document of Joint Collaborative Team on Video Coding, JCTVC-F130, July 2011.

- H. Sasai, T. Nishi, “Modified MVD coding for CABAC,” Document of Joint Collaborative Team on Video Coding, JCTVC-F423, July 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.