# Draft Changes

### Residual coding CABAC syntax

| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x | ae(v) |
| last_significant_coeff_y | ae(v) |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| last_significant_coeff_x_suffix | ae(v) |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| while( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) { |  |
| numCoeff++ |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| } |  |
| for( n = numCoeff − 1; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| numLastSubset = numCoeff >> 4 |  |
| for( i = numLastSubset − 1; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset` ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset` ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset` ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = 0 |  |
| } |  |
| } |  |
| } |  |

### Residual coding CABAC semantics

last_significant_coeff_x, last_significant_coeff_y specify the column and the row position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_x and last_significant_coeff_y shall be in the range from 0 to ( 1 << log2TrafoSize ) − 1, inclusive.

When scanIdx is equal to 2, the coordinates are swapped as follows.

temp  			=  last_significant_coeff_x
last_significant_coeff_x  =  last_significant_coeff_y		(7-1)
last_significant_coeff_y  =  temp

last_significant_coeff_x_prefix, last_significant_coeff_y_prefix, last_significant_coeff_x_suffix, last_significant_coeff_y_suffix: The values of last_significant_coeff_x and last_significant_coeff_y specify the column and the row position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_x and last_significant_coeff_y shall be in the range from 0 to ( 1 << (log2TrafoSize-1) ) , inclusive. The value of last_significant_coeff_x can be binarized into last_significant_coeff_x_prefix and last_significant_coeff_x_suffix, and the value of last_significant_coeff_y can be binarized into last_significant_coeff_y_prefix and last_significant_coeff_y_suffix, as specified in subclause 9.3.2.10. The values of last_significant_coeff_x_prefix and last_significant_coeff_y_prefix shall be in the range from 0 to ( 1 << (log2TrafoSize-1) ) − 1, inclusive.

When scanIdx is equal to 2, the coordinates are swapped as follows.

temp  			=  last_significant_coeff_x
last_significant_coeff_x  =  last_significant_coeff_y		(7-2)
last_significant_coeff_y  =  temp

#### Initialisation process for context variables

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| residual_coding() | last_significant_coeff_x last_significant_coeff_x_prefix | Error: Reference source not found | 0..40 | 41..81 | 82..122 |
|  | last_significant_coeff_y last_significant_coeff_y_prefix | Error: Reference source not found | 0..40 | 41..81 | 82..122 |

### Binarization process

| Table 9-2 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| cbf_cr | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 4 |
|  | B |  | 0 | Error: Reference source not found | 8 |
| last_significant_coeff_x_prefix | I | TU, cMax = ( 1 << (log2TrafoSize-1) ) as specified in subclause Error: Reference source not found10 | 30 | Error: Reference source not found | 0 |
|  | P |  | 30 | Error: Reference source not found | 41 |
|  | B |  | 30 | Error: Reference source not found | 82 |
| last_significant_coeff_y_prefix | I | TU, cMax = ( 1 << (log2TrafoSize-1) ) as specified in subclause Error: Reference source not found10 | 30 | Error: Reference source not found | 0 |
|  | P |  | 30 | Error: Reference source not found | 41 |
|  | B |  | 30 | Error: Reference source not found | 82 |
| last_significant_coeff_x_suffix | all | FL, cMax = log2TrafoSize-1 as specified in subclause Error: Reference source not found10 | na | na | na, (uses Decode Bypass) |
| last_significant_coeff_x_suffix | all | FL, cMax = log2TrafoSize-1 as specified in subclause Error: Reference source not found10 | na | na | na, (uses Decode Bypass) |

#### Binarization process for last_significant_coeff_x_prefix, last_significant_coeff_y_prefix, last_significant_coeff_x_suffix, last_significant_coeff_y_suffix

Input to this process is a request for a binarization for the last_significant_coeff_x and last_significant_coeff_y, and a variable log2TrafoSize specifying the current transform unit size.

Output of this process is the binarization of the syntax element last_significant_coeff_x_prefix, last_significant_coeff_y_prefix, last_significant_coeff_x_suffix, last_significant_coeff_y_suffix.

The variables last_significant_coeff_n_prefix is derived as follows, n = x or y.

- If log2TrafoSize is equal to 2, the bin string of last_significant_coeff_n in TU code with cMax = 4 is assigned to last_significant_coeff_n_prefix.

- Otherwise if last_significant_coeff_n is smaller than the value of (1<<( log2TrafoSize-1)), the bin string of last_significant_coeff_n in TU code with cMax = (1<<( log2TrafoSize-1)) is assigned to last_significant_coeff_n_prefix.

- Otherwise, the bin string of (log2TrafoSize-1) bits of zeros is assigned to last_significant_coeff_n_prefix.

The variables last_significant_coeff_n_suffix is derived as follows, n = x or y.

- If log2TrafoSize is equal to 2, the last_significant_coeff_n_suffix is not available.

- Otherwise if last_significant_coeff_n is smaller than the value of (1<<( log2TrafoSize-1)), the last_significant_coeff_n_suffix is not available.

- Otherwise, the bin string of the value of (last_significant_coeff_n - (1<<( log2TrafoSize-1)) in FL code with cMax = log2TrafoSize -1 is assigned to last_significant_coeff_n_suffix.