## Inverse scanning process for transform coefficients

Inputs to this process are:

–	a variable nW specifying the width of the current transform unit,

–	a variable nH specifying the height of the current transform unit.

–	a list of (nW)x(nH) values transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ].

–	a variable cIdx specifying the chroma component of the current block.

Output of this process is a variable c containing a two-dimensional array of (nW)x(nH) values.

The variable nS is set equal to Sqrt(nW*nH).

The variable scanIdx, indicating wich scan is used to scan the transform coefficients (zigzag, horizontal or vertical), is derived as follows.

If PredMode is equal to MODE_INTRA, the following applies.

If cIdx is equal to 0, scanIdx is set to ScanType[ Log2( nS >> 2 ) ][ IntraPredMode ].

Otherwise (cIdx is equal to 1 or 2), scanIdx is set to ScanType[ Log2( nS >> 2 ) ][ IntraPredModeC ]

Otherwise (PredMode is not equal to MODE_INTRA),

if entropy_coding_mode_flag is equal to 0 (CAVLC), when nW is equal to nH, scanIdx is set to 0,when nW is larger than nH, scanIdx is set to 2, else scanIdx is set to 1;

else, scanIdx is set to 0.

When entropy_coding_mode_flag is equal to 1 (CABAC) and scanIdx is equal to 0 (zigzag), scanIdx is set to 3 (diagonal).