| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G202 |
| --- | --- |

| Title: | Non-CE2: Modified NSQT coefficient scan for CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-4.0, NSQT was adopted and non-square transform blocks were remapped to square blocks before coefficient coding. In CE2.C.1, experiments were carried out to remove the remapping procedure and code coefficients directly. However, extra zigzag scan patterns for non-square transform blocks were still required in CAVLC. In this contribution, based on CE2.C.1, non-square transform blocks are directly coded using vertical scan. Extra non-square zigzag scan patterns can be removed in CAVLC. The simulation results reportedly show that the proposed method achieves 0-0.1% luma bit rate reduction and 0.1-0.5% chroma bit rate reduction. No run time increase is observed on the encoder and decoder sides.

# Introduction

Both non-square quadtree transform (NSQT) and residual quadtree transform (RQT) are split recursively. RQT uses square transform units (TUs) including 2Nx2N and NxN, while NSQT introduces non-square TUs. At NSQT first level, 2Nx2N transform is still used, which is based on the consideration that 2Nx2N transform can get better result when residual inside a coding unit is small. What is more, using 2Nx2N TU at the first level can also save signaling bits. At NSQT second and third levels, non-square TU is used when the prediction unit (PU) is 2NxN or Nx2N. Then, for the case of 2NxN PU type, TU size at the second and third levels will be 2Nx0.5N and Nx0.25N, respectively; for the case of Nx2N PU type, TU size at the second and third levels will be 0.5Nx2N and 0.25NxN, respectively. At NSQT second level, a block is divided into four non-square TUs along with vertical direction if PU partition type is 2NxN, or four non-square TUs along with horizontal direction if PU partition type is Nx2N. At NSQT third level, a block is also divided into four non-square TUs along with horizontal or vertical direction. This method preserves the residual quadtree structure, and the coding efficiency is also improved. Figure 1(a) and Figure 1(b) illustrate the NSQT scheme. In HM-4.0, four non-square TU sizes are supported: 32x8, 8x32, 16x4, and 4x16. In this contribution, different NSQT coefficient scan orders are proposed for CAVLC. The rest of this document is organized as follows. Details of the proposed method will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

- The leaf CU is 2Nx2N, the PU type of the leaf CU is 2NxN, the 1st level NSQT uses 2Nx2N TU, the 2nd NSQT uses 2Nx0.5N TU, and the 3rd level uses Nx0.25N TU.

- The leaf CU is 2Nx2N, the PU type of the leaf CU is Nx2N, the 1st level NSQT uses 2Nx2N TU, the 2nd level NSQT uses 0.5Nx2N TU, and the 3rd level NSQT uses 0.25NxN TU.

Figure 1.  Illustration of NSQT

# Proposed method

For coding transform coefficients of a non-square TU, remapping will be performed first in HM-4.0 to reshape the non-square transform block into a square one. For example, a 32x8 non-square transform block will be reshaped into a 16x16 square transform block, and 16x4 will be reshaped into 8x8. The remapping procedure for CAVLC is illustrated in Figure 2. Then, the reshaped square transform blocks will use the same coefficient coding procedure as it is designed for the original square transform blocks using CAVLC. As it can be seen from Figure 2, in addition to the square zigzag scan pattern, extra non-square zigzag scan pattern is required.

Figure 2.  Illustration of remapping procedure from 16x4 to 8x8 in HM-4.0

In CE2.C.1 [1], experiments were carried out to remove the above-mentioned remapping procedure, where non-square TU block was directly coded using zigzag scan for CAVLC. Whether remapping is removed or not, there will still be extra scan patterns required for non-square TUs. However, zigzag scan patterns, especially for non-square blocks, are much more irregular compared to other scan patterns, such as horizontal and vertical scans. From implementation point of view, it demands extra storages to store those scan patterns.

In our proposed method, remapping is removed as it was done in CE2.C.1. Moreover, vertical scan is used to directly code the non-square transform blocks. Figure 3 shows an example of the vertical scan patterns within a non-square 16x4 block. In this way, irregular zigzag scan pattern can be replaced.

Figure 3.  Vertical scan pattern for 16x4 block

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-F900 [2], and LC-RA, LC-LDB, LC-LDP with NSQT were tested. The software is modified based on that provided by CE2.C.1, and the anchor is HM-4.0.

Simulation results are shown in Table 1. The proposed method achieves 0-0.1% luma bit rate reduction and 0.1-0.5% chroma bit rate reduction. Encoding and decoding run times are the same in comparison with the anchor.

Table 1.  Results of the proposed method

# Conclusions

In this contribution, based on CE.2.C.1, where remapping procedure is removed for NSQT, non-square blocks are directly coded using the vertical scan. Extra non-square zigzag scan patterns can be removed for CAVLC. The simulation results reportedly show that the proposed method achieves 0-0.1% luma bit rate savings and 0.1-0.5% chroma bit rate savings. No run time increase is observed.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- X. Zheng, Y. Yuan, Y. He, “CE2 subtest C.1: Harmonization of unified scan and NSQT,” Document of Joint Collaborative Team on Video Coding, JCTVC-G517, November 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.