# Draft Changes

### Prediction unit syntax

| prediction_unit( x0, y0 , log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2CUSize >= Log2MinIPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| else if (log2PUWidth != 6) { |  |
| mprm_pred_flag [ x0 ][ y0 ] | u(1) ¦ ae(1) |
| if(mprm_pred_flag [ x0 ][ y0 ]) |  |
| mprm_idx[ x0 ][ y0 ] | u(v) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| } else { |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### Prediction unit semantics

mvp_idx_l0[ x0 ][ y0 ] specifies the motion vector predictor index of list 0 where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When mvp_idx_l0[ x0 ][ y0 ] is not present, it shall be inferred to be equal to 0.

mvp_idx_l1[ x0 ][ y0 ] has the same semantics as mvp_idx_l0, with l0 and list 0 replaced by l1 and list 1, respectively.

mvp_idx_lc[ x0 ][ y0 ] has the same semantics as mvp_idx_l0, with l0 and list 0 replaced by lc and list combination, respectively.

prev_intra_luma_pred_flag[ x0 ][ y0 ], mpm_idx[ x0 ][ y0 ], mprm_pred_flag [ x0 ][ y0 ], mprm_idx[ x0 ][ y0 ] and rem_intra_luma_pred_mode[ x0 ][ y0 ] specify the intra prediction mode for luma samples. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When prev_intra_luma_pred_flag[ x0 ][ y0 ] is equal to 1, the intra prediction mode is inferred from a neighbouring intra-predicted prediction unit according to subclause 8. When mprm_pred_flag [ x0 ][ y0 ] is equal to 1, the intra prediction mode is derived from a neighbouring intra-predicted prediction unit according to subclause 8.

### Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PUSize specifying the size of the current prediction unit,

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

Output of this process is the variable IntraPredMode[ xB ][ yB ].

Table 8-1 specifies the value for the intra prediction mode and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_Vertical |
| 2 | Intra_Horizontal |
| 3 | Intra_DC |
| Otherwise (4..34) | Intra_Angular |
| 35 | Intra_FromLuma (used only for chroma) |

Table 8-2 specifies the number of luma intra prediction modes intraPredModeNum depending on log2PUSize.

Table 8-2 – Specification of intraPredModeNum

| log2PUSize | intraPredModeNum |
| --- | --- |
| 2 | 19 |
| 3 | 35 |
| 4 | 35 |
| 5 | 35 |
| 6 | 4 |

Table 8-3 specifies the mapping table used for converting the number of intra prediction modes.

Table 8-3 – Specification of mapIntraPredMode4 and mapIntraPredMode10

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mapIntraPredMode4[ value ] | 0 | 1 | 2 | 3 | 3 | 3 | 1 | 3 | 2 | 2 | 3 | 3 | 1 | 1 | 3 | 3 | 2 |  |
| mapIntraPredMode10[ value ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |  |
| value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| mapIntraPredMode4[ value ] | 2 | 3 | 3 | 3 | 3 | 1 | 1 | 1 | 3 | 3 | 3 | 3 | 2 | 2 | 2 | 2 | 3 | 3 |
| mapIntraPredMode10[ value ] | 3 | 3 | 4 | 5 | 5 | 1 | 1 | 6 | 6 | 7 | 4 | 8 | 8 | 2 | 2 | 9 | 9 | 3 |

IntraPredMode[ xB ][ yB ] labelled 0, 1, 2, .., 34 represents directions of predictions as illustrated in Figure 8-1.

Figure 8-1 – Intra prediction mode directions (informative)

Table 8-x – Specification of intraMPRM19_0, intraMPRM19_1, and intraMPRM19_2

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraMPRM19_0[value] | 1 | 12 | 17 | 1 | 15 | 11 | 13 | 14 | 16 | 18 | 7 | 4 | 5 | 1 | 6 | 8 | 2 | 9 | 10 |
| intraMPRM19_1[value] | 2 | 13 | 16 | 2 | 11 | 12 | 14 | 10 | 15 | 17 | 18 | 5 | 1 | 6 | 7 | 4 | 8 | 2 | 9 |
| intraMPRM19_2[value] | 9 | 2 | 1 | 9 | 3 | 1 | 1 | 3 | 2 | 2 | 3 | 15 | 13 | 12 | 3 | 11 | 17 | 16 | 3 |

Table 8-x – Specification of intraMPRM35_0, intraMPRM35_1, and intraMPRM35_2

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraMPRM35_0[value] | 1 | 22 | 31 | 1 | 27 | 20 | 24 | 26 | 29 | 33 | 7 | 19 | 21 | 23 | 25 | 28 | 30 |  |
| intraMPRM35_1[value] | 2 | 23 | 30 | 2 | 19 | 21 | 25 | 10 | 28 | 32 | 34 | 20 | 22 | 24 | 26 | 27 | 29 |  |
| intraMPRM35_2[value] | 9 | 2 | 1 | 9 | 3 | 1 | 1 | 3 | 2 | 2 | 3 | 15 | 13 | 12 | 3 | 11 | 17 |  |
| value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraMPRM35_0[value] | 32 | 34 | 4 | 11 | 5 | 12 | 1 | 13 | 6 | 14 | 15 | 8 | 16 | 2 | 17 | 9 | 18 | 10 |
| intraMPRM35_1[value] | 31 | 33 | 11 | 5 | 12 | 1 | 13 | 6 | 14 | 7 | 4 | 15 | 8 | 16 | 2 | 17 | 9 | 18 |
| intraMPRM35_2[value] | 16 | 3 | 27 | 19 | 22 | 23 | 22 | 23 | 3 | 3 | 19 | 27 | 30 | 31 | 30 | 31 | 3 | 3 |

IntraPredMode[ xB ][ yB ] is derived as the following ordered steps. [Ed. (WJ): proponent suggests to move this part to the syntax since the other syntax elements utilize IntraPredMode. But it seems too complex to move all the following process to the syntax table. Maybe it’s better to move this part to the semantics section or simply avoid the use of IntraPredMode to parse the syntax item]

- The derivation process for neighbouring treeblocks specified in subclause XXX with ( xB,  yB ) given as input and the output is assigned to tbAddrA and tbAddrB specifying the treeblock addresses of treeblocks covering ( xBA,  yBA ) and ( xBB, yBB ) respectively where ( xBA,  yBA ) is set equal to ( xB-1,  yB ) and ( xBB,  yBB ) is set equal to ( xB,  yB-1 ).

- For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If the treeblock with address tbAddrN is not available, intraPredModeN is set equal to -1

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to Intra_DC,

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

- For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

- If intraPredModeN is greater than or equal to intraPredModeNum

- If intraPredModeNum is equal to 4 then candIntraPredModeN is set equal to mapIntraPredMode4[ intraPredModeN ]

- Otherwise candIntraPredModeN is set equal to mapIntraPredMode10[ intraPredModeN ].

- Otherwise, candIntraPredModeN is set equal to intraPredModeN

- The candModeList[x] and NumMPMCand are derived as follows:

- NumMPMCand is set equal to 2

- If both candIntraPredModeN are not available, then the value 3 is assigned to candModeList[ 0 ] and the value 0 is assigned to candModeList[ 1 ]

- Otherwise, if only one candIntraPredModeN is available or if both candIntraPredModeN are the same, candModeList[ x ] is derived by applying the following procedure:

  - candIntraPredModeN is assigned to candModeList[ 0 ]

  - If candIntraPredModeN is not equal to 0, the value 0 is assigned to candModeList[ 1 ]

  - Otherwise, the value 3 is assigned to candModeList[ 1 ]

- Otherwise, both candIntraPredModeN are assigned to the candidate modes list with the smaller of the two candidates at candModeList[ 0 ] and the larger at candModeList[ 1 ].

- The candMPRM_A[x] and candMPRM_B[x] are derived as follows:

- If intraPredModeNum is equal to 19, candMPRM_A[0] is set equal to intraMPRM19_0[candIntraPredModeA], candMPRM_A[1] is set equal to intraMPRM19_1[candIntraPredModeA], candMPRM_A[2] is set equal to intraMPRM19_2[candIntraPredModeA], candMPRM_B[0] is set equal to intraMPRM19_0[candIntraPredModeB], candMPRM_B[1] is set equal to intraMPRM19_1[candIntraPredModeB], and candMPRM_B[2] is set equal to intraMPRM19_2[candIntraPredModeB].

- Otherwise, candMPRM_A[0] is set equal to intraMPRM35_0[candIntraPredModeA], candMPRM_A[1] is set equal to intraMPRM35_1[candIntraPredModeA], candMPRM_A[2] is set equal to intraMPRM35_2[candIntraPredModeA], candMPRM_B[0] is set equal to intraMPRM35_0[candIntraPredModeB], candMPRM_B[1] is set equal to intraMPRM35_1[candIntraPredModeB], and candMPRM_B[2] is set equal to intraMPRM35_2[candIntraPredModeB].

- The candMPRMList[x] are derived as follows:

- If candIntraPredModeA is equal to candIntraPredModeB, candMPRMList[n] is set equal to candMPRM_A[n], n = 0…2.

- Otherwise if candIntraPredModeA is equal to 0, candMPRMList[n] is set equal to candMPRM_B[n], n = 0…2.

- Otherwise if candIntraPredModeB is equal to 0, candMPRMList[n] is set equal to candMPRM_A[n], n = 0…2.

- Otherwise, candMPRMList[0] is set equal to 0 and the candMPRMList[1] and candMPRMList[2] are derived as follow:

  - If candMPRM_B[0] is equal to candIntraPredModeA, candMPRMList[1] is set equal to candMPRM_A[0] and candMPRMList[2] is set equal to candMPRM_B[1].

  - Otherwise if candMPRM_A[0] is equal to candIntraPredModeB, candMPRMList[1] is set equal to candMPRM_B[0], and candMPRMList [2] is derived as follow:

    - If candMPRM_B[1] is equal to candIntraPredModeA, candMPRMList [2] is set equal to candMPRM_A[1].

    - Otherwise, candMPRMList[2] is set equal to candMPRM_B[1].

  - Otherwise, candMPRMList[1] is set equal to candMPRM_B[0], and candMPRMList[2] is derived as follow:

    - If candMPRM_A[0] is equal to candMPRM_B[0], candMPRMList[2] is set equal to candMPRM_B[1].

    - Otherwise, candMPRMList [2] is set equal to candMPRM_A[0].

- IntraPredMode[ xB ][ yB ] is derived by applying the following procedure:

- If prev_intra_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[mpm_idx][ xB ][ yB ]]

- Other if mprm_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  candMPRMList [mprm_idx][ xB ][ yB ]]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following ordered steps:

  - IntraPredMode[ xB ][ yB ] = rem_intra_pred_mode

- The candMPMAllList[x] and NumMPMAllCand are derived as follows:

  - The value of 5 is assigned to NumMPMAllCand.

  - The modes in candModeList [x], with x=0,1, and the modes in candMPRMList[y], with y=0,1,2, are assigned to candMPMAllTempList[z], with z = 0…4

  - The available modes in candMPMAllTempList[x] are sorted in increasing order and are assigned to candMPMAllList[x], with x = 0…4

- for (cIdx = 0; cIdx < NumMPMAllCand; cIdx++) {

if (IntraPredMode[ xB ][ yB ]>= candMPMAllList [cIdx]) IntraPredMode[ xB ][ yB ]++}

  - If candModeList[ 0 ] is greater than candModeList[ 1 ], then two values are swapped. [Ed. (WJ): may be better expression needed]

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 0 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 1 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

### Derivation process for chroma intra prediction mode

[Ed.: (WJ) this subclause may be moved to the semantics of intra_chroma_pred_mode syntax]

Input to this process is a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture.

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PUSize specifying the size of the current prediction unit,

–	a variable IntraPredMode[ xB ][ yB ] specifying the luma intra prediction mode of the current prediction unit

Output of this process is the variable IntraPredModeC.

The chroma intra prediction mode IntraPredModeC is derived as specifed in Table 8-4 or Table 8-5 with intra_chroma_pred_mode, IntraPredMode[ xB ][ yB ] and chroma_pred_from_luma_enabled_flag as inputs.

The chroma intra prediction mode IntraPredModeC is derived as follow:

- If log2PUSize is equal to 6 or IntraPredMode[ xB ][ yB ] is equal to 0 or 3, the value of IntraPredModeC is derived as specifed in Table 8-4 or Table 8-5 with intra_chroma_pred_mode, IntraPredMode[ xB ][ yB ] and chroma_pred_from_luma_enabled_flag as inputs.

- Otherwise, the chroma intra prediction mode IntraPredModeC is derived by applying the following procedure:

  - If log2PUSize is equal to 2, the value of adjectMode0 is set equal to intraMPRM19_0[IntraPredMode[ xB ][ yB ]] and the value of adjectMode1 is set equal to intraMPRM19_1[IntraPredMode[ xB ][ yB ]].

  - Otherwise, the value of adjectMode0 is set equal to intraMPRM35_0[IntraPredMode[ xB ][ yB ]] and the value of adjectMode1 is set equal to intraMPRM35_1[IntraPredMode[ xB ][ yB ]].

  - The value of perpendicularMode is set equal to intraPerpendicularMode[IntraPredMode[ xB ][ yB ]].

  - The value of IntraPredModeC is derived as specifed in Table 8-x1 or Table 8-x2 with intra_chroma_pred_mode, IntraPredMode[ xB ][ yB ], adjectMode0, adjectMode1, perpendicularMode and chroma_pred_from_luma_enabled_flag as inputs.

Table 8-4 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 1

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X (4 <= X < 35 ) |
| 0 | 7 | 0 | 0 | 0 | 0 |
| 1 | 1 | 7 | 1 | 1 | 1 |
| 2 | 2 | 2 | 7 | 2 | 2 |
| 3 | 3 | 3 | 3 | 7 | 3 |
| 4 | LM | LM | LM | LM | LM |
| 5 | 0 | 1 | 2 | 3 | X |

Table 8-5 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 0

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 4 <= X < 35 ) |
| 0 | 7 | 0 | 0 | 0 | 0 |
| 1 | 1 | 7 | 1 | 1 | 1 |
| 2 | 2 | 2 | 7 | 2 | 2 |
| 3 | 3 | 3 | 3 | 7 | 3 |
| 4 | 0 | 1 | 2 | 3 | X |

Table 8-x – Specification of intraPerpendicularMode

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPerpendicularMode[value] | 0 | 2 | 1 | 0 | 7 | 9 | 8 | 4 | 6 | 5 | 4 | 18 | 17 | 16 | 15 | 14 | 13 |  |
| value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraPerpendicularMode [value] | 12 | 11 | 34 | 33 | 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 |

Table 8-x1 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 1

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X (4 <= X < 35 ) |
| 0 | - | 0 | 0 | - | 0 |
| 1 | - | adjectMode0 | adjectMode0 | - | adjectMode0 |
| 2 | - | adjectMode1 | adjectMode1 | - | adjectMode1 |
| 3 | - | perpendicularMode | perpendicularMode | - | perpendicularMode |
| 4 | - | LM | LM | - | LM |
| 5 | - | 1 | 2 | - | X |

Table 8-x2 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 0

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 4 <= X < 35 ) |
| 0 | - | 0 | 0 | - | 0 |
| 1 | - | adjectMode0 | adjectMode0 | - | adjectMode0 |
| 2 | - | adjectMode1 | adjectMode1 | - | adjectMode1 |
| 3 | - | perpendicularMode | perpendicularMode | - | perpendicularMode |
| 4 | - | 1 | 2 | - | X |

### Parsing process for VLC codes

Table 9-6 – codeNum and codeword with vlcNum equal to 20

| - codeNum | - Codeword |
| --- | --- |
| - 0 | - 100 |
| - 1 | - 101 |
| - 2 | - 0000 |
| - 3 | - 0001 |
| - 4 | - 0010 |
| - 5 | - 0011 |
| - 6 | - 0100 |
| - 7 | - 0101 |
| - 8 | - 0110 |
| - 9 | - 0111 |
| - 10 | - 1100 |
| - 11 | - 1101 |
| - 12 | - 1110 |
| - 13 | - 1111 |
| - 14 | - 011111 |
| - 15 | - 011110 |

Table 9-7 – codeNum and codeword with vlcNum equal to 21

| - codeNum | - Codeword |
| --- | --- |
| - 0 | - 100 |
| - 1 | - 0110 |
| - 2 | - 0010 |
| - 3 | - 1100 |
| - 4 | - 01000 |
| - 5 | - 01010 |
| - 6 | - 01110 |
| - 7 | - 00000 |
| - 8 | - 00001 |
| - 9 | - 00010 |
| - 10 | - 00011 |
| - 11 | - 00110 |
| - 12 | - 00111 |
| - 13 | - 11010 |
| - 14 | - 11011 |
| - 15 | - 11110 |
| - 16 | - 11100 |
| - 17 | - 11101 |
| - 18 | - 10100 |
| - 19 | - 10101 |
| - 20 | - 10110 |
| - 21 | - 10111 |
| - 22 | - 010010 |
| - 23 | - 010011 |
| - 24 | - 010110 |
| - 25 | - 010111 |
| - 26 | - 011110 |
| - 27 | - 011111 |
| - 28 | - 111110 |
| - 29 | - 111111 |
| - 30 | - 0110010 |
| - 31 | - 01111111 |
| - 32 | - 01111110 |

#### Initialisation process for intraModeTable

Outputs of this process are initial values of the variable array intraModeTable.

The variable array intraModeTable is initialized as specified in Table 9-8.

Table 9-8 – Specification of intraModeTable[k][codeNum]

|  | - codeNum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - k | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 | - 16 |
| - 0 | 1 | 6 | 2 | 0 | 8 | 12 | 3 | 5 | 9 | 7 | 11 | 4 | 10 | 13 |  |  |  |
| - 1 | 2 | 1 | 0 | 27 | 28 | 14 | 26 | 6 | 8 | 7 | 5 | 13 | 19 | 10 | 3 | 9 | 18 |

|  | - codeNum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - k | - 17 | - 18 | - 19 | - 20 | - 21 | - 22 | - 23 | - 24 | - 25 | - 26 | - 27 | - 28 | - 29 | - 30 | - 31 | - 32 | - 33 |
| - 1 | 15 | 29 | 20 | 4 | 25 | 12 | 11 | 16 | 17 | 21 | 24 | 23 | 22 |  |  |  |  |

#### Initialisation process for context variables

Table 9-9x – Values of variable m and n for mprm_pred_flag ctxIdx

| Initialisation variables | mprm_pred_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| m | 2 | 0 | 0 |
| n | 54 | 50 | 51 |

Table 9-10xx – Values of variable m and n for mprm_idx ctxIdx

| Initialisation variables | mprm_idx ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| m | 2 | 2 | 0 | 0 | 0 | 0 |
| n | 54 | 54 | 50 | 50 | 51 | 51 |

### 9.3.2 Binarization process

| Table 9-11 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| prev_intra_luma_pred_flag | I | FL, cMax = 1 | 0 | Table 9-9 | 0 |
|  | P |  | 0 | Table 9-9 | 1 |
|  | B |  | 0 | Table 9-9 | 2 |
| mprm_pred_flag | I | FL, cMax = 1 | 0 | Table 9-924x | 0 |
|  | P |  | 0 | Table 9-924x | 1 |
|  | B |  | 0 | Table 9-924x | 2 |
| mprm_idx | I | TU, cMax = 2 | 1 | Table 9-924x | 0 |
|  | P |  | 1 | Table 9-924x | 2 |
|  | B |  | 1 | Table 9-924x | 4 |
| rem_intra_luma_pred_mode | I | as specified in subclause 9.3.2.9 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |

#### Binarization process for rem_intra_luma_pred_mode

Input to this process is a request for the syntax element rem_intra_luma_pred_mode and a variable log2PUSize specifying the size of the current prediction unit.  cNumBins.

Output of this process is the binarization of the syntax element.

The binarization for rem_intra_luma_pred_mode is given by Table 9-1248.

Table 9-48 – Binarization for rem_intra_luma_pred_mode

| Value of rem_intra_luma_pred_mode | Bin string |  |  |
| --- | --- | --- | --- |
|  | log2PUSize = 6 | log2PUSize = 5, 4, 3 | log2PUSize = 2 |
| 0 | 0 | 0000 | 000 |
| 1 | 1 | 0001 | 001 |
| 2 |  | 00010 | 0010 |
| 3 |  | 00011 | 0011 |
| 4 |  | 00100 | 0100 |
| 5 |  | 00101 | 0101 |
| 6 |  | 00110 | 0110 |
| 7 |  | 00111 | 0111 |
| 8 |  | 01000 | 1010 |
| 9 |  | 01001 | 1011 |
| 10 |  | 01010 | 1100 |
| 11 |  | 01011 | 1101 |
| 12 |  | 01100 | 1110 |
| 13 |  | 01101 | 1111 |
| 14 |  | 01110 |  |
| 15 |  | 01111 |  |
| 16 |  | 10010 |  |
| 17 |  | 10011 |  |
| 18 |  | 10100 |  |
| 19 |  | 10101 |  |
| 20 |  | 10110 |  |
| 21 |  | 10111 |  |
| 22 |  | 11000 |  |
| 23 |  | 11001 |  |
| 24 |  | 11010 |  |
| 25 |  | 11011 |  |
| 26 |  | 11100 |  |
| 27 |  | 11101 |  |
| 28 |  | 11110 |  |
| 29 |  | 11111 |  |

Table 9-12 – Binarization for rem_intra_luma_pred_mode

| Value of rem_intra_luma_pred_mode | Bin string |
| --- | --- |
| less than 32 | FL, cMax = cNumBins |
| 32 | 111110 |
| 33 | 111111 |

#### Derivation process for ctxIdx

| Table 9-13 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| prev_intra_luma_pred_flag | Table 9-9 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| mprm_pred_flag | Error: Reference source not found24x | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| mprm_idx | Error: Reference source not found24xx | 0 | 0 | 1 | na | na | na |
|  |  | 2 | 0 | 1 | na | na | na |
|  |  | 4 | 0 | 1 | na | na | na |
| rem_intra_luma_pred_mode | Error: Reference source not found | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | 1 | 0 | 0 | 0 | 0 | 0 |
|  |  | 2 | 0 | 0 | 0 | 0 | 0 |