| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G203 |
| --- | --- |

| Title: | CE6b: Intra prediction mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Mei Guo, Xun Guo,Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Wei-Jung Chien, Xianglin Wang, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com Marta Karczewicz +1-858-658-5673 {wchien, xianglin, martak}@qualcomm.com |
| Source: | MediaTek Inc., Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports results of one joint proposal in CE6b, which is a harmonization of JCTVC-F062 and JCTVC-F459 including an intra luma prediction mode coding method and an intra chroma prediction mode coding method. In HM-4.0 intra luma prediction mode coding, two intra prediction modes are derived from its left block or upper block as the most probable modes (MPMs). In this contribution, three most probable remaining modes (MPRMs) are added according to the intra prediction modes of left block or upper block. Moreover, one intra prediction mode, (Hor.+6), is added for Intra_4x4 coding. For intra chroma prediction mode coding, the vertical, horizontal, and DC modes among the six intra prediction modes are replaced by two intra prediction modes adjacent to the coded intra luma prediction mode and one intra prediction mode perpendicular to the coded intra luma prediction mode. Simulation results reportedly show 0.4-0.5% luma bit rate reduction and 0.5-0.7% chroma bit rate reduction in all intra conditions.

# Introduction

In High-Efficiency Video Coding test model version 4.0 (HM-4.0), intra luma prediction mode coding uses two intra prediciton modes as most probable modes (MPMs). These MPMs are derived from the intra prediction modes of the left block and the upper block. If the current intra prediction mode (ModeC) is equal to one of the MPMs, prev_pred_mode_flag will be set as one, and mpm_idx will be utilized to indicate which MPM represents ModeC; otherwise (i.e. ModeC is not equal to any of the MPMs), prev_pred_mode_flag will be set as zero, and rem_intra_luma_pred_mode is derived and coded. The coding flow of intra luma prediction mode coding is described in Figure 1. However, we found that the remaining modes are still not equally distributed. To improve the coding efficiency, more MPMs are desirable. In this proposal, three most probable remaining modes (MPRMs) are added for better coding efficiency. As for intra chroma prediction mode coding, six intra prediction modes, which are Planar, Vertical, Horizontal, DC, DM, and LM, are used in HM-4.0. Among these six intra prediction modes, our analysis shows that the Vertical, Horizontal, and DC modes are rarely selected. In order to improve the chroma mode coding efficiency, instead of using fixed intra prediction modes, the Vertical, Horizontal, and DC modes are replaced by two intra prediction modes adjacent to DM and one intra prediction mode perpendicular to DM. The rest of this contribution is organized as follows. The algorithm will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

Figure 1.  The flow of the intra luma prediction mode coding in HM-4.0

# Algorithm description

## Proposed intra luma prediction mode coding

Figure 2 shows the flow of the proposed intra luma prediction mode coding. The most probable mode (MPM) coding is changed from one level in HM-4.0 to two levels. The first level is the same as the HM-4.0, and one more level of MPM coding, i.e. the most probable remaining mode (MPRM) coding is introduced. If the current intra prediction mode (ModeC) is not equal to one of the MPMs, prev_pred_mode_flag will be set as zero, and the MPRM coding will be used.

Figure 2.  The flow of the proposed intra luma prediction mode coding

In the MPRM coding, according to the intra prediction modes of the upper block (ModeA) and the left block (ModeL), three MPRMs are generated. If ModeC is equal to one of the MPRMs, mprm_pred_flag will be set as one, and the index of the selected MPRM, mprm_idx, will be represented by a truncated unary code. If ModeC is not equal to any MPRM, mprm_pred_flag will be set as zero, and rem_intra_luma_pred_mode will be derived and coded in a way similar to that in HM-4.0.

In the MPRM coding, to generate the three MPRMs, three intra prediction modes, (A1, A2, A3), are first selected from predefined tables with ModeA as the table input, as shown in Table 1 for prediction unit (PU) size equal to 4x4 and in Table 2 for PU size equal to 8x8, 16x16, or 32x32. Similarly, another three intra prediction modes, (L1, L2, L3) are then selected from Table 1 or Table 2 with ModeL as the table input. Roughly speaking, the first two output modes of Table 1 and Table 2 are two intra prediction modes that are closest to the input mode in prediction direction angle, and the third output mode of Table 1 and Table 2 chooses one of the two intra prediction modes that are second closest to the input mode in prediction direction angle or chooses DC prediction mode.

The derivation of MPRMs is as:

Table 1.  Neighboring mode table for PU size equal to 4x4

| Input | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1st output | 1 | 12 | 17 | 1 | 15 | 11 | 13 | 14 | 16 | 18 | 7 | 4 | 5 | 1 | 6 | 8 | 2 | 9 | 10 |
| 2nd output | 2 | 13 | 16 | 2 | 11 | 12 | 14 | 10 | 15 | 17 | 18 | 5 | 1 | 6 | 7 | 4 | 8 | 2 | 9 |
| 3rd output | 9 | 2 | 1 | 9 | 3 | 1 | 1 | 3 | 2 | 2 | 3 | 15 | 13 | 12 | 3 | 11 | 17 | 16 | 3 |

Table 2.  Neighboring mode table for PU size equal to 8x8, 16x16, or 32x32

| Input | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1st output | 1 | 22 | 31 | 1 | 27 | 20 | 24 | 26 | 29 | 33 | 7 | 19 | 21 | 23 | 25 | 28 | 30 | 32 | 34 |
| 2nd output | 2 | 23 | 30 | 2 | 19 | 21 | 25 | 10 | 28 | 32 | 34 | 20 | 22 | 24 | 26 | 27 | 29 | 31 | 33 |
| 3rd output | 9 | 2 | 1 | 9 | 3 | 1 | 1 | 3 | 2 | 2 | 3 | 15 | 13 | 12 | 3 | 11 | 17 | 16 | 3 |
| Input | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |  |  |
| 1st output | 4 | 11 | 5 | 12 | 1 | 13 | 6 | 14 | 15 | 8 | 16 | 2 | 17 | 9 | 18 | 10 |  |  |  |
| 2nd output | 11 | 5 | 12 | 1 | 13 | 6 | 14 | 7 | 4 | 15 | 8 | 16 | 2 | 17 | 9 | 18 |  |  |  |
| 3rd output | 27 | 19 | 22 | 23 | 22 | 23 | 3 | 3 | 19 | 27 | 30 | 31 | 30 | 31 | 3 | 3 |  |  |  |

- If ModeA is equal to ModeL, MPRM[n] is set as Ln, where n is equal to 0, 1, 2.

- Else if ModeA is equal to Planar, MPRM[n] is set as Ln, where n is equal to 0, 1, 2.

- Else if ModeL is equal to Planar, MPRM[n] is set as An, where n is equal to 0, 1, 2.

- Else, MPRM[0] is set as 0 (Planar) and the MPRM[1] and MPRM[2] are derived as:

  - If A0 is equal to ModeL, MPRM[1] is set as L0, and MPRM[2] is set as A1.

  - Else if L0 is equal to ModeA, MPRM[1] is set as A0, and MPRM[2] is derived as:

    - If A1 is equal to ModeL, MPRM[2] is set as L1.

    - Otherwise, MPRM[2] is set as A1.

  - Else, MPRM[1] is set as A0, and MPRM[2] is derived as:

    - If L0 is equal to A0, MPRM[2] is set as A1.

    - Otherwise, MPRM[2] is set as L0.

In fact, it is equivalent to interleave (A1, A2, A3) and (L1, L2, L3) as (A1, L1, A2, L2, A3, L3) and then pick the first three non-redundant intra prediction modes that are different from ModeA and ModeL.

## Proposed intra chroma prediction mode coding

As HM-4.0, six intra chroma prediction modes (intra_chroma_pred_mode[n], n = 0-5) can be signaled in the proposed method. The last two intra prediction modes, DM (direct mode: direct use of the coded intra luma prediction mode) and LM (luma mode: use reconstructed luma samples of the current PU, reconstructed luma samples of neighboring PUs, and reconstructed chroma samples of neighboring PUs to derive chroma prediction samples) remain unchanged. If the current PU size is equal to 64x64 or the intra luma prediction mode is equal to Planar or DC, the derivation of the first four intra chroma prediction modes is the same as that in HM-4.0 described in Section 1; otherwise, the derivation of the first four intra chroma prediction modes is as:

- intra_chroma_pred_mode[0] is set as Planar.

- intra_chroma_pred_mode[1] and intra_chroma_pred_mode[2] are set as two intra prediction modes adjacent to the coded intra luma prediction mode. That is, with the coded intra luma prediction mode as the input of Table 1 or Table 2, intra_chroma_pred_mode[1] is set as the 1st output mode and intra_chroma_pred_mode[2] is set equal as the 2nd output mode.

- intra_chroma_pred_mode[3] is set as the intra prediction mode perpendicular to the coded intra luma prediction mode. That is, with the coded intra luma prediction mode as the input of Table 3, intra_chroma_pred_mode[3] is set as the output.

Table 3.  Perpendicular mode table

| Input | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Output | 0 | 2 | 1 | 0 | 7 | 9 | 8 | 4 | 6 | 5 | 4 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 |
| Input | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |  |  |
| Output | 34 | 33 | 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 |  |  |  |

# Simulation results

The proposed approach is implemented on HM-4.0. The high efficiency all intra (HE-AI) condition and the low complexity all intra (LC-AI) condition were simulated according to JCTVC-F900 [1]. Two tests are shown. One is to use the neighboring mode tables as shown in Table 1 and Table 2. The other is to use the alternative neighboring mode tables as shown in Table 4 and Table 5. In these two tests, one intra prediction mode, (Hor. +6), is added for Intra_4x4 coding.

Table 4.  Alternative neighboring mode table for PU size equal to 4x4

| Input | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1st output | 1 | 12 | 17 | 1 | 11 | 12 | 13 | 10 | 16 | 17 | 7 | 4 | 1 | 1 | 3 | 4 | 2 | 2 | 10 |
| 2nd output | 2 | 13 | 16 | 2 | 15 | 11 | 14 | 14 | 15 | 3 | 3 | 5 | 5 | 6 | 6 | 8 | 8 | 9 | 9 |
| 3rd output | 9 | 3 | 1 | 9 | 3 | 1 | 1 | 3 | 2 | 2 | 14 | 15 | 13 | 12 | 7 | 11 | 17 | 16 | 3 |

Table 5.  Alternative neighboring mode table for PU size equal to 8x8, 16x16, or 32x32

| Input | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1st output | 1 | 22 | 31 | 1 | 19 | 21 | 3 | 3 | 29 | 3 | 3 | 19 | 22 | 23 | 3 | 27 | 30 | 31 | 3 |
| 2nd output | 2 | 23 | 30 | 2 | 27 | 20 | 24 | 10 | 28 | 32 | 7 | 20 | 21 | 24 | 25 | 28 | 29 | 32 | 33 |
| 3rd output | 32 | 2 | 1 | 32 | 3 | 3 | 25 | 34 | 3 | 33 | 26 | 4 | 1 | 1 | 26 | 4 | 2 | 2 | 34 |
| Input | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |  |  |
| 1st output | 4 | 11 | 12 | 1 | 1 | 13 | 3 | 3 | 4 | 15 | 16 | 2 | 2 | 17 | 3 | 3 |  |  |  |
| 2nd output | 11 | 5 | 5 | 12 | 13 | 6 | 6 | 7 | 15 | 8 | 8 | 16 | 17 | 9 | 9 | 10 |  |  |  |
| 3rd output | 20 | 3 | 3 | 23 | 22 | 3 | 14 | 10 | 28 | 3 | 3 | 31 | 30 | 3 | 18 | 7 |  |  |  |

Table 6 shows the results of the proposed intra prediction mode coding. The luma bit rate is reduced by 0.4% in the HE-AI and HE-LC. The chroma bit rate is reduced by 0.4%-0.5% and 0.7% in the HE-AI and HE-LC, respectively. The encoding times and decoding times are roughly unchanged.

Table 6.  Results of the proposed intra prediction mode coding

Table 7 shows the results of proposed intra prediction mode coding with alternative neighboring mode tables shown in Table 4 and Table 5. The luma bit rate is reduced by 0.4% and 0.5% in the HE-AI and HE-LC, respectively. The chroma bit rate is reduced by 0.3%-0.4% and 0.7% in the HE-AI and HE-LC, respectively. The encoding times and decoding times are roughly unchanged.

Table 7.  Results of the proposed intra prediction mode coding with alternative tables

# Conclusion

MediaTek and Qualcomm proposed a intra luma prediction mode coding method and a intra chroma prediction mode coding method. The intra luma prediction mode coding uses three additional most probable modes (MPMs) on top of the existing two MPMs, and a two-level MPM coding method is used. The intra chroma prediction mode coding replaces the original last three intra chroma prediction modes by two intra prediction modes adjacent to the coded intra luma prediction mode and one intra prediction mode perpendicular to the coded intra luma prediction mode. It is reported that 0.4%-0.5% luma bit rate reduction and 0.3-0.7% chroma bit rate reduction are achieved in the all intra conditions.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.