| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G205 |
| --- | --- |

| Title: | CE8.c.3: Multi-source SAO and ALF virtual boundary processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chih-Ming Fu, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Semih Esenlik, Matthias Narroschke, Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chihming.fu, chiayang.tsai, yuwen.huang, shawmin.lei}@mediatek.com Thomas Wedi +49 6103 766 1194 {Semih.Esenlik, Matthias.Narroschke, Thomas.Wedi}@panasonic.de |
| Source: | MediaTek Inc., Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution reports results of CE8.c.3. In HM-4.0, SAO requires 0.2 luma pixel line buffers (PLBs) and 0.2 chroma PLBs, and ALF requires 4.1 luma PLBs and four chroma PLBs for practical real-time decoders. In JCTVC-F054 and JCTVC-F055, virtual boundary (VB) processing was proposed to remove all these line buffers. In JCTVC-F272, partial use of pre-DF pixels as SAO and ALF inputs was proposed to reduce line buffers. In order to achieve zero line buffer and good visual quality for SAO and ALF, the two methods are combined as follows. Due to the DF in HM-4.0, luma VBs and chroma VBs are set as four and two pixels above horizontal LCU boundaries, respectively. For to-be-processed pixels above the VB, any required pixel below the VB is replaced by a pre-DF pixel. For to-be-processed pixels below the VB, any pixel above the VB is avoided by modifying pixel classification for SAO and filter shapes for ALF. When compared with the JCTVC-F900 anchor, the proposed method reportedly causes 0.1%, 0.1%, 0.1%, and 0.2% coding efficiency losses for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, and is claimed to have similar visual quality as the anchor. Very minor VB artifacts can only be seen in very few pictures.

# Introduction

In High-Efficiency Video Coding test model version 4.0 (HM-4.0), in-loop filtering contains deblocking filter (DF), sample adaptive offset (SAO), and adaptive loop filter (ALF). These in-loop filters can significantly enhance coding efficiency; however, they require pixel line buffers (PLBs) in practical real-time decoders. These line buffers can significantly increase the on-chip memory size, especially for videos with large picture widths. Therefore, it is desirable to reduce line buffers with acceptable coding efficiency loss. In JCTVC-F054 [1] and JCTVC-F055 [2], MediaTek proposed virtual boundary (VB) processing to remove all SAO and ALF line buffers. In JCTVC-F272 [3], Panasonic proposed partial use of pre-DF pixels as SAO and ALF inputs to reduce a portion of SAO and ALF line buffers. After the 6th JCT-VC meeting in Torino, Core Experiment 8 (CE8) for non-DF in-loop filtering [4] was established, and the line buffer reduction proposals were included in CE8. In this contribution, MediaTek’s and Panasonic’s methods are combined, and results are reported. The rest of this contribution is organized as follows. The line buffer requirement of the in-loop filtering in HM-4.0 will be analyzed in Section 2, virtual boundary processing will be proposed in Section 3, simulation results will be shown in Section 4, and finally conclusions will be given in Section 5.

# Line buffer analysis

## Deblocking filter

In HM-4.0, DF applies filtering on 8x8 block boundaries. Figure 1 shows a vertical boundary on the left side of the figure and a horizontal boundary side on the right side of the figure, where 8x8 blocks are partly shown (4x8 or 8x4). In one picture, all vertical boundaries can be horizontally filtered in parallel, and then all horizontal boundaries can be vertically filtered in parallel. During processing of a luma boundary, four pixels of each side (p0-p3, q0-q3) are involved in filter parameter derivation, and at most three pixels of each side (p0-p2, q0-q2) can be changed after filtering. For luma horizontal DF, pre-DF pixels (i.e. pixels before horizontal DF) are used for deriving filter parameters and filtering. For luma vertical DF, pre-DF pixels are used for deriving filter parameters, and H-DF pixels (i.e. pixels after horizontal DF) are used for filtering. During processing of a chroma boundary, two pixels of each side (p0-p1, q0-q1) are involved in filter parameter derivation, and at most one pixel of each side (p0, q0) is changed after filtering. For chroma horizontal DF, pre-DF pixels are used for deriving filter parameters and filtering. For chroma vertical DF, H-DF pixels are used for deriving filter parameters and filtering.

Figure 1.  Illustration of 8x8 block boundaries for DF

## Sample adaptive offset

In HM-4.0, SAO is applied to luma and chroma, and each component is independently processed. SAO can divide one picture into multiple LCU-aligned regions, and each region can select one SAO type among two band offset (BO) types, four edge offset (EO) types, and no processing (OFF). For each to-be-processed pixel, BO uses the pixel intensity to classify the pixel into a band. The pixel intensity range is equally divided into 32 bands, as shown in Figure 2. After pixel classification, one offset is derived for all pixels of each band, and the offsets of center 16 bands or outer 16 bands are selected and coded. As for EO, it uses two neighboring pixels of a to-be-processed pixel to classify the pixel into a category. Similarly, one offset is derived for all pixels of each category after pixel classification except for category 0, which is forced to use zero offset. Table 1 shows the EO pixel classification, where “c” denotes the pixel to be classified. Figure 3 shows four EO types with different selections of neighboring pixels.

Figure 2.  Pixel intensity range equally divided into 32 bands by BO

Table 1.  EO pixel classification

Figure 3.  Four EO types

## Adaptive loop filter

In HM-4.0, ALF has two filter shape options, cross11x5 and snowflake5x5, for luma and chroma, as shown in Figure 4. In each picture, the luma component can choose one shape, and the chroma components can choose one shape. Up to 16 luma filters and at most one chroma filter can be applied for each picture. In order to allow localization of ALF, there are two modes for luma pixels to select filters. One is a region-based adaptation (RA) mode, and the other is a block-based adaptation (BA) mode. The RA mode divides one luma picture into 16 regions. Once the picture size is known, the 16 regions are determined and fixed. The regions can be merged, and one filter is used for each region after merging. The BA mode uses edge activity and direction as a property for each 4x4 block. Calculating the property of a 4x4 block requires 5x5 pixels, as shown in Figure 5. After properties of 4x4 blocks are calculated, they are classified into 15 categories. The categories can be merged, and one filter is used for each category after merging. As for the chroma components, since they are relatively flat, no local adaptation is used, and the two components of a picture share one filter.

Figure 4.  Two filter shape options

Figure 5.  Pixels required for edge property calculation of a 4x4 block in BA mode

## LCU-based decoding

Figure 6 shows the current implementation of HM-4.0 software decoder. The decoding process is divided into two parts. One is LCU-based processing including intra prediction (IP), motion compensation (MC), inverse transform (IT), inverse quantization (IQ), and reconstruction (REC), and the other is picture-based processing including DF, SAO, and ALF. Entropy decoding (ED) belongs to the picture-based processing when SPS, PPS, or slice-level syntax elements are parsed and belongs to the LCU-based processing when syntax elements of LCUs are parsed. In PC-based software, picture-based processing is easier to implement than LCU-based processing for DF, SAO, and ALF. However, in hardware and embedded software, picture-based processing is practically unacceptable due to its high picture buffer requirement. Using on-chip picture buffers is very expensive, and using off-chip picture buffers significantly increases external memory access, power consumption, and data access latency. Therefore, DF, SAO, and ALF will definitely be changed from picture-based to LCU-based decoding in real products.

Figure 6.  Block diagram of HM-4.0 software decoder

When LCU-based processing is used for DF, SAO, and ALF, the entire decoding process can be done LCU by LCU in a raster scan with an LCU-pipelining fashion for parallel processing of multiple LCUs. In this case, line buffers are required for DF, SAO, and ALF because processing one LCU row requires pixels from the above LCU row. If off-chip line buffers (e.g. DRAM) are used, the external memory bandwidth and power consumption will be increased; if on-chip line buffers (e.g. SRAM) are used, the chip area will be increased. Therefore, although line buffers are already much smaller than picture buffers, it is still desirable to reduce line buffers.

## Luma line buffers of in-loop filtering

Figure 7 explains the number of luma line buffers required for DF, SAO, and ALF with LCU-based decoding. Given a current LCU, lines …A-J are first processed by horizontal DF and then by vertical DF. Next, it is suggested to delay the horizontal DF for lines K-N until the lower LCU comes in order to avoid line buffers of H-DF pixels for the vertical DF for lines K-N, which cannot be processed at this moment because the lower LCU is not yet available and DF needs four pre-DF pixels and four H-DF pixels on each side of the horizontal boundary for deriving filter parameters and filtering, respectively, as illustrated by the 4-pixel yellow stripe. Therefore, four lines (K-N) of pre-DF pixels have to be stored for DF. Next, SAO is going to be applied on DF output pixels. Since the vertical DF for lines K-N will not change line K, horizontal DF can be additionally applied on line K for SAO to process line J, as illustrated by the 3x3 red square. Please note that the H-DF pixels of line K will not be stored in the line buffer and have to be generated again when the lower LCU comes, which is not a problem in hardware. After SAO processes lines …A-J, the 4x4 block property, as illustrated by the 4x4 blue square, can be calculated. At this moment ALF can process lines …A-H. After this, no further process can be done for the current LCU until the lower LCU comes. When the lower LCU comes, lines K-P… are first processed by DF, and then lines K-P… are processed by SAO. When SAO processes line K, line J is required. Fortunately, only EO partial results from comparing pixels on line K and pixels on line J have to be stored instead of pixel values. The partial results need 2 bits per pixel, which requires only 20% and 25% of the pixel line buffer for high efficiency (HE) conditions using 10-bit pixels and low complexity (LC) conditions using 8-bit pixels, respectively. Therefore, one line (J) of SAO partial results has to be stored for SAO. Next, the 4x4 block properties for lines I-P… are calculated. Finally, lines I-P… can be filtered by ALF. When line I is filtered, it requires lines G-K, as illustrated by the filter shape painted in green. However, calculating block properties of lines I-J still needs lines F-J. Therefore, five lines (F-J) of SAO output pixels have to be stored for ALF. If a filter index line buffer can be used to store BA mode filter selections for lines G-J, it is not necessary to compute the block properties again during filtering of lines I-J. In this way, one line (F) of SAO output pixels can be saved for ALF. The filter index line buffer (4 bits per block) requires only about 10% size of the pixel line buffer. Therefore, four lines (G-J) of SAO output pixels and one line of filter indices (4x4 blocks on lines G-K) have to be stored for ALF. In total, the entire in-loop filtering requires about 8.3 luma line buffers. When the entire decoding system is considered, since the intra luma prediction already stores one line (N) of pre-DF pixels, this luma line buffer can be shared.

Figure 7.  Illustration of luma line buffers required for DF, SAO, and ALF

## Chroma line buffers of in-loop filtering

Figure 8 explains the number of chroma line buffers required for DF, SAO, and ALF with LCU-based decoding. Given a current LCU, lines …A-M are first processed by horizontal DF, and then lines …A-L are processed by vertical DF. Next, it is suggested to delay the horizontal DF for line N until the lower LCU comes in order to share the line buffer of pre-DF pixels of line N with intra chroma prediction. As for the vertical DF for lines M-N, it has to wait for the lower LCU. Please recall that chroma DF reads two H-DF pixels and may write one pixel on each side of the boundary, as illustrated by the 2-pixel yellow stripe. Therefore, one line (M) of H-DF pixels and one line (N) of pre-DF pixels have to be stored for DF. Next, SAO is going to be applied on DF output pixels. Since the vertical DF for lines M-N will not change line M, H-DF pixels of line M are also DF output pixels of line M, and SAO can process line L, as illustrated by the 3x3 red square. After SAO processes lines …A-L, ALF can process lines …A-J. After this, no further process can be done for the current LCU until the lower LCU comes. When the lower LCU comes, lines N-P… are first processed by horizontal DF, and then lines M-P… are processed by vertical DF and by SAO. When SAO processes line M, line L is required. Fortunately, only EO partial results from comparing pixels on line M and pixels on line L have to be stored instead of pixel values. The partial results need 2 bits per pixel, which requires only 20% and 25% of the pixel line buffer for HE conditions using 10-bit pixels and LC conditions using 8-bit pixels, respectively. Therefore, one line (L) of SAO partial results has to be stored for SAO. Finally, lines K-P… can be filtered by ALF. When line K is filtered, it requires lines I-M, as illustrated by the filter shape painted in green. Therefore, four lines (I-L) of SAO output pixels have to be stored for ALF. In total, the entire in-loop filtering requires about 6.2 chroma line buffers.

Figure 8.  Illustration of chroma line buffers required for DF, SAO, and ALF

# Proposed virtual boundary processing

## Virtual boundary

In order to eliminate the line buffer requirements of SAO and ALF, we introduce the concept of virtual boundary (VB). As shown in Figure 9, VBs are upward shifted horizontal LCU boundaries by N pixels. For each LCU, SAO and ALF can process pixels above the VB before the lower LCU comes but cannot process pixels below the VB until the lower LCU comes, which is caused by DF. With consideration of the DF in HM-4.0, the space between the proposed VB and the horizontal LCU boundary is set as four pixels for luma (i.e. N=4 in Figure 9) and two pixels for chroma (i.e. N=2 in Figure 9). SAO and ALF are modified to ensure that every to-be-processed pixel on one side of a VB does not require any data access from the other side of the VB unless the data can become available in time without using any additional line buffer.

Figure 9.  Illustration of VBs in one picture

Figure 7 can be used to explain the luma case where the VB is painted in red and four pixels above the horizontal LCU boundary. When the current LCU is processed, lines K-N cannot be processed by vertical DF because the lower LCU is not yet available. At this moment, if SAO and ALF processes for lines …A-J do not require any pixel below the VB, lines …A-J can be processed without the lower LCU. When the lower LCU comes, lines K-P… are processed by DF, SAO, and ALF. At this time, if SAO and ALF processes for lines K-P… do not require any pixel above the VB, line buffers for storing lines F-J can be removed.

Figure 8 can be used to explain the chroma case where the VB is painted in red and two pixels above the horizontal LCU boundary. When the current LCU is processed, lines M-N cannot be processed by vertical DF because the lower LCU is not yet available. At this moment, if SAO and ALF processes for lines …A-L do not require any pixel below the VB, lines …A-L can be processed without the lower LCU. When the lower LCU comes, lines M-P… are processed by DF, SAO, and ALF. At this time, if SAO and ALF processes for lines M-P… do not require any pixel above the VB, line buffers for storing lines I-L can be removed.

With the proposed VB principle, ALF and SAO do not need any line buffer for LCU-based decoding, and the number of line buffers for the entire in-loop filtering is reduced from 8.3 to four for luma and from 6.2 to two for chroma.

## Virtual boundary processing for SAO

The general concept of VB processing is to prevent accessing any pixel on the other side of the VB. Consequently, in SAO, only 45-, 90-, and 135-degree EO types are relevant and need to be modified for the first lines of to-be-processed pixels beside the VB. The proposed VB processing for SAO can be explained by using Figure 10. Figure 10(a), Figure 10(b), and Figure 10(c) show the neighboring pixels for 90-, 135-, and 45-degree EO types, respectively. Figure 10(d), Figure 10(e), and Figure 10(f) are the cases where the current to-be-processed pixel is on the first line above the VB. Figure 10(g), Figure 10(h), and Figure 10(i) are the cases where the current to-be-processed pixel is on the first line below the VB.

Figure 10.  Different cases of virtual boundary processing for SAO

The concept of the proposed VB processing for SAO is to use the offset from a neighboring pixel according to the EO type direction with a sanity check when the neighboring pixel is similar to the current to-be-processed pixel and to use zero offsets in the rest cases that originally need cross-VB data access. In Figure 10, c is the current to-be-processed pixel, a is the vertically neighboring pixel on the same side of the VB, b is the to-be-compared neighboring pixel on the same side of the VB, and d is the to-be-avoided pixel on the other side of the VB. The proposed VB processing for SAO is in Figure 11. In this proposal, TH is set to 1, so if c is equal to a and the sanity check is passed, c will use the b’s offset; otherwise, c will use zero offset. Please note that for 90-degree EO, a and b are the same pixel. Since there is only one neighboring pixel to be compared because of the proposed non-cross-VB data access, the EO pixel classification is modified as Table 2, where the EO category of c can only be 2, 3 or 0. The sanity check can be passed in the following conditions. If c’s EO category is 2 in Table 2, the b’s EO category can only be 1, 2, or 0 in Table 1 to pass the sanity check. Similarly, if c’s EO category is 3 in Table 2, the b’s EO category can only be 3, 4, or 0 in Table 1 to pass the sanity check. If c’s EO category is 0 in Table 2, the sanity check is always passed.

Figure 11.  Proposed virtual boundary processing for SAO

Table 2.  Modified EO pixel classification

## Virtual boundary processing for ALF

In ALF decoding, there are two major procedures: block property calculation and filtering. During block property calculation, when cross-VB data access is needed in the original block property calculation process, vertically repetitive padding is used to replace pixels on the other side of the VB by pixels of the first line on the same side of the VB. During filtering, we modify the filter shapes when cross-VB data access is needed in the original filtering process. Figure 12 shows different cases of VB processing for ALF, where each square is a pixel, the solid square is the to-be-processed pixel, the red line is the VB, the upper part denotes cases for the snowflake5x5 filter shape, and the lower part denotes cases for the cross11x5 filter shape. In order to avoid cross-VB data access, we propose to skip filtering or apply partial filtering in different cases. In partial filtering, filter taps on the other side of the VB and their symmetric taps on the same side of the VB are not used, and these unused coefficients are added to the coefficient of the center pixel to make the output pixel closer to the to-be-processed pixel than the normal filtering. Applying partial filtering can result in better objective coding efficiency than skipping filtering; however, partial filtering might cause undesirable visual artifacts and degrade subjective qualities when a large portion of coefficients is not used. Figure 12 (b) and (c) show the cases for to-be-processed pixels on the first line of the VB. In these cases, if the snowflake5x5 is selected, filtering is skipped because a significant portion of the filter footprint is on the other side of the VB; otherwise (if the cross11x5 is selected), partial filtering is applied. Figure 12 (a) and (d) show the cases for to-be-processed pixels on the second line of the VB. In these cases, both the snowflake5x5 and the cross11x5 use partial filtering because the portion of filter footprint on the other side of the VB is relatively small. It is safe to apply partial filtering without degrading subjective qualities. Please note that there is no additional coded coefficient, and the change of coefficients and filter shapes is only applied during ALF decoding at VBs.

(a)                (b)                 (c)                 (d)

Figure 12.  Different cases of virtual boundary processing for ALF

## Using pre-DF pixels

In order to achieve good coding efficiency and visual quality and to remove all SAO and ALF line buffers simultaneously, MediaTek’s and Panasonic’s methods can be combined as follows. For to-be-processed pixels above the VB, the below-VB pixels, which are originally required by SAO and ALF in HM-4.0 (i.e. lines K-N in Figure 7 and lines M-N in Figure 8), can be replaced by pre-DF pixels, which are available before the lower LCU comes. That is, in Figure 11 (d), (e), and (f), SAO does not use a in the VB processing and use the pre-DF pixel to replace the SAO input pixel for d, and in Figure 12 (c) and (d), ALF input pixels below the VB are replaced by the pre-DF pixels, and the filter shapes are kept the same during filtering. During SAO and ALF processes for to-be-processed pixels above the VB, using pre-DF pixels below the VB can provide more information than using nothing below the VB, so the coding efficiency and visual quality can be further improved. For to-be-processed pixels below the VB, the non-cross-VB data access principle is still applied when they are processed after the lower LCU comes in order to remove all line buffers. That is, the VB processing cases of Figure 11 (g), (h), and (i) and Figure 12 (a) and (b) described in Section 3.2 and Section 3.3 are still required.

# Simulation results

HM-4.0 is used as the anchor. High efficiency all intra (HE-AI), high efficiency random access (HE-RA), high efficiency low delay B (HE-LDB), high efficiency low delay P (HE-LDP), low complexity all intra (LC-AI), low complexity random access (LC-RA), low complexity low delay B (LC-LDB), and low complexity low delay P (LC-LDP) common test conditions defined in JCTVC-F900 [5] are tested. Table 3 shows that the proposed multi-source SAO and ALF VB processing results in 0.1%, 0.1%, 0.1%, and 0.2% luma BD-rate differences for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, where positive numbers mean losses and negative numbers mean gains. Table 3 also shows that the proposed multi-source SAO and ALF VB processing results in 0.0%, 0.0%, 0.0%, and 0.1% luma BD-rate differences for LC-AI, LC-RA, LC-LDB, and LC-LDP, respectively, when ALF is disabled. The encoding time is roughly unchanged, while the decoding time is slightly increased by 2-3%. As for the subjective quality, after carefully viewing of all video sequences, we observed that the visual quality of this proposal is very close to that of the anchor. Very minor VB artifacts can only be seen in very few pictures. Interested readers are strongly encouraged to contact us for evaluating the visual quality.

Table 3.  Results of CE8.c.3, multi-source SAO and ALF VB processing

# Conclusions

MediaTek and Panasonic implemented multi-source SAO and ALF virtual boundary processing to remove all SAO and ALF line buffers in LCU-based decoding systems. Simulation results reportedly show that the proposed virtual boundary processing achieves 0.1%, 0.1%, 0.1%, and 0.2% losses for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, with similar subjective quality in comparison with HM-4.0.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei, “Adaptive Loop Filter with Zero Pixel Line Buffers for LCU-based Decoding,” Document of Joint Collaborative Team on Video Coding, JCTVC-F054, July 2011.

- C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei, “Sample Adaptive Offset with Zero Pixel Line Buffers for LCU-based Decoding,” Document of Joint Collaborative Team on Video Coding, JCTVC-F055, July 2011.

- S. Esenlik, M. Narroschke, T. Wedi, “CE8 subtest 3: Line memory reduction for in-loop filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-F272, July 2011.

- T. Yamakage, I. S. Chong, M. Narroschke, Y.-W. Huang, “Description of Core Experiment 8 (CE8): Non-deblocking loop filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-F908, July 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.