##### Modification process for luma and chroma samples

Inputs to this process are:

–	a variable cIdx specifying chroma component index,

–	a sample position ( xC, yC ),

–	a pair of variables ( rx, ry ) specifying the region index,

–	a variable saoDepth specifying the split depth of the region,

–	an array saoValueArray specifying offset values,

–	a block size nS.

Output of this process is a modified picture buffer for the chroma component cIdx.

Let recSaoPicture represents the processed sample array of the current picture of chroma component cIdx and saoTypeIdx is set equal to sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ].

Let preDF represents the processed sample array of the current picture of chroma component cIdx before deblocking process.

Variable bitDepth is set equal to BitDepthY if cIdx is equal to 0, otherwise, set equal to BitDepthC.

Variable LcuHeight is set equal to 1<<Log2MaxCUSize if cIdx is equal to 0, otherwise, set equal to 1<< (Log2MaxCUSize-1).

Variable VBLine is set equal to LcuHeight-4 if cIdx is equal to 0, otherwise, set equal to LcuHeight-2.

Depending on the value of saoTypeIdx, the following applies:

–	If saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Table 8-1.

- A variable edgeIdx is specified as

if ( (yC+j) % LcuHeight == VBLine-1  &&  yC+ LcuHeight < img_Height  &&  saoTypeIdx != 1)

{

edgeIdx = 2 + Sign( recPicture[ xC + i, yC + j ] – recPicture[ xC + i + hPos[0], yC + j + vPos[0] ] )

+  Sign( recPicture[ xC + i, yC + j ] – preDF[ xC + i + hPos[1], yC + j + vPos[1] ] );	(8-1)

}

else if ( (yC+j) % LcuHeight == VBLine  &&  yC+ LcuHeight < img_Height  &&  saoTypeIdx != 1)

{

edgeIdx  = 2;

if (recPicture[ xC + i, yC + j ] == recPicture[ xC + i, yC + j + 1 ] )

{

edgeIdxP = Sign( recPicture[ xC + i, yC + j ] – recPicture [ xC + i + hPos[1], yC + j + vPos[1] ] );

edgeIdx1  = 2 + ∑k( Sign( recPicture [xC + i + hPos[1], yC + j + vPos[1] ] –

recPicture[ xC + i + hPos[1] + hPos[ k ], yC + j + vPos[1]  + vPos[ k ] ] ) ) with k = 0..1

edgeIdx  = (edgeIdxP * edgeIdx1   >= 0) ? edgeIdx1  :  2 ;

}

}

else

edgeIdx = 2 + ∑k( Sign( recPicture[ xC + i, yC + j ] –

recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-2)

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + saoValueArray[ edgeTable[ edgeIdx ] ]	(8-3)

with i = 0..nS-1 and j = 0..nS-1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

–	Otherwise, if saoTypeIdx is equal to one of the values of 5 or 6, the following ordered steps applies:

- A variable bandShift is set equal to BitDepthY – 5 if cIdx is equal to 0, otherwise, set equal to BitDepthC – 5.

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] +

saoValueArray[ bandTable[ saoTypeIdx – 5 ][ bandIdx ] ]	(8-4)

with i = 0..nS-1 and j = 0..nS-1 where bandIdx is set equal to ( recPicture[ xC + i, yC + j ] >> bandShift ) and bandTable is specified in Table 8-2.

–	Otherwise (sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] is equal to 0), the following applies:

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] with i = 0..nS-1 and j = 0..nS-1	(8-5)

[Ed. (WJ): copy operation is necessary to use recSaoPicture later.]

Table 8-1 – Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | -1 | 0 | -1 | 1 |
| hPos[1] | 1 | 0 | 1 | -1 |
| vPos[0] | 0 | -1 | -1 | -1 |
| vPos[1] | 0 | 1 | 1 | 1 |

Table 8-2 – Specification of array bandTable according to bandIdx

| bandIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| bandTable[1][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| bandTable[0][bandIdx] | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |

#### Derivation process for filter index array for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

Output of this process is the two-dimensional filter index array of (nS)x(nS), fIdx.

A variable nS is set equal to ( 1 << log2CUSize ).

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. The boundary padding process specified in subclause Error: Reference source not found shall be applied to generate the padded sample of pre-deblocked pixels, preDF. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

The filter index array fIdx is specified in the follows:

- When alf_region_adaptation_flag is equal to 1, the following ordered steps apply.

- The variables xIdx and yIdx are derived as

regionTab[16] = { 0, 1, 4, 5, 15, 2, 3, 6, 14, 11, 10, 7, 13, 12, 9, 8 }	(8-6)
	offset = 1 << (Log2MaxCUSize – 1) 		(8-7)
	xInterval = ( ( ( PicWidthInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-8)
	yInterval = ( ( ( PicHeightInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-9)
	xIdx = Min( 3, Floor( ( xC + x ) / ( xInterval << Log2MaxCUSize ) ) )	(8-10)
	yIdx = Min( 3, Floor( ( yC + y ) / ( yInterval << Log2MaxCUSize ) ) )	(8-11)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = regionTab[ ( yIdx << 2 ) + xIdx ]		(8-12)

- Otherwise (alf_region_flag is equal to 0), the following ordered steps apply.

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] with x, y = 0, 2, 4,...(nS) is derived as

LcuHeight = 1 << Log2MaxCUSize;                                                        
   VBLine = LcuHeight – 4;
   if ( (yC+y)  % LcuHeight == VBLine  &&  yC+ LcuHeight < img_height )
  {
     varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y ] – s’’[ xC+x+1, yC+y ] |	(8-13)   
     varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x, yC+y ] – s’’[ xC+x, yC+y+1 ] |	(8-14)
   }
   else
  {
     varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y ] – s’’[ xC+x+1, yC+y ] |	(8-15)   
     varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x, yC+y-1 ] – s’’[ xC+x, yC+y+1 ] |	(8-16) 
  }

- The variables varTempH1[ x, y ], varTempV1[ x, y ], and varTemp3[ x, y ] with x, y = 0..( (nS) – 1 )>>2 are derived as

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 0, 2	(8-17)
	varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 0, 2	(8-18)
	varTemp3[ x ][ y ] = ( varTempH1[ x ][ y ] + varTempV1[ x ][ y ] ) >> 2	(8-19)

- The variable direction is derived as

- If varTempV1[ x >> 2 ][ y >> 2 ] is greater than varTempH1[ x >> 2 ][ y >> 2 ] << 1,

direction = 1

- Otherwise, if varTempH1[ x >> 2 ][ y >> 2 ] is greater than varTempV1[ x >> 2 ][ y >> 2 ] << 1,

direction = 2

- Otherwise,

direction = 0

- The variable avgvar is derived as

varTab[16] = { 0, 1, 2, 2, 2, 3, 3, 3, 3, 3, 4, 4, 4, 4, 4, 4 }	(8-20)
	avgVar = Clip3( 0, 15, (varTemp3[ x >> 2 ][ y >> 2 ] * 1024 ) >> (3 + BitDepthY) )	(8-21)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = Clip3( 0, 4, varTab[avgVar] ) + 5 * direction	(8-22)

#### Filtering process for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a filter index array of (nS)x(nS), fIdx.

Output of this process is the filtered reconstruction of luma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. The boundary padding process specified in subclause Error: Reference source not found shall be applied to generate the padded sample of pre-deblocked pixels, preDF. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfFilterShape is set equal to alf_filter_shape_flag.

Each sample of luma picture recFiltPictureL[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, is derived as follows.

LcuHeight = 1 << Log2MaxCUSize;                                                        
 VBLine = LcuHeight – 4;
 if ( (yC + y) % LcuHeight == VBLine-2  &&  yC+ LcuHeight < img_height  )
{
    if (alf_filter_shape_flag == 0)     
   {
      recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=013s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=1416preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]
   }
   else 
   {
       recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=013s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=1414preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]
    }
}
else if ( (yC + y) % LcuHeight == VBLine-1  &&  yC+ LcuHeight < img_height  )
{
   if (alf_filter_shape_flag == 0)     
   {
      recFiltPictureL[xC+x][yC+y]=             (cL[fIdx[x][y]][N]+i=010s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=1116preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            
   }
   else 
   {
       recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=012s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=1314preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]
    }
}
else if ( (yC + y) % LcuHeight == VBLine  &&  yC+ LcuHeight < img_height  )
{
   if (alf_filter_shape_flag == 0)     
   {
      recFiltPictureL[xC+x][yC+y]=s''[xC+x,yC+y]
   }
   else 
   {
       recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=212s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=012∗s''[xC+x,yC+y]∗cL[fIdx[x][y]][i]
    }
}
else if ( (yC + y) % LcuHeight == VBLine+1  &&  yC+ LcuHeight < img_height   )
{  
    if (alf_filter_shape_flag == 0)     
   {
      recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=313s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=022∗s''[xC+x,yC+y]∗cL[fIdx[x][y]][i]
   }
   else 
   {
       recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=113s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=002∗s''[xC+x,yC+y]∗cL[fIdx[x][y]][i]
    }
}
else
{     
    recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]    (8-23)
}

where N is set equal to AlfNumCoeffLuma-1 and horPos[i] and verPos[i] are specified in Error: Reference source not found and Error: Reference source not found, respectively.

#### Filtering process for chroma samples

Inputs of this process are:

–	a chroma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

–	a variable cIdx specifying the chroma component index.

Output of this process is the filtered reconstruction of chroma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the chroma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx, and the output is assigned to the luma sample array s’’. The boundary padding process specified in subclause Error: Reference source not found shall be applied to generate the padded sample of pre-deblocked pixels, preDF. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

A variable nS is set equal to ( 1 << log2CUSize ) and variable alfFilterShape is set equal to alf_filter_shape_flag.

Filtered samples of chroma picture recFiltPicture[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, are derived as follows.

LcuHeight = 1 << (Log2MaxCUSize-1);                                                        
 VBLine = LcuHeight – 2;
 if ( (yC + y) % LcuHeight == VBLine-2  &&  yC+ LcuHeight < img_height  )
{
    if (alf_filter_shape_flag == 0)     
   {
      recFiltPicture[xC+x][yC+y]=             cC[N]+i=013s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=1416preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]
   }
   else 
   {
       recFiltPicture[xC+x][yC+y]=             cC[N]+i=013s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=1414preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]
    }
}
else if ( (yC + y) % LcuHeight == VBLine-1  &&  yC+ LcuHeight < img_height  )
{
   if (alf_filter_shape_flag == 0)     
   {
      recFiltPicture[xC+x][yC+y]=             (cC[N]+i=010s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=1116preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            
   }
   else 
   {
       recFiltPictureC[xC+x][yC+y]=             cC[N]+i=012s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=1314preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]
    }
}
else if ( (yC + y) % LcuHeight == VBLine  &&  yC+ LcuHeight < img_height  )
{
   if (alf_filter_shape_flag == 0)     
   {
      recFiltPicture[xC+x][yC+y]=s''[xC+x,yC+y]
   }
   else 
   {
       recFiltPicture[xC+x][yC+y]=             cC[N]+i=212s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=012∗s''[xC+x,yC+y]∗cC[i]
    }
}
else if ( (yC + y) % LcuHeight == VBLine+1  &&  yC+ LcuHeight < img_height   )
{  
    if (alf_filter_shape_flag == 0)     
   {
      recFiltPicture[xC+x][yC+y]=             cC[N]+i=313s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=022∗s''[xC+x,yC+y]∗cC[i]
   }
   else 
   {
       recFiltPicture[xC+x][yC+y]=             cC[N]+i=113s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=002∗s''[xC+x,yC+y]∗cC[i]
    }
}
else
{     
    recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]    (8-24)
}

where N is set equal to AlfNumCoeffChma-1 and horPos[i] and verPos[i] are specified in Error: Reference source not found and Error: Reference source not found, respectively.