| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G206 |
| --- | --- |

| Title: | CE8.c.4: SAO and ALF virtual boundary processing with cross9x9 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chia-Yang Tsai, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Semih Esenlik, Matthias Narroschke, Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai, chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com Thomas Wedi +49 6103 766 1194 {Semih.Esenlik, Matthias.Narroschke, Thomas.Wedi}@panasonic.de |
| Source: | MediaTek Inc., Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution reports results of CE8.c.4-1 and CE8.c.4-2, which are a combination of CE8.c.2 using single-source SAO and ALF virtual boundary (VB) processing and CE8.d.1 using cross9x9 and snowflake5x5 and a combination of CE8.c.3 using multi-source SAO and ALF VB processing and CE8.d.1, respectively. When compared with the JCTVC-F900 anchor, CE8.c.4-1 reportedly achieves 0.0%, -0.2%, -0.3%, and 0.1% BD-rates for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, and CE8.c.4-2 reportedly achieves 0.0%, -0.2%, -0.4%, -0.1% BD-rates for the four conditions respectively, where negative numbers mean gains and positive numbers mean losses. The gain of using cross9x9 is roughly unchanged when CE8.c.2 and CE8.c.3 are considered. It is also reported that both CE8.c.4-1 and CE8.c.4-2 have subjective qualities close to HM-4.0, and CE8.c.4-2 is better than CE8.c.4-1.

# Introduction

In High-Efficiency Video Coding test model version 4.0 (HM-4.0), in-loop filtering contains deblocking filter (DF), sample adaptive offset (SAO), and adaptive loop filter (ALF). In Core Experiment 8 (CE8) [1], non-DF loop filtering issues are studied. CE8.c.2 [2] is from JCTVC-F054 and JCTVC-F055 to remove SAO and ALF line buffers with single-source SAO and ALF. CE8.c.3 [3] is from JCTVC-F054, JCTVC-F055, and JCTVC-F272 to remove SAO and ALF line buffers with multi-source SAO and ALF. CE8.d.1 [4] shows results of using cross9x9 and snowflake5x5 as ALF shapes. In this contribution, the two line buffer reduction proposals are combined with the new filter shape proposal. The rest of this contribution is organized as follows. The applied virtual boundary (VB) processing will be explained in Section 2, simulation results will be provided in Section 3, and conclusions will be shown in Section 4.

# Proposed virtual boundary processing

In CE8.c.4-1, the VB processing for SAO is exactly the same as that of CE8.c.2. The VB processing for ALF using snowflake5x5 is also the same as that of CE8.c.2. The VB processing for ALF using cross9x9 is similar to that for ALF using cross11x5 in CE8.c.2, where the first and second lines beside the VB use padding and averaging, and the third and fourth lines beside the VB use padding without averaging.

In CE8.c.4-2, the VB processing for SAO is exactly the same as that of CE8.c.3. The VB processing for ALF using snowflake5x5 is different. As shown in Figure 12 of CE8.c.3, the first line skips filtering; however, the second line uses the modified filtering (padding and averaging) described in Section 3.3 of CE8.c.2. The VB processing for ALF using cross9x9 is similar to that for ALF using cross11x5, where the first, second, third, and fourth lines beside the VB all use the partial filtering described in Section 3.3 CE8.c.3.

# Simulation results

HM-4.0 is used as the anchor. High efficiency all intra (HE-AI), high efficiency random access (HE-RA), high efficiency low delay B (HE-LDB), and high efficiency low delay P (HE-LDP) common test conditions in JCTVC-F900 [5] are tested. Low complexity (LC) common test conditions are not tested because VB processing are unchanged, and results can be found in CE8.c.2 and CE8.c.3 reports. Table 1 shows that CE8.c.4-1, which combines CE8.c.2 (single-source SAO and ALF VB processing) and CE8.d.1 (cross9x9 and snowflake5x5), results in 0.0%, -0.2%, -0.3%, and 0.1% luma BD-rate differences for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, where negative numbers mean gains and positive numbers mean losses. Table 2 shows that CE8.c.4-2, which combines CE8.c.3 (multi-source SAO and ALF VB processing) and CE8.d.1, results in 0.0%, -0.2%, -0.4%, and -0.1% luma BD-rate differences for the four conditions respectively. The gain of using cross9x9 and snowflake5x5 in comparison with using cross11x5 and snowflake5x5 is roughly unchanged when CE8.c.2 and CE8.c.3 are applied. As for the subjective quality, after carefully viewing of all video sequences, we observed that the visual qualities of CE8.c.4-1 and CE8.c.4-2 are very close to that of the anchor, and CE8.c.4-2 is better than CE8.c.4-1. Interested readers are strongly encouraged to contact us for evaluating the visual quality.

# Conclusions

If ALF line buffer removal techniques are adopted, it is not necessary to use fewer vertical taps than horizontal taps. In this contribution, it is reported that the line buffer removal in CE8.c.2 in combination with the new filter shapes in CE8.d.1 achieves 0.0%, -0.2%, -0.3%, and 0.2% BD-rates for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, and the line buffer removal in CE8.c.3 in combination with the new filter shapes in CE8.d.1 achieves 0.0%, -0.2%, -0.4%, and -0.1% BD-rates for the four conditions respectively. Both the two combinations have similar subjective qualities as HM-4.0, but the second combination is better than the first combination.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of CE8.c.4-1, single-source SAO and ALF VB processing with cross9x9

Table 2.  Results of CE8.c.4-2, multi-source SAO and ALF VB processing with cross9x9

# References

- T. Yamakage, I. S. Chong, M. Narroschke, Y.-W. Huang, “Description of Core Experiment 8 (CE8): Non-deblocking loop filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-F908, July 2011.

- C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei, “CE8.c.2: Single-source SAO and ALF virtual boundary processing,” Document of Joint Collaborative Team on Video Coding, JCTVC-G204, November 2011.

- C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, S. Esenlik, M. Narroschke, T. Wedi, “CE8.c.3: Multi-source SAO and ALF virtual boundary processing,” Document of Joint Collaborative Team on Video Coding, JCTVC-G205, November 2011.

- C.-Y. Tsai, C.-Y. Chen, C.-M. Fu, Y.-W. Huang, S. Lei, I. S. Chong, M. Karczewicz, T. Yamakage, T. Watanabe, T. Chujoh, “CE8.d.1: Snowflake5x5 and cross9x9 for luma and chroma ALF shapes,” Document of Joint Collaborative Team on Video Coding, JCTVC-G208, November 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.