| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G207 |
| --- | --- |

| Title: | CE8.c.5: Non-cross-slices SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Ming Fu, Chia-Yang Tsai, Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chihming.fu, chiayang.tsai, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
|  | Madhukar Budagavi Texas Instruments 12500 TI Blvd., Dallas 75243, TX, USA | Tel: Email: | Madhukar Budagavi +1-214-480-1861 madhukar@ti.com |
| Source: | MediaTek Inc., Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution reports results of CE8.c.5. In HM-4.0, non-cross-slices SAO skips each to-be-processed pixel requiring any pixel from any other slice. However, the skipping technique may cause some potential problem in visual quality. In JCTVC-F093 and JCTVC-F232, any pixel from any other slice is avoided by using a padding technique and by modifying pixel classification patterns, respectively. In this proposal, it is found that the padding technique and modification of pixel classification patterns are conceptually the same and can be combined into one single solution. Simulation results reportedly show that the proposed method achieves no coding efficiency loss and better subjective quality in comparison with the anchor when non-cross-slices SAO and multiple slices per picture are used.

# Introduction

In High Efficiency Video Coding test model version 4.0 (HM-4.0), sample adaptive offset (SAO) is applied to luma and chroma, and each component is independently processed. SAO can divide one picture into multiple LCU-aligned regions, and each region can select one SAO type among two band offset (BO) types, four edge offset (EO) types, and no processing (OFF). For each to-be-processed pixel, BO uses the pixel intensity to classify the pixel into a band. The pixel intensity range is equally divided into 32 bands, as shown in Figure 1. After pixel classification, one offset is derived for all pixels of each band, and the offsets of center 16 bands or outer 16 bands are selected and coded. As for EO, it uses two neighboring pixels of a to-be-processed pixel to classify the pixel into a category. Similarly, one offset is derived for all pixels of each category after pixel classification except for category 0, which is forced to use zero offset. Table 1 shows the EO pixel classification, where “c” denotes the pixel to be classified. Figure 2 shows four EO types with different selections of neighboring pixels.

Figure 1.  Pixel intensity range equally divided into 32 bands by BO

Table 1.  EO pixel classification

Figure 2.  Four EO types

In HM-4.0, in order to support multiple slices that can be processed in parallel, one flag in the sequence parameter set (SPS) is used to support non-cross-slices in-loop filtering. When the non-cross-slices in-loop filtering is enabled, SAO skips processing of pixels that originally require pixels from any different slice. However, the skipping may cause some undesirable visual artifact. In JCTVC-F093 [2], padding was proposed to solve this problem. In JCTVC-F232 [3], new EO patterns were proposed to improve the subjective quality. Then, JCTVC-F093 and JCTVC-F232 were included in Core Experiment 8 (CE8) [4] SubtestC as Tool 5 and Tool 6, respectively. In this contribution, we will show that the padding and the modified EO patterns are conceptually the same and can be combined as one solution. The rest of this contribution is organized as follows. The proposed non-cross-slices SAO will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed non-cross-slices processing

The general concept of non-cross-slices SAO is to avoid using any the pixel from any other slice. We propose a directional padding technique according to the direction of EO type as show in Figure 3. In Figure 3(a), 0-degree EO is selected, so the horizontal padding is applied at vertical slice boundaries. In Figure 3(b), 90-degree EO is selected, so the vertical padding is applied at horizontal slice boundaries. In Figure 3(c) and (d), 135-degree EO and 45-degree EO are selected, respectively, so padding in diagonal directions are applied at slice boundaries. Using the directional padding with the original pixel classification shown in Table 1 can be implemented by using new EO patterns as shown in Figure 3(e), (f), (g), (h), (i), (j), (k), and (l) with new pixel classification shown in Table 2, which can be derived from Table 1 where c is always equal to one neighbor. Please note that there are four exceptions in the examples of Figure 3. For corner pixels E and U using 135-degree EO and corner pixels A and Y using 45-degree EO, the two neighboring pixels are both the same as the to-be-processed pixel, so the category and offset are always be zero in these cases, which means SAO is always skipped for these corner pixels using diagonal EO. In order to avoid potential artifacts, when the two diagonally neighboring pixels are both outside the current slice, the upper pixel is checked first. If the upper pixel is in the current slice, the new pattern in Figure 3(g) will be chosen to use the upper pixel with the modified pixel classification in Table 2, or equivalently the padding for pixel U in Figure 3(c) and pixel Y in Figure 3(d) will be used with the original pixel classification in Table 1; otherwise, the new pattern in Figure 3(h) will be chosen to use the lower pixel with the modified pixel classification in Table 2, or equivalently the padding for pixel E in Figure 3(c) and pixel A in Figure 3(d) will be used with the original pixel classification in Table 1.

Figure 3.  Proposed padding directions and equivalent EO patterns; (a) padding for 0-degree EO; (b) padding for 90-degree EO; (c) padding for 135-degree EO; (d) padding for 45-degree EO; (e) equivalent pattern for pixels A, F, K, P, U using 0-degree EO; (f) equivalent pattern for pixels E, J, O, T, Y using 0-degree EO; (g) equivalent pattern for pixels A, B, C, D, E using 90-degree EO, for pixel U using 135-degree EO, and for pixel Y using 45-degree EO; (h) equivalent pattern for pixels U, V, W, X, Y using 90-degree EO, for pixel E using 135-degree EO, and for pixel A using 45-degree EO; (i) equivalent pattern for pixels P, K, F, A, B, C, D using 135-degree EO; (j) equivalent pattern for pixels V, W, X, Y, T, O, J using 135-degree EO; (k) equivalent pattern for pixels F, K, P, U, V, W, X using 45-degree EO; (l) equivalent pattern for pixels B, C, D, E, J, O, T using 45-degree EO.

Table 2.  Modified EO pixel classification

# Simulation results

HM-4.0 is used as the anchor. High efficiency all intra (HE-AI), high efficiency random access (HE-RA), high efficiency low delay B (HE-LDB), low complexity all intra (LC-AI), low complexity random access (LC-RA), and low complexity low delay B (LC-LDB) common test conditions defined in JCTVC-F900 [4] are used, and slice-related settings were modified as SliceMode=1 (fixed number of LCUs per slice), SliceArgument=10 (number of LCUs per slice), cross-slices DF and ALF, and non-cross-slices SAO. Please note that in the High Efficiency Video Coding (HEVC) Working Draft version 4.0 (WD-4.0), all in-loop filters must be cross-slices or non-cross-slices. However, we realized the artifacts caused by non-cross-slices DF and ALF make the subjective evaluation of different non-cross-slices SAO algorithms very difficult. Therefore, we modified the software to support these test conditions agreed in CE8. Table 3 shows that the proposed non-cross-slices SAO results in 0.0%, 0.0%, 0.0%, 0.0%, 0.0%, and 0.0% luma BD-rate differences for HE-AI, HE-RA, HE-LDB, LC-AI, LC-RA, and LC-LDB, respectively. The encoding time and the decoding time are roughly unchanged. As for the subjective quality, after carefully viewing of all video sequences, we observed that the visual quality of this proposal is better than that of the anchor. Interested readers are strongly encouraged to contact us for evaluating the visual quality.

# Conclusions

MediaTek and TI implemented a padding method for non-cross-slices SAO. Simulation results reportedly show that the proposed method achieves the same objective coding efficiency and better subjective quality in comparison with HM-4.0 when non-cross-slices SAO and multiple slices per picture are enabled.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-M. Fu, C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei, “Sample adaptive offset with padding at LCU, slice, and image boundaries, JCTVC-F093, July 2011.

- Madhukar Budagavi, “SAO LCU boundary processing,” Document of Joint Collaborative Team on Video Coding, JCTVC-F232, July 2011.

- T. Yamakage, I. S. Chong, M. Narroschke, Y.-W. Huang, “Description of Core Experiment 8 (CE8): Non-deblocking loop filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-F908, July 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.

Table 3.  Results of CE8.c.5, non-cross-slices SAO