| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G208 |
| --- | --- |

| Title: | CE8.d.1: Snowflake5x5 and cross9x9 for luma and chroma ALF shapes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Yang Tsai, Ching-Yeh Chen, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA Tomoo Yamakage, Takashi Watanabe, Takeshi Chujoh TOSHIBA 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chiayang.tsai, chingyeh.chen, chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com Tomoo Yamakage +81-44-549-2152 {tomoo.yamakage, takashi39.watanabe, takeshi.chujoh}@toshiba.co.jp |
| Source: | MediaTek Inc., Qualcomm Inc., TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution reports results of CE8.d.1. In HM-4.0, snowflake5x5 and cross11x5 filter shapes have nine and eight multiplications, respectively, and are used for both luma and chroma in ALF. In this proposal, snowflake5x5 and cross9x9 are used, and they both have nine multiplications to better utilize multipliers without increasing the number of multipliers in hardware. Simulation results reportedly show 0.1%, 0.4%, 0.6% and 0.3% coding efficiency gains for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, with roughly the same encoding time and 1%-2% decoding time increase. Apparently cross9x9 needs more line buffers than cross11x5, so it is suggested to combine this proposal with ALF line buffer removal techniques.

# Introduction

In High-Efficiency Video Coding test model version 4.0 (HM-4.0), snowflake5x5 and cross11x5 filter shapes are applied in adaptive loop filter (ALF). The filter shapes and coefficients are shown in Figure 1. Snowflake5x5 has nine multiplications and cross11x5 has eight. In cross11x5, the vertical tap size is smaller than the horizontal tap size for reducing line buffers. In hardware, the worst-case has to be considered, and therefore the number of multipliers in ALF is nine. However, if cross11x5 is selected, one multiplier will be idle. In order to increase the hardware utilization, it is desirable to design different filter shapes with the same number of multiplications. Moreover, with consideration of ALF line buffer removal techniques, it is desirable to have the same number of taps horizontally and vertically. The rest of this contribution is organized as follows. The proposed filter shapes will be shown in Section 2, simulation results will be shown in Section 3, and finally conclusions will be given in Section 4.

Figure 1.  ALF shapes in HM-4.0

# Proposed filter shapes

The proposed filter shapes for ALF are snowflake5x5 and cross9x9, as shown in Figure 2. It can be seen that snowflake5x5 and cross9x9 both need nine multiplications, which can enhance the utilization of multipliers in hardware. In addition, the worst-case number of the multiplications is the same as that in the ALF of HM-4.0, which means the number of multipliers in hardware is not increased. However, the worst-case vertical tap size is increased, which means more line buffers are required. Fortunately, the proposed filter shapes can be easily combined with ALF line buffer removal techniques.

Figure 2.  Proposed ALF shapes

# Simulation results

The proposed ALF shapes are implemented on HM-4.0. High efficiency all intra (HE-AI), high efficiency random access (HE-RA), high efficiency low delay B (HE-LDB), and high efficiency low delay P (HE-LDP) common test conditions were tested according to JCTVC-F900 [1]. The results are shown in Table 1. The luma BD-rates of the proposed filter shapes are -0.1%, -0.4%, -0.6%, and -0.3% for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, where negative numbers mean gains. The encoding time is roughly unchanged, and the decoding time is increased by 1-2%.

Table 1.  Results of CE8.d.1, using snowflake5x5 and cross9x9 for ALF

# Conclusions

MediaTek, Qualcomm, and TOSHIBA implemented snowflake5x5 and cross9x9 filter shapes for ALF. Compared with the snowflake5x5 and cross11x5 in HM-4.0, the proposed filter shapes have the same worst-case number of multiplications and better coding efficiency. Simulation results reportedly show 0.1%, 0.4%, 0.6%, and 0.3% bit rate reductions for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, with no encoding time change and 1%-2% decoding time increase. If the proposed filter shapes are combined with ALF line buffer removal techniques, the increased vertical tap size will not result in any additional line buffer.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.