HEVC Working Draft modifications for JCTVC-G208 based on WD document JCTVC-F803_d5.doc

The modifications are highlighted in yellow .

#### 8.6.3.1 Boundary padding process

….

Depending on alf_filter_shape, the following applies:

- If alf_filter_shape is equal to 0, a variable alfTapX is set equal to 5, a variable alfTapY is set equal to 5, a variable nExtPixelsX is set equal to ( alfTapX >> 1 ) and a variable nExtPixelsY is set equal to ( alfTapY >> 1 ).

- Otherwise, if alf_filter_shape is equal to 1, a variable alfTapX is set equal to 11, a variable alfTapY is set equal to 59, a variable nExtPixelsX is set equal to ( alfTapX >> 1 ) and a variable nExtPixelsY is set equal to ( alfTapY >> 1 ).

#### 8.6.3.4 Filtering process for luma samples

……

Table 8-18 – Specification of horPos[ i ] according to alfFilterShape for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfFilterShape = 0 | -2 | 0 | 2 | -1 | 0 | 1 | -2 | -1 | 0 | 1 | 2 | -1 | 0 | 1 | -2 | 0 | 2 |
| alfFilterShape = 1 | 0 | 0 | 0 | 0 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 0 | 0 | 0 | 0 |

Table 8-19 – Specification of verPos[ i ] according to alfFilterShape for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfFilterShape = 0 | -2 | -2 | -2 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 |
| alfFilterShape = 1 | -4 | -3 | -2 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 |

- alfFilterShape = 0            (b) alfFilterShape = 1

Figure 8-8 Mapping between geometric position and luma adaptive loop filter index according to alfFilterShape (informative)