| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G210 |
| --- | --- |

| Title: | CE1: Subtest 12, Entropy coding comparisons with simplified RDO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Test Report |  |  |
| Author(s) or Contact(s): | T. Davies 14 Waterside Drive Langley, SL3 6EZ, UK | Email: | thdavies@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

Various coding conditions are simulated with both CAVLC and CABAC. Under low-complexity (LC) common conditions it is reported that CABAC provides gain between 5.4% and 6.2% (6.8% including class F). The performance of the entropy coders is also investigated with two encoder restrictions: no RDOQ, and no adaption during RDO mode search. These assumptions together reduce the gap between CABAC and CAVLC by 0.775% (0.875% including class F) averaged across LC settings. When RDOQ is off and RDO adaption is off  the gap is reported to be between 5.0% and  5.5% for LC settings (4.9% and 5.9% including class F).

# Introduction

This contribution reports the results of Experiment 12 on entropy coding [1]. JCTVC-F162 [2] reported the results of comparing CABAC and CAVLC performance, under the assumption of encoding techniques that more accurately reflect real-world constraints than the full RDO approach employed in HM. These techniques included: multiple tiles (for high-level parallelism); no-sub-LCU probability model update (PMU) during RDO mode decision; and RDOQ disabled. Each of these factors reportedly reduced the difference between CABAC and CAVLC performance.

JCTVC-F162 was tested in the context of HM3.2. This contribution re-tests two of these factors (sub-LCU RDO probability adaption and RDOQ disabled) in the context of HM 4.0.

Acknowledgement: We are very grateful to HHI for reporting a bug with the implementation for turning off sub-LCU PMU in RDO [3]. The results reported here have been corrected and correspond to those reported in [3].

# The experiments

In the first experiment, HM4.0 was used to determine the performance of CAVLC and CABAC under LC common conditions [4]. It was established in [2] that it was sufficient to assess performance in LC conditions alone.

Overall an average of 5.4-6.2% gap was found between CAVLC and CABAC across all conditions. This is lower than the range 6.1%-7.6% found for HM3.2 [2].

In the second experiment, HM4.0 was modified so  that RDO search was performed without context adaption within an LCU for trial encodings – contexts are re-initialised at the beginning of LCU RDO search. This approach is already used in CAVLC. This is equivalent to estimating bit costs with fixed entropy estimates within an LCU, and does not affect actual encoding.

Experiment 3 is the same as Experiment 2 except that additionally RDOQ is turned off. RDOQ is most likely to be used in a real encoder when a more restricted set of modes are searched as it is relatively expensive, and many applications may not use it at all. The gap between CABAC and CAVLC is reduced to 5.0%-5.5% (4.9% to 5.9% including class F).

# Results

Results are presented in the following tables. Although the average over Class F sequences is shown in each case, the Overall averages reported do not include Class F. Note that in general Class F shows both a larger gap between CABAC and CAVLC in these experiments, and a correspondingly larger reduction in that gap for simplified encoding.

## Experiment 1: CABAC versus CAVLC

Software: HM4.0

Configurations tested: All LC common conditions for 4.0 [4].

Simulation parameters: --SymbolMode=0 versus --SymbolMode=1 on LC settings

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -5.6% | 1.5% | 1.8% |
| Class B | -5.8% | 0.8% | 1.7% |
| Class C | -4.6% | 0.6% | 0.7% |
| Class D | -3.9% | 1.1% | 1.1% |
| Class E | -7.4% | 6.5% | 6.2% |
| Class F | -5.2% | -1.1% | -1.4% |
| Overall (-F) | -5.4% | 1.8% | 2.1% |
| Overall (+F) | -5.4% | 1.3% | 1.5% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Class A | -8.2% | 8.0% | 6.9% |
| Class B | -7.4% | 3.4% | 4.7% |
| Class C | -5.4% | 1.9% | 2.6% |
| Class D | -4.6% | 3.7% | 4.2% |
| Class E |  |  |  |
| Class F | -7.1% | -2.3% | -2.7% |
| Overall (-F) | -6.5% | 4.2% | 4.6% |
| Overall (+F) | -6.6% | 3.0% | 3.2% |
| Enc Time[%] | 92% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -7.3% | 5.8% | 9.9% |
| Class C | -5.4% | 4.1% | 5.2% |
| Class D | -4.7% | 8.5% | 8.7% |
| Class E | -6.9% | 6.3% | 7.5% |
| Class F | -9.1% | -3.2% | -3.2% |
| Overall (-F) | -6.1% | 6.2% | 8.0% |
| Overall (+F) | -6.7% | 4.3% | 5.7% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -7.3% | 7.4% | 11.5% |
| Class C | -5.5% | 4.6% | 5.8% |
| Class D | -4.7% | 9.0% | 8.4% |
| Class E | -7.1% | 7.4% | 9.2% |
| Class F | -9.3% | -2.9% | -3.3% |
| Overall (-F) | -6.2% | 7.1% | 8.9% |
| Overall (+F) | -6.8% | 5.1% | 6.4% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |

## Experiment 2: CABAC versus CAVLC, no sub-LCU RDO PMU

Software: modified HM4.0 with sub-LCU PMU disabled for CABAC (disabled by default for CAVLC)

Configurations tested: All LC common conditions for 4.0 [4].

Simulation parameters: --SymbolMode=0 versus --SymbolMode=1 on LC settings

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -5.4% | 1.9% | 2.3% |
| Class B | -5.4% | 1.6% | 2.8% |
| Class C | -4.2% | 1.5% | 1.7% |
| Class D | -3.6% | 1.8% | 2.1% |
| Class E | -7.1% | 7.2% | 7.2% |
| Class F | -4.8% | -0.2% | -0.4% |
| Overall (-F) | -5.0% | 2.5% | 3.0% |
| Overall (+F) | -5.0% | 2.1% | 2.4% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 107% |  |  |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Class A | -8.0% | 8.3% | 7.7% |
| Class B | -7.1% | 4.1% | 5.8% |
| Class C | -5.1% | 2.6% | 3.5% |
| Class D | -4.2% | 4.5% | 4.7% |
| Class E |  |  |  |
| Class F | -6.8% | -1.7% | -1.6% |
| Overall (-F) | -6.2% | 4.8% | 5.5% |
| Overall (+F) | -6.3% | 3.6% | 4.1% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -7.0% | 6.5% | 10.8% |
| Class C | -5.1% | 5.0% | 6.3% |
| Class D | -4.4% | 9.1% | 9.1% |
| Class E | -6.9% | 8.1% | 9.2% |
| Class F | -8.7% | -2.3% | -2.1% |
| Overall (-F) | -5.9% | 7.1% | 9.0% |
| Overall (+F) | -6.4% | 5.2% | 6.7% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Low delay P LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -7.0% | 8.7% | 12.8% |
| Class C | -5.1% | 5.4% | 7.1% |
| Class D | -4.4% | 9.5% | 9.5% |
| Class E | -6.9% | 9.2% | 10.1% |
| Class F | -8.9% | -2.2% | -1.7% |
| Overall (-F) | -5.9% | 8.2% | 10.0% |
| Overall (+F) | -6.5% | 6.1% | 7.7% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 103% |  |  |

## Experiment 3: CABAC versus CAVLC, no sub-LCU RDO PMU, no RDOQ

Software: modified HM4.0 with sub-LCU PMU disabled for CABAC (disabled by default for CAVLC)

Configurations tested: All LC common conditions for 4.0 [4].

Simulation parameters: --SymbolMode=0 –RDOQ=0 versus --SymbolMode=1 –RDOQ=0 on LC settings

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -4.8% | -2.9% | -4.2% |
| Class B | -5.8% | -3.8% | -3.1% |
| Class C | -4.2% | -2.8% | -2.5% |
| Class D | -3.4% | -2.7% | -2.4% |
| Class E | -7.1% | -1.2% | -1.0% |
| Class F | -4.2% | -2.8% | -2.7% |
| Overall (-F) | -5.0% | -2.8% | -2.7% |
| Overall (+F) | -4.9% | -2.8% | -2.7% |
| Enc Time[%] | 127% |  |  |
| Dec Time[%] | 114% |  |  |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Class A | -7.2% | 3.2% | 2.0% |
| Class B | -6.4% | -0.9% | -0.3% |
| Class C | -4.4% | -1.3% | -0.6% |
| Class D | -3.7% | -1.0% | -0.1% |
| Class E |  |  |  |
| Class F | -6.1% | -3.6% | -3.3% |
| Overall (-F) | -5.5% | -0.1% | 0.2% |
| Overall (+F) | -5.6% | -0.7% | -0.5% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 107% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -6.0% | 1.5% | 3.3% |
| Class C | -4.4% | 1.2% | 2.2% |
| Class D | -3.6% | 3.1% | 4.0% |
| Class E | -6.7% | -0.2% | 2.1% |
| Class F | -7.7% | -4.0% | -3.9% |
| Overall (-F) | -5.1% | 1.5% | 3.0% |
| Overall (+F) | -5.6% | 0.4% | 1.6% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Low delay P LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -6.4% | 2.2% | 4.2% |
| Class C | -4.7% | 1.2% | 2.4% |
| Class D | -3.9% | 2.7% | 4.5% |
| Class E | -7.1% | 0.6% | 2.1% |
| Class F | -7.9% | -4.1% | -3.7% |
| Overall (-F) | -5.5% | 1.8% | 3.4% |
| Overall (+F) | -5.9% | 0.6% | 2.0% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 105% |  |  |

# Conclusions

When a number of realistic constraints are applied to encoding techniques, the BD-Rate difference between CABAC and CAVLC is substantially reduced. When both RDOQ and sub-LCU RDO PMU are disabled, the reductions in BD-Rate gain are (with class F in brackets) AI: 0.4% (0.5%), RA: 1.0% (1.0%), LD(B): 1.0% (1.1%), and LD(P): 0.7% (0.9%). Overall the BD-Rate gap in Subtest 3 is reduced by an average of 0.775% (0.875% with class F) from that in Subtest 1.

Note that the BD-R differences for Class F are, except in the case of All Intra, larger than for other classes, and the reductions are also larger. This may reflect inadequate tuning of context (CABAC) and VLC tables (CAVLC) for this kind of content.

# References

[1] JCTVC-F901, “Description of Core Experiment 1: Entropy Coding”, R.Joshi (Qualcomm), E. Alshina (Samsung), H. Sasai (Panasonic), H. Kirchhoffer (HHI), J. Lainema (Nokia), JCTVC 6th Meeting, Turin, Italy, 14-22 July 2011

[2] JCTVC-F162, “Entropy coding performance simulations”, T. Davies and A. Fuldseth (Cisco Systems), JCTVC 6th Meeting, Turin, Italy, 14-22 July 2011

[3] JCTVC-G663, “CE1: Report of test results related to PIPE-based Unified Entropy Coding”, H. Kirchhoffer, B. Bross, A. Henkel, D. Marpe, T. Nguyen, M. Preiß, M. Siekmann, J. Stegemann, T. Wiegand (HHI), JCTVC 7th Meeting Geneva, CH, 21-30 November 2011.

[4]	JCTVC-F900, “Common test conditions and software reference configurations”, F. Bossen, -VC 6th Meeting, Turin, Italy, 14-22 July 2011

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).