#### Derivation process for filter index array for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

Output of this process is the two-dimensional filter index array of (nS)x(nS), fIdx.

A variable nS is set equal to ( 1 << log2CUSize ).

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. The boundary padding process specified in subclause Error: Reference source not found shall be applied to generate the padded sample of pre-deblocked pixels, preDF, and pre-SAO pixels, preSAO with the same parameters. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

The filter index array fIdx is specified in the follows:

- When alf_region_adaptation_flag is equal to 1, the following ordered steps apply.

- The variables xIdx and yIdx are derived as

regionTab[16] = { 0, 1, 4, 5, 15, 2, 3, 6, 14, 11, 10, 7, 13, 12, 9, 8 }	(8-1)
	offset = 1 << (Log2MaxCUSize – 1) 		(8-2)
	xInterval = ( ( ( PicWidthInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-3)
	yInterval = ( ( ( PicHeightInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-4)
	xIdx = Min( 3, Floor( ( xC + x ) / ( xInterval << Log2MaxCUSize ) ) )	(8-5)
	yIdx = Min( 3, Floor( ( yC + y ) / ( yInterval << Log2MaxCUSize ) ) )	(8-6)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = regionTab[ ( yIdx << 2 ) + xIdx ]		(8-7)

- Otherwise (alf_region_flag is equal to 0), the following ordered steps apply.

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] with x, y = 0, 2, 4,...(nS) is derived as

LcuHeight = 1 << Log2MaxCUSize;                                                        
   VBLine = LcuHeight – 3;
   if ( (yC+y)  % LcuHeight == VBLine-1 && yC+ LcuHeight < img_height )
  {
    varTempH[x][y] = | ( preDF’’[ xC+x, yC+y ] << 1 ) – preDF’’[ xC+x-1, yC+y ] –preDF[ xC+x+1, yC+y ] |    (8-8)   
     varTempV[x][y] = | ( preDF’’[ xC+x, yC+y ] << 1 ) – preDF’’[ xC+x, yC+y ] – s’’[ xC+x, yC+y+1 ] |	(8-9)
   }
   else
  {
     varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y ] – s’’[ xC+x+1, yC+y ] |	(8-10)   
     varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x, yC+y-1 ] – s’’[ xC+x, yC+y+1 ] |	(8-11) 
  }

- The variables varTempH1[ x, y ], varTempV1[ x, y ], and varTemp3[ x, y ] with x, y = 0..( (nS) – 1 )>>2 are derived as

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 0, 2	(8-12)
	varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 0, 2	(8-13)
	varTemp3[ x ][ y ] = ( varTempH1[ x ][ y ] + varTempV1[ x ][ y ] ) >> 2	(8-14)

- The variable direction is derived as

- If varTempV1[ x >> 2 ][ y >> 2 ] is greater than varTempH1[ x >> 2 ][ y >> 2 ] << 1,

direction = 1

- Otherwise, if varTempH1[ x >> 2 ][ y >> 2 ] is greater than varTempV1[ x >> 2 ][ y >> 2 ] << 1,

direction = 2

- Otherwise,

direction = 0

- The variable avgvar is derived as

varTab[16] = { 0, 1, 2, 2, 2, 3, 3, 3, 3, 3, 4, 4, 4, 4, 4, 4 }	(8-15)
	avgVar = Clip3( 0, 15, (varTemp3[ x >> 2 ][ y >> 2 ] * 1024 ) >> (3 + BitDepthY) )	(8-16)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = Clip3( 0, 4, varTab[avgVar] ) + 5 * direction	(8-17)

#### Filtering process for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a filter index array of (nS)x(nS), fIdx.

Output of this process is the filtered reconstruction of luma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. The boundary padding process specified in subclause Error: Reference source not found shall be applied to generate the padded sample of pre-deblocked pixels, preDF, and pre-SAO pixels, preSAO with the same parameters. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfFilterShape is set equal to alf_filter_shape_flag.

Each sample of luma picture recFiltPictureL[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, is derived as follows.

LcuHeight = 1 << Log2MaxCUSize;                                                        
 VBLine = LcuHeight – 3;
 if ( (yC + y) % LcuHeight == VBLine-4  &&  yC+ LcuHeight < img_height  
       &&  alf_filter_shape_flag == 1)
 {  
      recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=015s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +preDF[xC+x+horPos[16],yC+y+verPos[16]]∗cL[fIdx[x][y]][16]      
 }
else if ( (yC + y) % LcuHeight == VBLine-3  &&  yC+ LcuHeight < img_height  
                &&  alf_filter_shape_flag == 1)
{  
    recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=014s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=1516preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]
}
else if ( (yC + y) % LcuHeight == VBLine-2  &&  yC+ LcuHeight < img_height  )
{
     recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=013s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=1416preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]
}
else if ( (yC + y) % LcuHeight == VBLine-1  &&  yC+ LcuHeight < img_height  )
{
   if (alf_filter_shape_flag == 0)     
   {
      recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y−1]][N]+i=010s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y−1]][i]            +i=1116preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y−1]][i]
   }
   else
   {
      recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y−1]][N]+i=112s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y−1]][i]            +i=1315preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y−1]][i]            +i=002∗s''[xC+x,yC+y]∗cL[fIdx[x][y−1]][i]
   }
}
else if ( (yC + y) % LcuHeight == VBLine  &&  yC+ LcuHeight < img_height  )
{
   if (alf_filter_shape_flag == 0)     
   {
      recFiltPictureL[xC+x][yC+y]=             (cL[fIdx[x][y]][N]+i=616s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=35preSAO[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=02preSAO[xC+x+horPos[i],yC+y+verPos[i]+1]∗cL[fIdx[x][y]][i]+s''[xC+x,yC+y])>>1
   }
   else
   {
      recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=413s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=33preSAO[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=022∗s''[xC+x,yC+y]∗cL[fIdx[x][y]][i]
   }
}
else if ( (yC + y) % LcuHeight == VBLine+1  &&  yC+ LcuHeight < img_height  
             &&  alf_filter_shape_flag == 1 )
{
   recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=314s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=22preSAO[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=012∗s''[xC+x,yC+y]∗cL[fIdx[x][y]][i]
}
else if ( (yC + y) % LcuHeight == VBLine+2  &&  yC+ LcuHeight < img_height  
             &&  alf_filter_shape_flag == 1 )
{
   recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=215s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=11preSAO[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=002∗s''[xC+x,yC+y]∗cL[fIdx[x][y]][i]
}
else if ( (yC + y) % LcuHeight == 0  &&  yC+ LcuHeight < img_height  
             &&  alf_filter_shape_flag == 1 && yC > 0 )
{
   recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=116s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=00preSAO[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]
}
else
{     
    recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]    (8-18)
}

where N is set equal to AlfNumCoeffLuma-1 and horPos[i] and verPos[i] are specified in Error: Reference source not found and Table 8-, respectively.

Table 8-16 – Specification of horPos[ i ] according to alfFilterShape for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfFilterShape = 0 | -2 | 0 | 2 | -1 | 0 | 1 | -2 | -1 | 0 | 1 | 2 | -1 | 0 | 1 | -2 | 0 | 2 |
| alfFilterShape = 1 | 0 | 0 | 0 | 0 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 0 | 0 | 0 | 0 |

Table 8-17 – Specification of verPos[ i ] according to alfFilterShape for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfFilterShape = 0 | -2 | -2 | -2 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 |
| alfFilterShape = 1 | -4 | -3 | -2 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 |

- alfFilterShape = 0            (b) alfFilterShape = 1

Figure 8-1 Mapping between geometric position and luma adaptive loop filter index according to alfFilterShape (informative)

#### Filtering process for chroma samples

Inputs of this process are:

–	a chroma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

–	a variable cIdx specifying the chroma component index.

Output of this process is the filtered reconstruction of chroma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the chroma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx, and the output is assigned to the luma sample array s’’. The boundary padding process specified in subclause Error: Reference source not found shall be applied to generate the padded sample of pre-deblocked pixels, preDF, and pre-SAO pixels, preSAO with the same parameters. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

A variable nS is set equal to ( 1 << log2CUSize ) and variable alfFilterShape is set equal to alf_filter_shape_flag.

Filtered samples of chroma picture recFiltPicture[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, are derived as follows.

LcuHeight = 1 << (Log2MaxCUSize-1);                                                        
   VBLine = LcuHeight – 1;
   if ( (yC + y) % LcuHeight == VBLine-4  &&  yC+ LcuHeight < img_height  
         &&  alf_filter_shape_flag == 1)
   {  
        recFiltPicture[xC+x][yC+y]=             cC[N]+i=015s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +preDF[xC+x+horPos[16],yC+y+verPos[16]]∗cC[16]      
   }
   else if ( (yC + y) % LcuHeight == VBLine-3  &&  yC+ LcuHeight < img_height  
                &&  alf_filter_shape_flag == 1)
   {  
        recFiltPicture[xC+x][yC+y]=             cC[N]+i=014s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=1516preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]
}
else if ( (yC + y) % LcuHeight == VBLine-2  &&  yC+ LcuHeight < img_height  )
{
     recFiltPicture[xC+x][yC+y]=             cC[N]+i=013s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=1416preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]
}
else if ( (yC + y) % LcuHeight == VBLine-1  &&  yC+ LcuHeight < img_height  )
{
   if (alf_filter_shape_flag == 0)     
   {
      recFiltPicture[xC+x][yC+y]=             cC[N]+i=010s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=1116preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]
   }
   else
   {          
       recFiltPictureL[xC+x][yC+y]=             cC[N]+i=112s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=1315preDF[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]+i=002∗s''[xC+x,yC+y]∗cC[i]
   }
}
else if ( (yC + y) % LcuHeight == VBLine  &&  yC+ LcuHeight < img_height  )
{
   if (alf_filter_shape_flag == 0)     
   {
       recFiltPicture[xC+x][yC+y]=             (cC[N]+i=616s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=35preSAO[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=02preSAO[xC+x+horPos[i],yC+y+verPos[i]+1]∗cC[i]+s''[xC+x,yC+y])>>1
   }
   else
   {
      recFiltPicture[xC+x][yC+y]=             cC[N]+i=413s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=33preSAO[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=022∗s''[xC+x,yC+y]∗cC[i]
   }
}
else if ( (yC + y) % LcuHeight == 0  &&  yC+ LcuHeight < img_height  
             &&  alf_filter_shape_flag == 1  &&  yC > 0)
{
    recFiltPicture[xC+x][yC+y]=             cC[N]+i=314s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=22preSAO[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=012∗s''[xC+x,yC+y]∗cC[i]
}
else if ( (yC + y) % LcuHeight == 1  &&  yC+ LcuHeight < img_height  
             &&  alf_filter_shape_flag == 1  &&  yC > 1)
{   
    recFiltPicture[xC+x][yC+y]=             cC[N]+i=215s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=11preSAO[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]+i=002∗s''[xC+x,yC+y]∗cC[i]
}
else if ( (yC + y) % LcuHeight == 2  &&  yC+ LcuHeight < img_height  
             &&  alf_filter_shape_flag == 1 && yC > 2 )
{
   recFiltPicture[xC+x][yC+y]=             cC[N]+i=116s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=00preSAO[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]
}
else
{     
    recFiltPictureC[xC+x][yC+y]=             cC[N]+i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]                               (8-19)
}

where N is set equal to AlfNumCoeffChma-1 and horPos[i] and verPos[i] are specified in Error: Reference source not found and Table 8-, respectively.