##### Modification process for luma and chroma samples

Inputs to this process are:

–	a variable cIdx specifying chroma component index,

–	a sample position ( xC, yC ),

–	a pair of variables ( rx, ry ) specifying the region index,

–	a variable saoDepth specifying the split depth of the region,

–	an array saoValueArray specifying offset values,

–	a block size nS.

Output of this process is a modified picture buffer for the chroma component cIdx.

Let recSaoPicture represents the processed sample array of the current picture of chroma component cIdx and saoTypeIdx is set equal to sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ].

Variable bitDepth is set equal to BitDepthY if cIdx is equal to 0, otherwise, set equal to BitDepthC.

Variable LcuHeight is set equal to 1<<Log2MaxCUSize if cIdx is equal to 0, otherwise, set equal to 1<< (Log2MaxCUSize-1).

Variable VBLine is set equal to LcuHeight-4 if cIdx is equal to 0, otherwise, set equal to LcuHeight-2.

Depending on the value of saoTypeIdx, the following applies:

–	If saoTypeIdx is equal to one of the values of 1, 2, 3 or 4, the following ordered steps apply:

- Arrays hPos[2] and vPos[2] are specified in Table 8-1.

- A variable edgeIdx is specified as

if ( (yC+j) % LcuHeight == VBLine  &&  yC+ LcuHeight < img_Height  &&  saoTypeIdx != 1)

{

if ( (xC+i) % 2 == 0)

{

upSign0 = Sign( recPicture[ xC + i, yC + j ] – recPicture[ xC + i + hPos[0], yC + j + vPos[0] ] );

upSign1 = Sign( recPicture[ xC + i + 1, yC + j ] – recPicture[ xC + i + 1 + hPos[0], yC + j + vPos[0] ] );

upSign   = (upSign0 == upSign1)  ?  upSign1 : 0 ;

}

edgeIdx = 2 + upSign +  Sign( recPicture[ xC + i, yC + j ] – recPicture[ xC + i + hPos[1], yC + j + vPos[1] ] );	(8-1)

}

else

edgeIdx = 2 + ∑k( Sign( recPicture[ xC + i, yC + j ] –

recPicture[ xC + i + hPos[ k ], yC + j + vPos[ k ] ] ) ) with k = 0..1	(8-2)

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] + saoValueArray[ edgeTable[ edgeIdx ] ]	(8-3)

with i = 0..nS-1 and j = 0..nS-1 where 	edgeTable[5] = { 1, 2, 0, 3, 4}.

–	Otherwise, if saoTypeIdx is equal to one of the values of 5 or 6, the following ordered steps applies:

- A variable bandShift is set equal to BitDepthY – 5 if cIdx is equal to 0, otherwise, set equal to BitDepthC – 5.

- The reconstructed picture buffer is modified as

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] +

saoValueArray[ bandTable[ saoTypeIdx – 5 ][ bandIdx ] ]	(8-4)

with i = 0..nS-1 and j = 0..nS-1 where bandIdx is set equal to ( recPicture[ xC + i, yC + j ] >> bandShift ) and bandTable is specified in Table 8-2.

–	Otherwise (sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] is equal to 0), the following applies:

recSaoPicture[ xC + i, yC + j ] = recPicture[ xC + i, yC + j ] with i = 0..nS-1 and j = 0..nS-1	(8-5)

[Ed. (WJ): copy operation is necessary to use recSaoPicture later.]

Table 8-1 – Specification of hPos[2] and vPos[2] according to the type of sample adaptive offset process

| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| hPos[0] | -1 | 0 | -1 | 1 |
| hPos[1] | 1 | 0 | 1 | -1 |
| vPos[0] | 0 | -1 | -1 | -1 |
| vPos[1] | 0 | 1 | 1 | 1 |

Table 8-2 – Specification of array bandTable according to bandIdx

| bandIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| bandTable[1][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| bandTable[0][bandIdx] | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |

#### Derivation process for filter index array for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

Output of this process is the two-dimensional filter index array of (nS)x(nS), fIdx.

A variable nS is set equal to ( 1 << log2CUSize ).

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

The filter index array fIdx is specified in the follows:

- When alf_region_adaptation_flag is equal to 1, the following ordered steps apply.

- The variables xIdx and yIdx are derived as

regionTab[16] = { 0, 1, 4, 5, 15, 2, 3, 6, 14, 11, 10, 7, 13, 12, 9, 8 }	(8-6)
	offset = 1 << (Log2MaxCUSize – 1) 		(8-7)
	xInterval = ( ( ( PicWidthInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-8)
	yInterval = ( ( ( PicHeightInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-9)
	xIdx = Min( 3, Floor( ( xC + x ) / ( xInterval << Log2MaxCUSize ) ) )	(8-10)
	yIdx = Min( 3, Floor( ( yC + y ) / ( yInterval << Log2MaxCUSize ) ) )	(8-11)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = regionTab[ ( yIdx << 2 ) + xIdx ]		(8-12)

- Otherwise (alf_region_flag is equal to 0), the following ordered steps apply.

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] with x, y = 0, 2, 4,...(nS) is derived as

varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y ] – s’’[ xC+x+1, yC+y ] |	(8-13)   
   
   LcuHeight = 1 << Log2MaxCUSize;                                                        
   VBLine = LcuHeight – 4;
   if ( (yC+y)  % LcuHeight == VBLine  &&  yC+ LcuHeight < img_height )
     varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x, yC+y ] – s’’[ xC+x, yC+y+1 ] |	(8-14)
   else
     varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x, yC+y-1 ] – s’’[ xC+x, yC+y+1 ] |	(8-15)

- The variables varTempH1[ x, y ], varTempV1[ x, y ], and varTemp3[ x, y ] with x, y = 0..( (nS) – 1 )>>2 are derived as

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 0, 2	(8-16)
	varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 0, 2	(8-17)
	varTemp3[ x ][ y ] = ( varTempH1[ x ][ y ] + varTempV1[ x ][ y ] ) >> 2	(8-18)

- The variable direction is derived as

- If varTempV1[ x >> 2 ][ y >> 2 ] is greater than varTempH1[ x >> 2 ][ y >> 2 ] << 1,

direction = 1

- Otherwise, if varTempH1[ x >> 2 ][ y >> 2 ] is greater than varTempV1[ x >> 2 ][ y >> 2 ] << 1,

direction = 2

- Otherwise,

direction = 0

- The variable avgvar is derived as

varTab[16] = { 0, 1, 2, 2, 2, 3, 3, 3, 3, 3, 4, 4, 4, 4, 4, 4 }	(8-19)
	avgVar = Clip3( 0, 15, (varTemp3[ x >> 2 ][ y >> 2 ] * 1024 ) >> (3 + BitDepthY) )	(8-20)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = Clip3( 0, 4, varTab[avgVar] ) + 5 * direction	(8-21)

#### Filtering process for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a filter index array of (nS)x(nS), fIdx.

Output of this process is the filtered reconstruction of luma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

A variable nS is set equal to ( 1 << log2CUSize ) and a variable alfFilterShape is set equal to alf_filter_shape_flag.

Each sample of luma picture recFiltPictureL[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, is derived as follows.

LcuHeight = 1 << Log2MaxCUSize;                                                        
   VBLine = LcuHeight – 4;
   if ( ( (yC + y) % LcuHeight == VBLine  ||  (yC + y) % LcuHeight == VBLine -1  )
         &&  yC+ LcuHeight < img_height )
   {  
      if (alf_filter_shape_flag == 0)
        recFiltPictureL[xC+x][yC+y]=s''[xC+x,yC+y]
     else
        recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=412s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=032∗s''[xC+x,yC+y]∗cL[fIdx[x][y]][i]      
   }
   else if ( ((yC + y) % LcuHeight == VBLine +1  ||  (yC + y) % LcuHeight == VBLine -2 )
                && yC+ LcuHeight < img_height )
   {  
      if (alf_filter_shape_flag == 0  &&  (yC + y) % LcuHeight == VBLine +1)
        recFiltPictureL[xC+x][yC+y]=             (cL[fIdx[x][y]][N]+i=3N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]              +i=02s''[xC+x+horPos[i],yC+y+verPos[i]+1]∗cL[fIdx[x][y]][i])+s''[xC+x,yC+y])>>1
     else if (alf_filter_shape_flag == 0  &&  (yC + y) % LcuHeight == VBLine -2)
         recFiltPictureL[xC+x][yC+y]=             (cL[fIdx[x][y]][N]+i=013s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=14N−1s''[xC+x+horPos[i],yC+y+verPos[i]−1]∗cL[fIdx[x][y]][i]+s''[xC+x,yC+y])>>1
      else 
        recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=313s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=022∗s''[xC+x,yC+y]∗cL[fIdx[x][y]][i]  
   }
   else if ( ( (yC + y) % LcuHeight == VBLine +2  ||  (yC + y) % LcuHeight == VBLine -3  )
                &&  yC+ LcuHeight < img_height  &&  alf_filter_shape_flag == 1)
   {     
     recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=214s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +i=012∗s''[xC+x,yC+y]∗cL[fIdx[x][y]][i]
   }
   else if ( ( (yC + y) % LcuHeight == VBLine +3  ||  (yC + y) % LcuHeight == VBLine -4  )
                &&  yC+ LcuHeight < img_height  &&  alf_filter_shape_flag == 1)
   {     
     recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=115s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]            +2∗s''[xC+x,yC+y]∗cL[fIdx[x][y]][0]
   }   
   else
   {     
     recFiltPictureL[xC+x][yC+y]=             cL[fIdx[x][y]][N]+i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cL[fIdx[x][y]][i]    (8-22)
   }

where N is set equal to AlfNumCoeffLuma-1 and horPos[i] and verPos[i] are specified in Error: Reference source not found and Table 8-, respectively.

Table 8-16 – Specification of horPos[ i ] according to alfFilterShape for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfFilterShape = 0 | -2 | 0 | 2 | -1 | 0 | 1 | -2 | -1 | 0 | 1 | 2 | -1 | 0 | 1 | -2 | 0 | 2 |
| alfFilterShape = 1 | 0 | 0 | 0 | 0 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 0 | 0 | 0 | 0 |

Table 8-17 – Specification of verPos[ i ] according to alfFilterShape for adaptive loop filter process of luma samples

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfFilterShape = 0 | -2 | -2 | -2 | -1 | -1 | -1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 |
| alfFilterShape = 1 | -4 | -3 | -2 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 |

- alfFilterShape = 0            (b) alfFilterShape = 1

Figure 8-1 Mapping between geometric position and luma adaptive loop filter index according to alfFilterShape (informative)

#### Filtering process for chroma samples

Inputs of this process are:

–	a chroma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left chroma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit.

–	a variable cIdx specifying the chroma component index.

Output of this process is the filtered reconstruction of chroma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the chroma location ( xC, yC ), the size of coding unit log2CUSize and the chroma component index cIdx, and the output is assigned to the luma sample array s’’. [Ed. (WJ): s’’ is now a picture-size array, but actually CU size + appropriate margin is enough]

A variable nS is set equal to ( 1 << log2CUSize ) and variable alfFilterShape is set equal to alf_filter_shape_flag.

Filtered samples of chroma picture recFiltPicture[ xC + x ][ yC + y ] with x, y = 0..(nS)-1, are derived as follows.

LcuHeight = 1 << (Log2MaxCUSize-1);                                                        
   VBLine = LcuHeight – 2;
   if ( ( (yC + y) % LcuHeight == VBLine  ||  (yC + y) % LcuHeight == VBLine -1  )
         &&  yC+ LcuHeight < img_height )
   {  
      if (alf_filter_shape_flag == 0)
        recFiltPicture[xC+x][yC+y]=s''[xC+x,yC+y]
     else
        recFiltPicture[xC+x][yC+y]=             cC[N]+i=412s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]+i=032∗s''[xC+x,yC+y]∗cC[i]      
   }
   else if ( ( (yC + y) % LcuHeight == VBLine +1  ||  (yC + y) % LcuHeight == VBLine -2  )
                &&  yC+ LcuHeight < img_height )
   {  
      if (alf_filter_shape_flag == 0  &&  (yC + y) % LcuHeight == VBLine +1)
        recFiltPicture[xC+x][yC+y]=             (cC[N]+i=3N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]              +i=02s''[xC+x+horPos[i],yC+y+verPos[i]+1]∗cC[i])+s''[xC+x,yC+y])>>1
     else if (alf_filter_shape_flag == 0  &&  (yC + y) % LcuHeight == VBLine -2)
       recFiltPicture[xC+x][yC+y]=             (cC[N]+i=013s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]            +i=14N−1s''[xC+x+horPos[i],yC+y+verPos[i]−1]∗cC[i]+s''[xC+x,yC+y])>>1
      else 
        recFiltPicture[xC+x][yC+y]=             cC[N]+i=313s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]+i=022∗s''[xC+x,yC+y]∗cC[i]  
   }
   else if ( ( (yC + y) % LcuHeight == 0  ||  (yC + y) % LcuHeight == VBLine -3  )
                &&  yC>0  &&  alf_filter_shape_flag == 1)
   {     
     recFiltPicture[xC+x][yC+y]=             cC[N]+i=214s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]+i=012∗s''[xC+x,yC+y]∗cC[i]
   }
   else if ( ( (yC + y) % LcuHeight == 1  ||  (yC + y) % LcuHeight == VBLine -4  )
                &&  yC>1  &&  alf_filter_shape_flag == 1)
   {     
     recFiltPicture[xC+x][yC+y]=             cC[N]+i=115s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]+2∗s''[xC+x,yC+y]∗cC[0]
   }   
   else
   {     
     recFiltPicture[xC+x][yC+y]=cC[N]+i=0N−1s''[xC+x+horPos[i],yC+y+verPos[i]]∗cC[i]   (8-23)
   }

where N is set equal to AlfNumCoeffChma-1 and horPos[i] and verPos[i] are specified in Error: Reference source not found and Table 8-, respectively.