| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G212 |
| --- | --- |

| Title: | Non-CE8.c.7: Single-source SAO and ALF virtual boundary processing with cross9x9 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chih-Ming Fu, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Semih Esenlik, Matthias Narroschke, Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chihming.fu, chiayang.tsai, yuwen.huang, shawmin.lei}@mediatek.com Thomas Wedi +49 6103 766 1194 {Semih.Esenlik, Matthias.Narroschke, Thomas.Wedi}@panasonic.de Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com |
| Source: | MediaTek Inc., Panasonic Corporation, Qualcomm Inc. |  |  |

_____________________________

# Abstract

In HM-4.0, SAO requires 0.2 luma pixel line buffer (PLB) and 0.2 chroma PLB, and ALF requires 4.1 luma PLBs and four chroma PLBs. In JCTVC-F054 and JCTVC-F055, virtual boundary (VB) processing was proposed to remove all line buffers for ALF and SAO, respectively. In JCTVC-G206, the two prior proposals and using snowflake5x5 and cross9x9 ALF shapes are first combined as CE8.c.4-1 to remove all SAO and ALF line buffers, and this contribution provides two solutions to improve the visual quality of CE8.c.4-1. Due to the DF in HM-4.0, the luma VB and the chroma VB are set as four pixels and two pixels above the horizontal LCU boundary, respectively, and processing each pixel on one side of a VB avoids any data access from the other side of the VB. Non-CE8.c.7-1 gives up SAO VB processing and has SAO line buffers, while non-CE8.c.7-2 applies SAO VB processing method that can reduce 50% SAO line buffers. Both solutions apply ALF VB processing that can remove all ALF line buffers. When compared with the JCTVC-F900 anchor, both solutions reportedly improve coding efficiency and achieve 0.0%, -0.1%, -0.3%, and -0.0% BD-rates for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, and the same visual quality. No VB artifact is observed.

# Introduction

In High-Efficiency Video Coding test model version 4.0 (HM-4.0), in-loop filtering contains deblocking filter (DF), sample adaptive offset (SAO), and adaptive loop filter (ALF). These in-loop filters can significantly enhance coding efficiency; however, they require pixel line buffers in practical real-time decoders with largest coding unit (LCU) based processing. These line buffers can significantly increase on-chip memory sizes, especially for videos with large picture widths. Therefore, it is desirable to reduce line buffers. In JCTVC-F054 [1] and JCTVC-F055 [2], MediaTek proposed virtual boundary (VB) processing to remove all SAO and ALF line buffers. After the 6th JCT-VC meeting in Torino, Core Experiment 8 (CE8) for non-DF loop filtering [3] was established, and the line buffer reduction proposals were included in CE8. In JCTVC-G206 [4], MediaTek’s VB processing is combined with new filter shapes proposed in JCTVC-G208 [5] as CE8.c.4-1, and this contribution further improves CE8.c.4-1. The rest of this contribution is organized as follows. The line buffers of the in-loop filtering in HM-4.0 will be analyzed in Section 2, VB processing will be proposed in Section 3, simulation results will be shown in Section 4, and finally conclusions will be given in Section 5.

# Line buffer analysis

## Deblocking filter

In HM-4.0, DF applies filtering on 8x8 block boundaries. Figure 1 shows a vertical boundary on the left side of the figure and a horizontal boundary side on the right side of the figure, where 8x8 blocks are partly shown (4x8 or 8x4). In one picture, all vertical boundaries can be horizontally filtered in parallel, and then all horizontal boundaries can be vertically filtered in parallel. During processing of a luma boundary, four pixels of each side (p0-p3, q0-q3) are involved in filter parameter derivation, and at most three pixels of each side (p0-p2, q0-q2) can be changed after filtering. For luma horizontal DF, pre-DF pixels (i.e. pixels before horizontal DF) are used for deriving filter parameters and filtering. For luma vertical DF, pre-DF pixels are used for deriving filter parameters, and H-DF pixels (i.e. pixels after horizontal DF) are used for filtering. During processing of a chroma boundary, two pixels of each side (p0-p1, q0-q1) are involved in filter parameter derivation, and at most one pixel of each side (p0, q0) is changed after filtering. For chroma horizontal DF, pre-DF pixels are used for deriving filter parameters and filtering. For chroma vertical DF, H-DF pixels are used for deriving filter parameters and filtering.

Figure 1.  Illustration of 8x8 block boundaries for DF

## Sample adaptive offset

In HM-4.0, SAO is applied to luma and chroma, and each component is independently processed. SAO can divide one picture into multiple LCU-aligned regions, and each region can select one SAO type among two band offset (BO) types, four edge offset (EO) types, and no processing (OFF). For each to-be-processed pixel, BO uses the pixel intensity to classify the pixel into a band. The pixel intensity range is equally divided into 32 bands, as shown in Figure 2. After pixel classification, one offset is derived for all pixels of each band, and the offsets of center 16 bands or outer 16 bands are selected and coded. As for EO, it uses two neighboring pixels of a to-be-processed pixel to classify the pixel into a category. Similarly, one offset is derived for all pixels of each category after pixel classification except for category 0, which is forced to use zero offset. Table 1 shows the EO pixel classification, where “c” denotes the pixel to be classified. Figure 3 shows four EO types with different selections of neighboring pixels.

Figure 2.  Pixel intensity range equally divided into 32 bands by BO

Table 1.  EO pixel classification

Figure 3.  Four EO types

## Adaptive loop filter

In HM-4.0, ALF has two filter shape options, cross11x5 and snowflake5x5, for luma and chroma, as shown in Figure 4. In each picture, the luma component can choose one shape, and the chroma components can choose one shape. Up to 16 luma filters and at most one chroma filter can be applied for each picture. In order to allow localization of ALF, there are two modes for luma pixels to select filters. One is a region-based adaptation (RA) mode, and the other is a block-based adaptation (BA) mode. The RA mode divides one luma picture into 16 regions. Once the picture size is known, the 16 regions are determined and fixed. The regions can be merged, and one filter is used for each region after merging. The BA mode uses edge activity and direction as a property for each 4x4 block. Calculating the property of a 4x4 block requires 5x5 pixels, as shown in Figure 5. After properties of 4x4 blocks are calculated, they are classified into 15 categories. The categories can be merged, and one filter is used for each category after merging. As for the chroma components, since they are relatively flat, no local adaptation is used, and the two components of a picture share one filter.

Figure 4.  Two filter shape options

Figure 5.  Pixels required for edge property calculation of a 4x4 block in BA mode

## LCU-based decoding

Figure 6 shows the current implementation of HM-4.0 software decoder. The decoding process is divided into two parts. One is LCU-based processing including intra prediction (IP), motion compensation (MC), inverse transform (IT), inverse quantization (IQ), and reconstruction (REC), and the other is picture-based processing including DF, SAO, and ALF. Entropy decoding (ED) belongs to the picture-based processing when SPS, PPS, or slice-level syntax elements are parsed and belongs to the LCU-based processing when syntax elements of LCUs are parsed. In PC-based software, picture-based processing is easier to implement than LCU-based processing for DF, SAO, and ALF. However, in hardware and embedded software, picture-based processing is practically unacceptable due to its high picture buffer requirement. Using on-chip picture buffers is very expensive, and using off-chip picture buffers significantly increases external memory access, power consumption, and data access latency. Therefore, DF, SAO, and ALF will definitely be changed from picture-based to LCU-based decoding in real products.

Figure 6.  Block diagram of HM-4.0 software decoder

When LCU-based processing is used for DF, SAO, and ALF, the entire decoding process can be done LCU by LCU in a raster scan with an LCU-pipelining fashion for parallel processing of multiple LCUs. In this case, line buffers are required for DF, SAO, and ALF because processing one LCU row requires pixels from the above LCU row. If off-chip line buffers (e.g. DRAM) are used, the external memory bandwidth and power consumption will be increased; if on-chip line buffers (e.g. SRAM) are used, the chip area will be increased. Therefore, although line buffers are already much smaller than picture buffers, it is still desirable to reduce line buffers.

## Luma line buffers of in-loop filtering

Figure 7 explains the number of luma line buffers required for DF, SAO, and ALF with LCU-based decoding. Given a current LCU, lines …A-J are first processed by horizontal DF and then by vertical DF. Next, it is suggested to delay the horizontal DF for lines K-N until the lower LCU comes in order to avoid line buffers of H-DF pixels for the vertical DF for lines K-N, which cannot be processed at this moment because the lower LCU is not yet available and DF needs four pre-DF pixels and four H-DF pixels on each side of the horizontal boundary for deriving filter parameters and filtering, respectively, as illustrated by the 4-pixel yellow stripe. Therefore, four lines (K-N) of pre-DF pixels have to be stored for DF. Next, SAO is going to be applied on DF output pixels. Since the vertical DF for lines K-N will not change line K, horizontal DF can be additionally applied on line K for SAO to process line J, as illustrated by the 3x3 red square. Please note that the H-DF pixels of line K will not be stored in the line buffer and have to be generated again when the lower LCU comes, which is not a problem in hardware. After SAO processes lines …A-J, the 4x4 block property, as illustrated by the 4x4 blue square, can be calculated. At this moment ALF can process lines …A-H. After this, no further process can be done for the current LCU until the lower LCU comes. When the lower LCU comes, lines K-P… are first processed by DF, and then lines K-P… are processed by SAO. When SAO processes line K, line J is required. Fortunately, only EO partial results from comparing pixels on line K and pixels on line J have to be stored instead of pixel values. The partial results need 2 bits per pixel, which requires only 20% and 25% of the pixel line buffer for high efficiency (HE) conditions using 10-bit pixels and low complexity (LC) conditions using 8-bit pixels, respectively. Therefore, one line (J) of SAO partial results has to be stored for SAO. Next, the 4x4 block properties for lines I-P… are calculated. Finally, lines I-P… can be filtered by ALF. When line I is filtered, it requires lines G-K, as illustrated by the filter shape painted in green. However, calculating block properties of lines I-J still needs lines F-J. Therefore, five lines (F-J) of SAO output pixels have to be stored for ALF. If a filter index line buffer can be used to store BA mode filter selections for lines G-J, it is not necessary to compute the block properties again during filtering of lines I-J. In this way, one line (F) of SAO output pixels can be saved for ALF. The filter index line buffer (4 bits per block) requires only about 10% size of the pixel line buffer. Therefore, four lines (G-J) of SAO output pixels and one line of filter indices (4x4 blocks on lines G-K) have to be stored for ALF. In total, the entire in-loop filtering requires about 8.3 luma line buffers. When the entire decoding system is considered, since the intra luma prediction already stores one line (N) of pre-DF pixels, this luma line buffer can be shared.

Figure 7.  Illustration of luma line buffers required for DF, SAO, and ALF

## Chroma line buffers of in-loop filtering

Figure 8 explains the number of chroma line buffers required for DF, SAO, and ALF with LCU-based decoding. Given a current LCU, lines …A-M are first processed by horizontal DF, and then lines …A-L are processed by vertical DF. Next, it is suggested to delay the horizontal DF for line N until the lower LCU comes in order to share the line buffer of pre-DF pixels of line N with intra chroma prediction. As for the vertical DF for lines M-N, it has to wait for the lower LCU. Please recall that chroma DF reads two H-DF pixels and may write one pixel on each side of the boundary, as illustrated by the 2-pixel yellow stripe. Therefore, one line (M) of H-DF pixels and one line (N) of pre-DF pixels have to be stored for DF. Next, SAO is going to be applied on DF output pixels. Since the vertical DF for lines M-N will not change line M, H-DF pixels of line M are also DF output pixels of line M, and SAO can process line L, as illustrated by the 3x3 red square. After SAO processes lines …A-L, ALF can process lines …A-J. After this, no further process can be done for the current LCU until the lower LCU comes. When the lower LCU comes, lines N-P… are first processed by horizontal DF, and then lines M-P… are processed by vertical DF and by SAO. When SAO processes line M, line L is required. Fortunately, only EO partial results from comparing pixels on line M and pixels on line L have to be stored instead of pixel values. The partial results need 2 bits per pixel, which requires only 20% and 25% of the pixel line buffer for HE conditions using 10-bit pixels and LC conditions using 8-bit pixels, respectively. Therefore, one line (L) of SAO partial results has to be stored for SAO. Finally, lines K-P… can be filtered by ALF. When line K is filtered, it requires lines I-M, as illustrated by the filter shape painted in green. Therefore, four lines (I-L) of SAO output pixels have to be stored for ALF. In total, the entire in-loop filtering requires about 6.2 chroma line buffers.

Figure 8.  Illustration of chroma line buffers required for DF, SAO, and ALF

# Proposed virtual boundary processing

## Virtual boundary

In order to eliminate the line buffer requirements of SAO and ALF, we introduce the concept of virtual boundary (VB). As shown in Figure 9, VBs are upward shifted horizontal LCU boundaries by N pixels. For each LCU, SAO and ALF can process pixels above the VB before the lower LCU comes but cannot process pixels below the VB until the lower LCU comes, which is caused by DF. With consideration of the DF in HM-4.0, the space between the proposed VB and the horizontal LCU boundary is set as four pixels for luma (i.e. N=4 in Figure 9) and two pixels for chroma (i.e. N=2 in Figure 9). Next, SAO is modified for every to-be-processed pixel on one side of a VB to reduce data access from the other side of the VB. Finally, ALF is modified for every to-be-processed pixel on one side of a VB to avoid any data access from the other side of the VB.

Figure 9.  Illustration of VBs in one picture

Figure 7 can be used to explain the luma case where the VB is painted in red and four pixels above the horizontal LCU boundary. When the current LCU is processed, lines K-N cannot be processed by vertical DF because the lower LCU is not yet available. At this moment, if SAO and ALF processes for lines …A-J do not require any pixel below the VB, lines …A-J can be processed without the lower LCU. When the lower LCU comes, lines K-P… are processed by DF, SAO, and ALF. At this time, if SAO and ALF processes for lines K-P… can use reduced or zero data above the VB, line buffers for storing lines F-J can be reduced or removed.

Figure 8 can be used to explain the chroma case where the VB is painted in red and two pixels above the horizontal LCU boundary. When the current LCU is processed, lines M-N cannot be processed by vertical DF because the lower LCU is not yet available. At this moment, if SAO and ALF processes for lines …A-L do not require any pixel below the VB, lines …A-L can be processed without the lower LCU. When the lower LCU comes, lines M-P… are processed by DF, SAO, and ALF. At this time, if SAO and ALF processes for lines M-P… can use reduced or zero data above the VB, line buffers for storing lines I-L can be reduced or removed.

In this contribution, two solutions will be provided. Non-CE8.c.7-1 applies VB processing to ALF only, so the number of line buffers for the entire in-loop filtering is reduced from 8.3 to 4.2 for luma and from 6.2 to 2.2 for chroma. Non-CE8.c.7-2 applies VB processing to both SAO and ALF, and the number of line buffers for the entire in-loop filtering becomes 4.1 for luma and 2.1 for chroma. The ALF VB processing in both solutions is exactly the same as that in CE8.c.4-2.

## Luma virtual boundary processing

The luma VB processing can be divided into two parts. The first part handles pixels above the VB, while the second part handles pixels below the VB. Figures 10-18 illustrate the proposed luma VB processing for pixels above the VB. In Figure 10, all pixels of the current LCU have been processed by REC, and four lines (p0-p3) of pre-DF pixels are stored in DF line buffers. In Figure 11, pixels above the VB and one line (p3) below the VB are processed by horizontal DF. Please recall that luma DF reads four pixels and writes up to three pixels on each side of the 8x8 block boundary. In Figure 12, pixels above the VB are processed by vertical DF. In Figure 13, pixels above the VB are going to be processed by SAO. At this moment, line p3 has been processed by horizontal DF in Figure 11 and will not be changed by vertical DF. Therefore, DF output pixels of line p3 are available for SAO to process line p4. During SAO processing for line p4, each to-be-processed pixel on line p4 (denoted as C) needs to be compared with a neighboring pixel on line p3 (denoted as N) if nonzero degree EO is selected, and these SAO partial results are stored in SAO line buffer. In non-CE8.c.7-1, each to-be-processed pixel requires two bits to represent C>N, C<N, or C==N, so the number of SAO pixel line buffers is equivalent to 0.2 and 0.25 in HE conditions using 10-bit pixels and LC conditions using 8-bit pixels, respectively. In non-CE8.c.7-2, the partial results of two to-be-processed pixels (C1, C2) and two neighboring pixels (N1, N2) are compressed from four bits to two bits to represent C1>N1 && C2>N2, C1<N1 && C2<N2, or none of the above, so the number of SAO pixel line buffers is equivalent to 0.1 and 0.125 in HE and LC, respectively. In Figure 14, all pixels above the VB have been processed by SAO. In Figures 15-17, pixels above the VB are going to be processed by ALF. During filtering, SAO output pixels below the VB may be originally needed. In these cases, filtering has to be modified. In Figure 15, where snowflake5x5 is selected for ALF, filtering line p5 uses padding and averaging, and filtering line p4 is skipped. Padding means a pixel on the other side of the VB is replaced by its nearest pixel on the same side of the VB, and averaging means the filter output pixel is averaged with the filter input pixel as the final ALF output pixel. In Figure 16, where cross9x9 is selected for ALF, the filter size is reduced to 9x7 and 9x5 for filtering line p7 and line p6, respectively, with adding the discarded coefficients to the center pixel. In Figure 17, where cross9x9 is also selected for ALF, the filter size is reduced to 9x3 and 9x1 for filtering line p5 and line p4, respectively, with adding the discarded coefficients to the center pixel. In this way, it is unnecessary to change ALF syntax, and normalization of coefficients with multiplications and divisions can be avoided. In Figure 18, all pixels above the VB have been processed by ALF. At this moment, pixels above the VB can be written to a decoded picture buffer before the lower LCU comes.

Figure 10.  Luma, above VB, before DF

Figure 11.  Luma, above VB, after horizontal DF

Figure 12.  Luma, above VB, after vertical DF

Figure 13.  Luma, above VB, store SAO partial results when SAO processes line p4

Figure 14.  Luma, above VB, after SAO

Figure 15.  Luma, above VB, modified ALF snowflake filtering for processing lines p4-p5

Figure 16.  Luma, above VB, modified ALF cross filtering for processing lines p6-p7

Figure 17.  Luma, above VB, modified ALF cross input pixels for processing lines p4-p5

Figure 18.  Luma, above VB, after ALF

Figures 19-28 illustrate the proposed luma VB processing for pixels below the VB. In Figure 19, four lines (p0-p3) of pre-DF pixels can be read from the DF line buffers. In Figure 20, pixels below the VB are processed by horizontal DF. Please note that calculating horizontal DF decisions of lines p0-p3 requires pre-DF pixels of lines p0-p7. In order to save line buffers for lines p4-p7, these horizontal DF decisions are computed and stored in a decision line buffer during the horizontal DF for lines p3-p7 in Figure 11. The decision buffer only requires one bit per 8x8 block and can be simply implemented as on-chip registers or SRAMs. In Figure 21, pixels below the VB are processed by vertical DF. Please remember that vertical DF decisions use pre-DF pixels, so the vertical DF decisions at the horizontal LCU boundary have to be calculated before the horizontal DF is done in Figure 20. In Figure 22, pixels below the VB are going to be processed by SAO. During SAO processing for line p4 in Figure 13, each pixel on line p4 (denoted as C) was compared with a neighboring pixel on line p3 (denoted as N) for nonzero degree EO, and these SAO partial results were stored in SAO line buffer. Now, the partial results can be read from SAO line buffer for SAO processing of line p3. In non-CE8.c.7-1, each to-be-processed pixel on line p3 (N) and its corresponding pixel on line p4 (C) decode two bits from SAO line buffer to know whether it is C>N, C<N, or C==N. In non-CE8.c.7-2, two to-be-processed pixels on line p3 (N1, N2) and two corresponding pixels on line p4 (C1, C2) decode two bits from SAO line buffer to know whether it is C1>N1 && C2>N2, C1<N1 && C2<N2, and none of the above. If none of the above is selected, C1==N1 && C1==N2 will be used in the EO process. In Figure 23, all pixels below the VB (and above the next VB) have been processed by SAO. In Figures 24-27, pixels below the VB are going to be processed by ALF. Originally, calculating ALF block properties of lines p0-p3 requires SAO output pixels of lines p0-p4. However, SAO output pixels of line p4 are not available any more. Therefore, as shown in Figure 24, repetitive padding in the vertical direction is applied to generate pixels of line p4 from SAO output pixels of line p3. During filtering, SAO output pixels above the VB may also be originally needed. In these cases, filtering has to be modified. In Figure 25, where snowflake5x5 is selected for ALF, filtering line p3 is skipped, and filtering line p2 uses padding and averaging. In Figure 26, where cross9x9 is selected for ALF, the filter size is reduced to 9x1 and 9x3 for filtering line p3 and line p2, respectively, with adding the discarded coefficients to the center pixel. In Figure 27, where cross9x9 is also selected for ALF, the filter size is reduced to 9x5 and 9x7 for filtering line p1 and line p0, respectively, with adding the discarded coefficients to the center pixel. In Figure 28, all pixels below the VB (and above the next VB) have been processed by ALF. At this moment, pixels below the VB (and above the next VB) can be written to a decoded picture buffer.

Figure 19.  Luma, below VB, before DF

Figure 20.  Luma, below VB, after horizontal DF

Figure 21.  Luma, below VB, after vertical DF

Figure 22.  Luma, below VB, read SAO partial results when SAO processes line p3

Figure 23.  Luma, below VB, after SAO

Figure 24.  Luma, below VB, modified ALF block properties for processing lines p0-p3

Figure 25.  Luma, below VB, modified ALF snowflake filtering for processing lines p2-p3

Figure 26.  Luma, below VB, modified ALF cross filtering for processing lines p2-p3

Figure 27.  Luma, below VB, modified ALF cross filtering for processing lines p0-p1

Figure 28.  Luma, below VB, after ALF

The design principles of luma VB processing are summarized in Table 2. In Figure 11 and Figure 20, it can be seen that line p3 is processed by horizontal DF twice. This only happens in LCU-based processing, not in picture-based processing like HM. The redundant computation causes very minor impact in hardware because the DF hardware has been already allocated and the DF is not the throughput bottleneck of the system. The redundant computation can be avoided by adding one line buffer to store H-DF pixels of line p3; however, this is definitely not a good trade-off in hardware.

Table 2.  Summary of the proposed luma VB processing

## Chroma virtual boundary processing

The proposed chroma VB processing has the same concepts and similar principles as the proposed luma VB processing and is summarized in Table 3. Figures 29-37 show the chroma VB processing for pixels above the VB, and Figures 38-46 show the chroma VB processing for pixels below the VB. Please note that there is no redundant computation of chroma horizontal DF for line p1 because the chroma vertical DF does not use any pre-DF pixel and the H-DF pixels of line p1 are already stored in the line buffer.

Table 3.  Summary of the proposed chroma VB processing

Figure 29.  Chroma, above VB, before DF

Figure 30.  Chroma, above VB, after horizontal DF

Figure 31.  Chroma, above VB, after vertical DF

Figure 32.  Chroma, above VB, store SAO partial results when SAO processes line p2

Figure 33.  Chroma, above VB, after SAO

Figure 34.  Chroma, above VB, modified ALF snowflake filtering for processing lines p2-p3

Figure 35.  Chroma, above VB, modified ALF cross filtering for processing lines p4-p5

Figure 36.  Chroma, above VB, modified ALF cross filtering for processing lines p2-p3

Figure 37.  Chroma, above VB, after ALF

Figure 38.  Chroma, below VB, before DF

Figure 39.  Chroma, below VB, after horizontal DF

Figure 40.  Chroma, below VB, after vertical DF

Figure 41.  Chroma, below VB, read SAO partial results when SAO processes line p1

Figure 42.  Chroma, below VB, after SAO

Figure 43.  Chroma, below VB, modified ALF snowflake filtering for processing lines p0-p1

Figure 44.  Chroma, below VB, modified ALF cross filtering for processing lines p0-p1

Figure 45.  Chroma, below VB, modified ALF cross filtering for processing lines q0-q1

Figure 46.  Chroma, below VB, after ALF

# Simulation results

HM-4.0 is used as the anchor. High efficiency all intra (HE-AI), high efficiency random access (HE-RA), high efficiency low delay B (HE-LDB), high efficiency low delay P (HE-LDP), low complexity all intra (LC-AI), low complexity random access (LC-RA), low complexity low delay B (LC-LDB), and low complexity low delay P (LC-LDP) common test conditions defined in JCTVC-F900 [6] are tested. Table 4 shows that the proposed VB processing for ALF only (non-CE8.c.7-1) causes 0.0%, -0.1%, -0.3%, and 0.0% luma BD-rate differences for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, where positive numbers mean losses and negative numbers mean gains. Table 4 also shows that non-CE8.c.7-1 results in exactly the same results as HM-4.0 when ALF is disabled. Table 5 shows that the proposed VB processing for SAO and ALF (non-CE8.c.7-2) causes 0.0%, -0.1%, -0.3%, and 0.0% luma BD-rate differences for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively. Table 5 also shows that non-CE8.c.7-2 results in 0.0%, 0.0%, 0.0%, and 0.0% luma BD-rate differences for LC-AI, LC-RA, LC-LDB, and LC-LDP, respectively, when ALF is disabled. In both solutions, the encoding time and the decoding time are roughly unchanged. As for the subjective quality, after carefully viewing of all video sequences, we observed that the visual qualities of the two proposed solutions are the same as that of the anchor. No VB artifact is observed. Interested readers are strongly encouraged to contact us for evaluating the visual quality.

# Conclusions

MediaTek, Panasonic, and Qualcomm implemented one solution of single-source ALF virtual boundary processing and one solution of single-source SAO and ALF virtual boundary processing to remove most SAO and ALF line buffers in LCU-based decoding systems. Moreover, cross9x9 and snowflake5x5 for ALF are combined. Simulation results reportedly show that the proposed two solutions both achieve 0.0%, -0.1%, -0.3%, and 0.0% BD-rates for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, with the same subjective quality in comparison with HM-4.0.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 4.  Results of non-CE8.c.7-1, single-source ALF VB processing with cross9x9

Table 5.  Results of non-CE8.c.7-2, single-source SAO and ALF VB processing with cross9x9

# References

- C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei, “Adaptive loop filter with zero pixel line buffers for LCU-based decoding,” Document of Joint Collaborative Team on Video Coding, JCTVC-F054, July 2011.

- C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei, “Sample adaptive offset with zero pixel line buffers for LCU-based decoding,” Document of Joint Collaborative Team on Video Coding, JCTVC-F055, July 2011.

- T. Yamakage, I. S. Chong, M. Narroschke, Y.-W. Huang, “Description of Core Experiment 8 (CE8): Non-deblocking loop filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-F908, July 2011.

- C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei, S. Esenlik, M. Narroschke, T. Wedi, “CE8.c.4: SAO and ALF virtual boundary processing with cross9x9,” Document of Joint Collaborative Team on Video Coding, JCTVC-G206, November 2011.

- C.-Y. Tsai, C.-Y. Chen, C.-M. Fu, Y.-W. Huang, S. Lei, I. S. Chong, M. Karczewicz, T. Yamakage, T. Watanabe, and T. Chujoh, “CE8.d.1: Snowflake5x5 and cross9x9 for luma and chroma ALF shapes,” Document of Joint Collaborative Team on Video Coding, JCTVC-G208, November 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.