| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G213 |
| --- | --- |

| Title: | Non-CE4: Performance comparison between temporal QP prediction and temporal MV prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution provides an additional analysis report of temporal QP prediction (TQPP) which is listed in CE4 Subtest 1 as the test 1.4. TQPP introduces additional buffer for storing QPs of reference frames for temporal prediction, as temporal MV prediction (TMVP) does. As a measure to evaluate the balance between compression performance and memory requirement, ratio of coding gain to bits per reference CU, namely relative coding gain, is introduced. For the comparison purpose, the performance of TMVP is evaluated in this contribution. It is reportedly shown that the relative coding gain of TQPP is more than that of TMVP.

# Introduction

In the current Working Draft (WD4) of HEVC [1], advanced motion vector prediction (AMVP) scheme is used for efficient compression of motion information. In AMVP, a temporal motion vector prediction (TMVP), where a temporally collocated motion vector is used as a candidate of the motion vector predictor, is employed. For TMVP, motion vectors should be stored into a buffer associated with each reference frame and the storage size can be significant due to fine granularity of motion vectors. A motion vector buffer compression method [2, 3] is therefore introduced into WD4 and thereby motion vectors are summarized on a 16x16 block basis by default.

Meanwhile, temporal QP prediction (TQPP), which is another temporal prediction scheme, has been proposed [4] and now it is tested in CE4 Subtest 1 as the test 1.4 [5, 6]. This contribution reports the performance of TMVP compared with that of TQPP. In addition to BD-rates and decoding/encoding times, a new metric is introduced to evaluate the balance between compression performance and memory consumption.

# Experiments

## Setup

The experiment was conducted with the HM 4.0 software under the common test conditions [6]. The performance of TMVP was evaluated by comparing HM 4.0 with the default setting against that with TMVP disabled. In order to disable TMVP, macro definitions in TypeDef.h were modified as follows.

#define MRG_NEIGH_COL 0
#define AMVP_NEIGH_COL 0

The computing platform used for this experiment is shown below.

- OS: Windows 7 Professional 64-bit

- CPU: Intel Xeon X5680 3.46GHz, 6 physical cores x 2 CPUs (12 cores in total)

- Memory: 32GiB

## Results

Results are summarized in Table 1. More detailed results are shown in the attached spreadsheet. These results reveal that TMVP achieves high coding gains at the expense of several percentage of encoding/decoding time increase.

Table 1: Summary results of HM 4.0 with TMVP against HM 4.0 without TMVP.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -2.5% | -1.9% | -2.0% | -2.6% | -2.3% | -2.4% |
| Class B | -2.1% | -1.5% | -1.4% | -2.3% | -1.4% | -1.4% |
| Class C | -2.2% | -1.9% | -2.0% | -2.3% | -1.9% | -2.0% |
| Class D | -2.6% | -2.0% | -2.0% | -2.8% | -1.9% | -2.1% |
| Class E |  |  |  |  |  |  |
| Overall | -2.4% | -1.8% | -1.8% | -2.5% | -1.9% | -2.0% |
|  | -2.4% | -1.8% | -1.8% | -2.5% | -1.9% | -2.0% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3.2% | -2.7% | -2.3% | -4.1% | -2.7% | -3.1% |
| Class C | -3.4% | -3.3% | -3.4% | -3.9% | -3.3% | -3.4% |
| Class D | -3.1% | -2.7% | -2.4% | -3.5% | -2.9% | -2.8% |
| Class E | -3.4% | -3.1% | -3.6% | -4.1% | -2.7% | -3.0% |
| Overall | -3.2% | -2.9% | -2.8% | -3.9% | -2.9% | -3.1% |
|  | -3.2% | -2.9% | -2.8% | -3.9% | -2.9% | -3.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 96% |  |  | 94% |  |  |

## Performance analysis compared with TQPP

TMVP needs buffer memory for storing motion data, although motion data are compressed on a 16x16 block basis by default. Here we analyze the balance between compression performance and memory consumption of TMVP and compare that of TMVP with that of TQPP. Supposing that a maximum of 4 reference frames are used in each list and that motion vector component range is [-511.75, +512.0] as in Level 3.1 or higher of MPEG-4 AVC/H.264, 54 (= (12x2+2+1)x2 bits need to be consumed for each 16x16 block on reference frame, whereas 6 bits need to be consumed for each 16x16 block on reference frame when enabling TQPP. Table 2 shows ratios of coding gain to bits per reference 16x16 block, which are calculated by dividing the negative of the luma BD-Rate value by the number of bits consumed for each 16x16 block. Table 3 shows another set of results, considering QP buffer memory may be byte-aligned as discussed in JCTVC-G068 [5].

As shown here, coding gain relative to memory requirement of TQPP is higher than that of TMVP. Here the same buffer compression scheme, where all data used for temporal prediction are stored for each 16x16 block, is applied both to TQPP and TMVP. The column highlighted in boldface type shows the values under the test condition in [5]. From these results, it is shown that the relative coding gain of TQPP is higher than that of TMVP in most of test cases, and it is almost equal to that of TMVP even in the worst case. The gain of TQPP attains up to about 20 times of that of TMVP when delta QP is signalled at 64x64 block level under the LB-HE and LB-LC configurations. Although it does not mean that TQPP is up to 20 times better than TMVP, it at least indicates that introducing TQPP into QP coding should be considered.

Table 2: Ratios of coding gain to bits per reference 16x16 block when data are unaligned.

|  | TMVP | TQPP |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 8x8 | 16x16 | 32x32 | 64x64 |
| RA.HE | 0.044% | 0.055% | 0.057% | 0.172% | 0.437% |
| RA.LC | 0.046% | 0.059% | 0.064% | 0.184% | 0.512% |
| LB.HE | 0.060% | 0.115% | 0.122% | 0.484% | 1.264% |
| LB.LC | 0.072% | 0.146% | 0.156% | 0.548% | 1.432% |

Table 3: Ratios of coding gain to bits per reference 16x16 block when data are byte-aligned.

|  | TMVP | TQPP |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 8x8 | 16x16 | 32x32 | 64x64 |
| RA.HE | 0.042% | 0.041% | 0.043% | 0.129% | 0.327% |
| RA.LC | 0.045% | 0.044% | 0.048% | 0.138% | 0.384% |
| LB.HE | 0.058% | 0.086% | 0.092% | 0.363% | 0.948% |
| LB.LC | 0.069% | 0.110% | 0.117% | 0.411% | 1.074% |

# Conclusions

This contribution has provided an additional analysis report of temporal QP prediction (TQPP) which is listed in CE4 Subtest 1 as the test 1.4. TQPP introduces additional buffer for storing QPs of reference frames for temporal prediction, as temporal MV prediction (TMVP) does. As a measure to evaluate the balance between compression performance and memory requirement, ratio of coding gain to bits per reference CU, namely relative coding gain, is introduced. For the comparison purpose, the performance of TMVP is evaluated in this contribution. It is reportedly shown that the relative coding gain of TQPP is more than that of TMVP. It is recommended that introducing TQPP technique into the QP coding of HEVC is considered

# References

[1] B. Bross, et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC document JCTVC-F803, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[2] Y. Su and A. Segall, “On motion vector competition,” JCT-VC document JCTVC-C257, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 3rd meeting, Guangzhou, October 2010.

[3] Y. Su and A. Segall, “CE9: Reduced resolution storage of motion vector data,” JCT-VC document JCTVC-D072, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 4th meeting, Daegu, January 2011.

[4] H. Aoki, et al., “CE4 Subtest2: QP prediction based on intra/inter prediction (test 2.4.b),” JCT-VC document JCTVC-F103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 6th meeting, Torino, July 2011.

[5] K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-F904, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 6th meeting, Torino, July 2011.

[6] H. Aoki, et al., “CE4 Subtest 1: QP prediction based on intra/inter prediction (test 1.4),” JCT-VC document JCTVC-G066, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, November 2011.