| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G214 |
| --- | --- |

| Title: | Non-CE8: Constrained ALF coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chia-Yang Tsai, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai, chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-4.0, ALF coefficients are unconstrained, which is difficult to decide the bit width of multipliers in hardware implementations. In this contribution, center coefficients are clipped within [0, 2), non-center coefficients are constrained within [-1, 1), and offsets are also clipped within [-2D, 2D), where D is the pixel bit depth. It is reported that the constrained coefficient ranges do not cause any coding efficiency loss or run time change.

# Introduction

In High-Efficiency Video Coding test model version 4.0 (HM-4.0), adaptive loop filter (ALF) can significantly improve coding efficiency. However, the coefficient range is unconstrained. Therefore, it is difficult to determine the bit width of multipliers in hardware. In this contribution, coefficient ranges are clipped to solve the problem. The rest of this contribution is organized as follows. The proposed method will be described in Section 2, simulation results will be shown in Section 3, and finally conclusions will be given in Section 4.

# Proposed method

Figure 1 shows the ALF coefficients of two filter shapes, snowflake5x5 and cross11x5, in HM-4.0. In snowflake5x5, C8 is the center coefficient, while the others are non-center coefficients. In cross11x5, C7 is the center coefficient, while the others are non-center coefficients.

Figure 1.  ALF coefficients in HM-4.0

After analyzing the ranges of coefficient values, it is found that center coefficients tend to be positive and close to 1.0, and non-center coefficients tend to be close to 0. Therefore, we propose to clip the center coefficient within [0, 2), non-center coefficients within [-1, 1), and offsets within [-2D, 2D), where D is the pixel bit depth.

In HM-4.0, the coefficient precision of the fractional part is 8-bit. That is to say, the proposed center coefficient range after clipping is [0, 1.99609375], and the proposed non-center coefficient range after clipping is [-1, 0.99609375]. In hardware, for a center coefficient, the multiplier inputs are N-bit for the pixel and 9-bit for the coefficient, and the multiplier output is (N+9)-bit, where N is the pixel bit depth; for a non-center coefficient, the multiplier inputs are N-bit for the pixel and 9-bit for the coefficient, and the multiplier output is (N+8)-bit. As for the ALF offset, the proposed range is [-(1<<N), 1<<N), and the fractional precision is the same as that in HM4.0.

# Simulation results

The software is implemented on HM-4.0. High efficiency all intra (HE-AI), high efficiency random access (HE-RA), high efficiency low delay B (HE-LDB), and high efficiency low delay P (HE-LDP) common test conditions defined in JCTVC-F900 [1] were tested. The results of the proposed coefficient range constraints are shown in Table 1. There is no impact on coding efficiency or run time.

# Conclusions

MediaTek proposed to constrain center coefficients, non-center coefficients, and offsets within [0, 2), [-1, 1), and [-2D, 2D), respectively, for ALF, where D is the pixel bit depth. Simulation results reportedly show that the proposed method does not change coding efficiency or run time.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.

Table 1.  Results of constraining ALF coefficient ranges