#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| if ( adaptive_loo_filter_enabled_flag ) |  |
| alf_max_num_filter_minus_one | u(4) |
| pcm_loop_filter_disable_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| inter_4x4_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Adaptive loop filter parameter syntax

| alf_param() { | Descriptor |
| --- | --- |
| adaptive_loop_filter_flag | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |
| alf_region_adaptation_flag | u(1) |
| alf_length_luma_minus_5_div2 | ue(v) |
| alf_no_filters_minus1 | ue(v) |
| if (alf_no_filters_minus1 == 1) |  |
| alf_start_second_filter | ue(v) |
| else if (alf_no_filters_minus1 > 1) { |  |
| for (i=1; i< 16; i++) |  |
| alf_filter_pattern[i] | u(1) |
| } |  |
| if (AlfNumFilters > 1) |  |
| alf_pred_method | u(1) |
| alf_min_kstart_minus1 | ue(v) |
| for (i=0; i < AlfMaxDepth; i++) |  |
| alf_golomb_index_bit[i] | u(1) |
| if (AlfNumFilters > alf_max_num_filter_minus_one+1) |  |
| AlfNumFilters = alf_max_num_filter_minus_one+1 |  |
| for (i=0; i< AlfNumFilters; i++) |  |
| for (j=0; j< AlfCodedLengthLuma; j++) |  |
| alf_coeff_luma[i][j] | ge(v) |
| alf_chroma_idc | ue(v) |
| if ( alf_chroma_idc ) { |  |
| alf_length_chroma_minus_5_div2 | ue(v) |
| for( i = 0; i< AlfCodedLengthChroma; i++ ) |  |
| alf_coeff_chroma[i] | se(v) |
| } |  |
| } |  |
| } |  |

#### Sequence parameter set RBSP semantics

alf_max_num_filter_minus_one specifies the max number of filters per picture.