| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G215 |
| --- | --- |

| Title: | Non-CE8: Limited number of filters per picture for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chia-Yang Tsai, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai, chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In ALF of HM-4.0, up to 16 filters per picture are used for luma, and 16 filters may be too many for low resolution applications. In this contribution, a maximum number of filters per picture can be given on the encoder side, and the ALF encoder will merge regions or classes accordingly. Simulation results reportedly show that when the maximum number of filters per picture is 10, no coding efficiency loss is observed. When the maximum number of filters per picture is six, 0.1% bit rate increase is observed in LDP. Proper values for the maximum number of filters can be discussed when common test conditions are defined. Allowed values can be decided when profiles and levels are defined.

# Introduction

In the adaptive loop filtering (ALF) of High-Efficiency Video Coding test model version 4.0 (HM-4.0), up to 16 luma filters and at most one chroma filter can be applied for each picture. In order to allow localization of ALF, there are two modes for luma pixels to select filters. One is a region-based adaptation (RA) mode, and the other is a block-based adaptation (BA) mode. The RA mode divides one luma picture into 16 regions. Once the picture size is known, the 16 regions are determined and fixed. The regions can be merged, and one filter is used for each region after merging. The BA mode uses edge activity and direction as a property for each 4x4 block. After properties of 4x4 blocks are calculated, they are divided into 15 classes. The classes can be merged, and one filter is used for each class after merging. As for the chroma components, since they are relatively flat, no local adaptation is used, and the two components of a picture share one filter. After analyzing the number of luma filters coded in each picture, as expected, it is highly dependent on picture resolution. A small resolution picture may only use one filter. Therefore, it is desirable to use different maximum numbers of filters per picture for different picture resolutions. The rest of this contribution is organized as follows. The proposed method will be described in Section 2, simulation results will be shown in Section 3, and finally conclusions will be given in Section 4.

# Proposed method

We tentatively signal a maximum number of filters per picture in the sequence parameter set (SPS). The only change of the encoding algorithm is to merge regions in RA mode or merge classes in BA mode if the maximum number of filters per picture is exceeded. There is no decoding process change. The temporary syntax element in SPS can be used for the decoder to check if the bitstream violates the constraint on the maximum number of filters per picture. After profiles and levels are defined, the added SPS syntax element should be removed from the HM software.

# Simulation results

The software is implemented on top of HM-4.0. High efficiency all intra (HE-AI), high efficiency random access (HE-RA), high efficiency low delay B (HE-LDB), and high efficiency low delay P (HE-LDP) common test conditions defined in JCTVC-F900 [1] were tested. When the maximum number of filters per picture is set as 10, the results are shown in Table 1. No coding efficiency loss is observed. When the maximum number of filters is set as six, the results are shown in Table 2. Only 0.1% coding efficiency loss is observed in HE-LDP.

# Conclusions

MediaTek proposed to constrain the maximum number of filters per picture for ALF. Proper constraint values should be defined during discussion of profiles and levels.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.

Table 1.  Results of using up to 10 filters per picture for luma

Table 2.  Results of using up to six filters per picture for luma