| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G217 |
| --- | --- |

| Title: | Non-CE9: Derivation process of reference indices for temporal merging candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@ jvckenwood.com fukushima.shigeru@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In HM4.0, the reference indices of temporal merging candidates are derived by the majority decision of the reference indices of three neighboring blocks.

This contribution proposes that simplified derivation process of reference indices for temporal merging candidates.

The simulation results report that the proposed technique provides no coding loss for random access settings and low delay B settings.

# Introduction

In HM4.0, the reference indices of temporal merging candidates are derived by the majority of the reference indices of three blocks. The conditional judgment for the majority decision is complicated.

This contribution proposes that simplified derivation process of reference indices for temporal merging candidates.

# Algorithm Description

## Current algorithm (HM4.0)

In HM4.0, the reference indices of temporal merging candidates are derived by the majority decision of the reference indices of the neighboring block A1, B1 Corner (B0, A1 or B2).

Figure 1 neighboring block referred in HM4.0

## Proposed algorithm

In the proposed technique, the reference indices of temporal merging candidates are derived from comparison of the reference indices of the neighboring block A1 and B1.

Figure 2 neighboring block referred in proposed technique

For the derivation of refIdxLXA and refIdxLXB, the following applies:

  - If neighboring block A1 is available, refIdxLXA is the refIdxLX of neighboring block A1.
Otherwise, refIdxLXA is not equal to -1

  - If neighboring block B1 is available, refIdxLXB is the refIdxLX of neighboring block B1.
Otherwise, refIdxLXB is not equal to -1

The refIdxLX is derived as follows:

  - If refIdxLXA and refIdxLXB are not equal to -1, refIdxLX is assigned to Min( refIdxLXA, refIdxLXB ).

  - Otherwise, if refIdxLXA is not equal to -1, refIdxLX is assigned to refIdxLXA.

  - Otherwise, if refIdxLXB is not equal to -1, refIdxLX is assigned to refIdxLXB.

  - Otherwise, refIdxLX is assigned to 0.

Figure 3 Proposed algorithm

## Implementation

### Software

This proposed technique is implemented into HM4.0 software.

# Experiments

## Test conditions

The common test conditions [1] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2 and 3 present the simulation results for random access, and low delay B, respectively.

Table 2 Simulation results for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3 Simulation results for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.1% |
| Class D | 0.0% | 0.3% | 0.3% | 0.0% | 0.3% | 0.0% |
| Class E | 0.1% | 0.3% | 1.0% | 0.0% | 0.0% | -0.1% |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

The simulation results report that the proposed technique provides no coding loss for random access settings and low delay B settings. (See the attached excel sheet)

# Conclusions

This contribution reports that the proposed technique achieves simplification without coding loss.

We recommend that the proposed technique be adopted to the next WD and HM.

# References

[1] “Common test conditions and software reference configurations”,

JCTVC-F900, Torino, July 2011

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. or JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

8.4.2.1.3	Derivation process of reference indices for temporal merging candidate

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

Outputs of this process are (with X being replaced by 0 or 1)

- the reference indices refIdxLX of the current prediction units.

For the derivation of refIdxLXA, the following applies.

- If the prediction unit covering luma location ( xP-1, yP + nPSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLXA is assigned to refIdxLX[ xP-1, yP + nPSH − 1 ].

- Otherwise, refIdxLXA is assigned to -1.

For the derivation of refIdxLXB, the following applies.

- If the prediction unit covering luma location ( xP + nPSW − 1, yP-1 ) is available and PredMode is not MODE_INTRA, refIdxLXB is assigned to refIdxLX[ xP + nPSW − 1, yP-1 ].

- Otherwise, refIdxLXB is assigned to -1.

The variable refIdxLX is derived as follows.

- If refIdxLXA and refIdxLXB are not equal to -1, refIdxLX is assigned to Min( refIdxLXA, refIdxLXB ).

- Otherwise, if refIdxLXA is not equal to -1, refIdxLX is assigned to refIdxLXA.

- Otherwise, if refIdxLXB is not equal to -1, refIdxLX is assigned to refIdxLXB.

- Otherwise, refIdxLX is assigned to 0.