| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G218 |
| --- | --- |

| Title: | Non-CE8: One-stage non-deblocking loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chia-Yang Tsai, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA Tomoo Yamakage, Takayuki Itoh, Takashi Watanabe, Takeshi Chujoh TOSHIBA 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chiayang.tsai, chingyeh.chen, chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com Tomoo Yamakage +81-44-549-2152 {tomoo.yamakage, takayuki2.itoh, takashi39.watanabe, takeshi.chujoh}@toshiba.co.jp |
| Source: | MediaTek Inc., Qualcomm Inc., TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In this contribution, SAO and ALF are combined into one stage by dividing one picture into filter units (FUs) and switching FUs among {ALF, SAO, OFF}, where FUs are LCU-aligned blocks coded in raster scan in APS and the FU-based syntax is also friendly for low latency. It is reported that the proposed method achieves 0.1%, 0.1%, 0.0%, and 0.0% bit rate reductions for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, with a picture-based 2-pass encoding algorithm. The proponents request to adopt this proposal in the next CE8 for studying LCU-based syntax and corresponding low latency encoding algorithms.

# Introduction

In High-Efficiency Video Coding test model version 4.0 (HM-4.0), loop filtering contains deblocking filter (DF), sample adaptive offset (SAO), and adaptive loop filter (ALF). Both SAO and ALF use picture-based encoding algorithms. Separating SAO and ALF as two stages may result in more picture buffer access. On the other hand, the gain of SAO and that of ALF are additive, so the current sequential configuration of SAO and ALF can have better coding efficiency than the parallel configuration that can switch between SAO and ALF, as first proposed in JCTVC-D119 [1]. In this contribution, JCTVC-D119 is improved to combine SAO and ALF into one stage with similar coding efficiency as the current 2-stage design. The rest of this contribution is organized as follows. The proposed one-stage non-DF loop filtering will be described in Section 2, simulation results will be shown in Section 3, and finally conclusions will be given in Section 4.

# Algorithm description

First, one picture can be divided into filter units (FUs), which are raster scanned LCU-aligned blocks. The FU size is coded in APS. FUs are also coded one by one in APS. Second, in each FU, one of three modes, {ALF, SAO, OFF}, can be chosen. The ALF mode uses the block-based adaptation (BA) method in HM-4.0 to select filters, and ALF offsets are replaced by SAO-like offsets, where pixel classification is applied to select offsets. The SAO mode uses the band offset (BO) and edge offset (EO) in HM-4.0. The OFF mode simply skips the FU from non-DF loop filtering. When low complexity conditions are used, the ALF mode is not chosen, which is compatible with the current test conditions. Third, in order to reduce side information, a merge-based syntax is designed for FUs to copy parameters from neighboring FUs, and a repeat row flag is used for FU rows to copy parameters from upper FU rows.

On the encoder side, a picture-based bottom-up merge quadtree algorithm is implemented. The quadtree has five levels from level 0 (entire picture) to level 4 (16x16 regions). The quadtree partitions are designed to align with largest coding unit (LCU) boundaries so that the FU-based syntax can be applied to represent the quadtree regions. Rate-distortion optimization is used for determining the quadtree structure and the best mode (ALF, SAO, or OFF) in each region, where the distortion is estimated from statistic data collected at DF output without real filtering as similarly implemented in HM-4.0 1-pass encoding option. Therefore, one-pass encoding can be achieved if no redesign of filter coefficients is applied.

In order to reduce the decoder storage of SAO and ALF information, a maximum number of filters per picture for ALF and a maximum number of offsets per picture for SAO are coded in sequence parameter set (SPS) and sent to decoders. In fact, these constraints can be defined when profiles and levels are discussed and can be removed from bitstream after constraints are clearly defined. In the current work, we allow at most 16 filters per picture for ALF, which is the same restriction as that of HM-4.0. Although syntax-wise the ALF mode can be used in regions of every level, the encoder only tries the ALF mode at levels 0-2 to reduce complexity. Moreover, up to 1, 4, and 15 filters are allowed in level-2, level-1, and level-0 regions, respectively, for BA of filter selection. Consequently, the constraint on the maximum number of filters per picture is always not violated.

# Simulation results

The proposed one-stage non-DF loop filtering is implemented on HM-4.0-dev (rev 1420). High efficiency all intra (HE-AI), high efficiency random access (HE-RA), high efficiency low delay B (HE-LDB), high efficiency low delay P (HE-LDP), low complexity all intra (LC-AI), low complexity random access (LC-RA), low complexity low delay B (LC-LDB), low complexity low delay P (LC-LDP) common test conditions were tested according to JCTVC-F900 [2]. The anchor (HM-4.0-dev (rev 1420)) is the sequential configuration of SAO and ALF, where the ALF uses the 14-pass encoding algorithm. The results are shown in Table 1, where our work uses 1-pass encoding plus 1-redesign of filter coefficients. The BD-rates of the proposed method are -0.1%, -0.1%, 0.0%, 0.0%, 0.0%, 0.1%, 0.4%, and 0.4% for HE-AI, HE-RA, HE-LDB, HE-LDP, LC-AI, LC-RA, LC-LDB, and LC-LDP respectively, where negative numbers mean coding gains.

# Conclusions

MediaTek, Qualcomm, and TOSHIBA combined SAO and ALF into one stage with a block-based syntax. Simulation results show the coding efficiency of the one-stage non-DF loop filtering can be close to that of the current sequential configuration of SAO and ALF. The proponents request this proposal to be further studied in the next CE8.

Table 1.  Results of the proposed one-stage SAO and ALF

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, M. Karczewicz, I. S. Chong, T. Yamakage, T. Chujoh, T. Watanabe, “CE8 Subset2: A joint proposal on improving the adaptive loop filter in TMuC0.9 by MediaTek, Qualcomm, and Toshiba,” Document of Joint Collaborative Team on Video Coding, JCTVC-D119, January 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.