| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G219 |
| --- | --- |

| Title: | Non-CE9: Construction of MVP list without using scaling operation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@ jvckenwood.com fukushima.shigeru@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In the HM4.0, when a MVP list is constructed, two-times scaling operations are performed at a maximum.

This contribution proposes a method which reduces the number of times of scaling operation.

The simulation results report that the proposed technique provides no gain and loss for random access and low delay B settings.

# Introduction

In the HM4.0, when a MVP list is constructed, two-times scaling operations are performed at a maximum.

This contribution proposes a method which reduces the number of times of scaling operations.

# Algorithm Description

## Current algorithm (HM4.0)

In the HM4.0, when a MVP list is constructed, two-times scaling operations are performed at a maximum. One is for spatial MVP candidates and the other is for a temporal MVP candidate.

Figure 1 Decoding process of MVP in the HM4.0

## Proposed algorithm

In this proposal, when a MVP list is constructed, no scaling operations are performed. After the MVP candidate is determined by mvp_idx, the scaling operation is performed.

Temporal picture distance td or POC needs to be managed with original MV in the MVP list for the scaling operation.

Figure 2 Decoding process of MVP in this proposal

The motion vector predictor mvpLX is derived in the following ordered steps.

- Original motion vectors mvLXA, mvLXB and temporal picture distance distLXA, distLXB is derived from the neighboring prediction unit partitions.

- A original motion vector mvLXCol and temporal picture distance distLXCol is derived from colPu.

- MVP list mvpListLX is constructed as follows.

  - mvLXA and distLXA, if availableFlagLXA is equal to 1

  - mvLXB and distLXB, if availableFlagLXB is equal to 1

  - mvLXCol and distLXCol, if availableFlagLXCol is equal to 1

- When motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

- When mvpListLX is empty, a zero motion vector is added into mvpistLX

- The variable numMVPCandLX is set to the number of elements within the mvpListLX and maxNumMVPCand is set to 2.

- mvpListLX is modified to contain exactly maxNumMVPCand motion vector predictor candidates.

- A scaled motion vector predictor mvpLX is derived from orgMvLX and distLX. The motion vector of mvpListLX[ mvp_idx_lX[ xP, yP ] ] is assigned to the original motion vector origMvLX and the temporal picture distance of mvpListLX[ mvp_idx_lX[ xP, yP ] ] is assigned to distLX.

## Implementation

### Software

This proposed technique is implemented into HM4.0 software.

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2 and 3 present the simulation results for random access and low delay B, respectively.

Table 2 Simulation results for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 3 Simulation results for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.3% |
| Class D | 0.0% | 0.0% | 0.4% | 0.0% | -0.1% | -0.1% |
| Class E | 0.0% | 0.3% | 0.7% | 0.0% | -0.6% | 0.1% |
| Overall | 0.0% | 0.0% | 0.3% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.2% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

The simulation results report that the proposed technique provides no gain and loss for random access and low delay B settings. (See the attached excel sheet)

# Conclusions

We recommend that the proposed technique be further studied in CE activity.

# Future Work

# References

[1] “WD4: Working Draft 4 of High-Efficiency Video Coding”,

JCTVC-F803, Torino, July 2011

[2] “Common test conditions and software reference configurations”,

JCTVC-F900, Torino, July 2011

[3] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. or JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD Changes

##### 8.4.2.1.7	Derivation process for luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Output of this process is

- the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The motion vector predictor mvpLX is derived in the following ordered steps.

- The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause  is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A, B as the output.

- The derivation process for temporal luma motion vector prediction in subclause 4 is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

- The motion vector predictor candidate list, mvpListLX, is constructed as follows.

- mvLXA and distLXA, if availableFlagLXA is equal to 1

- mvLXB and distLXB, if availableFlagLXB is equal to 1

- mvLXCol and distLXCol, if availableFlagLXCol is equal to 1

- When motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

- When mvpListLX is empty, a zero motion vector is added as follows.

mvpListLC[ 0 ][0] = 0	(8-1)

mvpListLC[ 0 ][1] = 0	(8-2)

- The variable numMVPCandLX is set to the number of elements within the mvpListLX and maxNumMVPCand is set to 2.

- The motion vector predictor list is modifed to contain exactly maxNumMVPCand motion vector predictor candidates as follows.

    - If numMVPCandLX is less than maxNumMVPCand, the derivation process for zero motion vector predictor candidates specified in subclause Error: Reference source not found is invoked with mvpListLX and numMVPCandLX given as input and the output is assigned to mvpListLX and numMVPCandLX.

    - Otherwise (numMVPCandLX is equal to or greater than maxNumMVPCand), all motion vector predictor candidates mvpListLX[ idx ] with idx greater than maxNumMVPCand − 1 are removed from the list.

- The derivation process for scaled motion vector predictor mvpLX in subclause 8.4.2.1.x is invoked with orgMvLX and distLX. The motion vector of mvpListLX[ mvp_idx_lX[ xP, yP ] ] is assigned to the original motion vector origMvLX and the temporal picture distance of mvpListLX[ mvp_idx_lX[ xP, yP ] ] is assigned to distLX.

##### 8.4.2.1.8	Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A, or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the picture dictance distLXN,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-3 – Spatial motion vector neighbours

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

The variable isScaledFlagLX with X being 0 or 1 is set equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0, 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], distLXA is set equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX[ xAk ][ yAk ], LX ).

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ],  distLXA is set equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY).

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], distLXA is set equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX[ xAk ][ yAk ], LX ).

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], distLXA is set equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY).

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], distLXB is set equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX[ xBk ][ yBk ], LX ).

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic , refIdxLY[ xBk ][ yBk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ),  availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], distLXB is set equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLY[ xBk ][ yBk ], LY ).

- When availableFlagLXA or availableFlagLXB is equal to 0 and isScaledFlagLX is equal to 0, for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], distLXB is set equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX[ xBk ][ yBk ], LX ).

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], distLXB is set equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLY[ xBk ][ yBk ], LY ).

##### 8.4.2.1.9	Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the temporal picture distance distLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8-3)

yPRb = yP + nPSH	(8-4)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1	(8-5)

yPCtr = ( yP + ( nPSH >> 1 ) – 1	(8-6)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol, distLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvCol and refIdxCol are derived as follows,

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

      - If all PicOrderCnts of pictures in all reference picture lists are smaller than PicOrderCnt( currPic ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of RefIdxLX.

      - Otherwise (one of PicOrderCnt of pictures in all reference picture lists are larger than PicOrderCnt( currPic ) ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of collocated_from_l0_flag.

and merge_flag is set equal to 0 and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - distLXCol is set equal to PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol )	(8-xxx)

and merge_flag is set equal to 1 and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol	(8-7)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-8)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-9)

mvLXCol = ClipMv( Sign( DistScaleFactor *  mvLXCol ) * 			
				( (Abs( DistScaleFactor *  mvLXCol ) + 127 ) >> 8 ) )	(8-10)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )	(8-11)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-12)

##### 8.4.2.1.x	Derivation process for scaled motion vector

The prediction mvpLX is derived as specified below.

  - If distLX is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvpLX = origMvLX	(8-13)

  - Otherwise, mvLX is derived as scaled version of the motion vector mvLX as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-14)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-15)

mvpLX = ClipMv( Sign( DistScaleFactor * origMvLX ) * 			
				( (Abs( DistScaleFactor * origMvLX ) + 127 ) >> 8 ) )	(8-16)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, distLX )	(8-17)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-18)