HEVC Working Draft modifications for JCTVC-G220 based on JCTVC-F747.

JCTVC-F747 was adopted in Torino meeting (July, 2011) but not written in WD yet.

The modifications are highlighted in yellow .

7.3.2.3 Adaptation parameter set RBSP syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_sample_adaptive_offset_flag | u(1) |
| aps_adaptive_loop_filter_flag | u(1) |
| if( aps_sample_adaptive_offset_flag ¦¦ aps_adaptive_loop_filter_flag ) { |  |
| aps_cabac_use_flag | u(1) |
| if( aps_cabac_use_flag ) { |  |
| aps_cabac_init_idc | ue(v) |
| aps_cabac_init_qp_minus26 | se(v) |
| } |  |
| } |  |
| if( aps_sample_adaptive_offset_flag ) { |  |
| sao_data_byte_count /* to enable skipping past data without parsing it */ | u(12) |
| byte_align() |  |
| sao_param() |  |
| if( aps_adaptive_loop_filter_flag) { |  |
| byte_align() |  |
| } |  |
| } |  |
| if( aps_adaptive_loop_filter_flag ) { |  |
| alf_data_byte_count /* to enable skipping past data without parsing it */ | u(8) |
| byte_align() |  |
| alf_param( ) |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

7.4.2.3 Adaptation parameter set semantics

aps_id identifies the adaptation parameter set that is referred to in the slice header. The value of aps_id shall be in the range of 0 to [TBD, perhaps level/profile dependent], inclusive.

aps_sample_adaptive_offset_flag equal to 1 specifies that the SAO is on for slices referred to the current APS; equal to 0 specifies that the SAO is off for slices referred to the current APS.  If there is no active APS, the aps_sample_adaptive_offset_flag value is inferred to be 0.

aps_adaptive_loop_filter_flag equal to 1 specifies that the ALF is on for slices referred to the current APS; equal to 0 specifies that the ALF is off for slices referred to the current APS.  If there is no active APS, the aps_adaptive_loop_filter_flag value is inferred to be 0.

aps_cabac_use_flag equal to 1 specifies that the CABAC decoding process shall be used for sao_param( ) when present, and alf_param( ) when present; equal to 0 specifies that the CAVLC decoding process shall be used for sao_param( ) when present, and alf_param( ) when present.

aps_cabac_init_idc specifies the index for determining the initialisation table used in the initialisation process for context variables of SAO and ALF. The value of cabac_init_idc shall be in the range of 0 to 2, inclusive.

aps_cabac_init_qp_minus26 specifies a quantization parameter minus 26 wherein the quantization parameter is used in the initialization process for context variables of SAO and ALF.

alf_data_byte_count specifies the number of bytes

sao_data_byte_point specifies the number of bytes