| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G220 |
| --- | --- |

| Title: | Non-CE8: Pure VLC for SAO and ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Yang Tsai, Chih-Ming Fu, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chiayang.tsai, chihming.fu, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-4.0-dev-miscs, SAO and ALF parameters in APS and CU-level ALF-on/off flags in slice header can be coded by CABAC. No other syntax elements in APS and slice header can be coded by CABAC. In this contribution, it is proposed to use pure VLC for SAO and ALF and to remove byte alignment bits for CABAC in APS. Simulation results reportedly show 0%, 0.1%, 0.2% and 0.2% coding efficiency gains for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively, when the APS coding is changed from CABAC to VLC. Simulation results also show no coding efficiency impact when the slice header coding is changed from CABAC to VLC.

# Introduction

In High-Efficiency Video Coding test model version 4.0-dev-miscs (HM-4.0-dev-miscs), sample adaptive offset (SAO) and adaptive loop filter (ALF) parameters are coded in adaptation parameter set (APS) [1]. The APS syntax is shown in Table 1. In Table 1, sao_param() and alf_param() contain parameters of SAO and ALF, respectively. They can be represented by CABAC or VLC according to aps_cabac_use_flag. To initialize CABAC context probabilities, aps_cabac_init_idc and aps_cabac_init_qp_minus26 are coded. Moreover, byte_align() is used to insert bits until byte alignment is reached. The purposes of byte alignment are to switch the entropy coder from VLC to CABAC and to enable skipping parameters without parsing it when CABAC is used.

In addition to parameters coded in APS, ALF also allows coding unit (CU) control flags coded in slice header to enable or disable ALF for each CU. The CU control flags can also be represented by CABAC or VLC according to the entropy coder selection flag in picture parameter set (PPS).

Although CABAC can generally have better coding efficiency than variable length coding (VLC), the coding gain may not be significant for picture-level parameters and on/off flags of equal probability. In addition, CABAC needs to initialize context probabilities and is more difficult to parse. Therefore, in APS and slice header, since the amount of information is not much, it is proposed to use pure VLC without CABAC for SAO and ALF. With pure VLC, byte alignment bits in APS can also be removed. In this way, the entire APS and slice header do not need CABAC.

The rest of this contribution is organized as follows. The proposed syntax will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

Table 1.  Current APS syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_sample_adaptive_offset_flag | u(1) |
| aps_adaptive_loop_filter_flag | u(1) |
| if( aps_sample_adaptive_offset_flag ¦¦ aps_adaptive_loop_filter_flag ) { |  |
| aps_cabac_use_flag | u(1) |
| if( aps_cabac_use_flag ) { |  |
| aps_cabac_init_idc | ue(v) |
| aps_cabac_init_qp_minus26 | se(v) |
| } |  |
| } |  |
| if( aps_sample_adaptive_offset_flag ) { |  |
| sao_data_byte_count /* to enable skipping past data without parsing it */ | u(12) |
| byte_align() |  |
| sao_param() |  |
| if( aps_adaptive_loop_filter_flag) { |  |
| byte_align() |  |
| } |  |
| } |  |
| if( aps_adaptive_loop_filter_flag ) { |  |
| alf_data_byte_count /* to enable skipping past data without parsing it */ | u(8) |
| byte_align() |  |
| alf_param( ) |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

# Proposed method

As show in Table 1, in order to initialize CABAC context probabilities, two flags are added. Besides, in order to deal with entropy coder switching between CABAC and VLC, byte alignment bits are inserted. These may suffer coding efficiency loss when CABAC is used. Therefore, it is desirable to use pure VLC and to remove byte alignment bits in APS. The proposed APS syntax is shown in Table 2.

The CU-level ALF-on/off control parameters are in slice header and can be coded by CABAC or VLC according to the entropy coder selection flag in PPS. However, if CABAC is used, both VLC and CABAC entropy coders have to be activated to parse the slice header. Therefore, we propose to use pure VLC for CU-level ALF-on/off control parameters, and thus only VLC is used for slice header. There is no syntax change in slice header.

Table 2.  Proposed APS syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_sample_adaptive_offset_flag | u(1) |
| aps_adaptive_loop_filter_flag | u(1) |
| if( aps_sample_adaptive_offset_flag ) { |  |
| sao_param() |  |
| } |  |
| if( aps_adaptive_loop_filter_flag ) { |  |
| alf_param( ) |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

# Simulation results

The software is based on HM-4.0-dev-miscs (rev. 1423). High efficiency all intra (HE-AI), high efficiency random access (HE-RA), high efficiency low delay B (HE-LDB), and high efficiency low delay P (HE-LDP) common test conditions are tested according to JCTVC-F900 [2].

The first experiment is to test pure VLC for APS. The proposed APS syntax in Table 2 is used. The results are shown in Table 3. The anchor is HM-4.0-dev-miscs (Rev. 1423). It can be seen that 0%, 0.1%, 0.2%, and 0.2% bit rate savings are acheived for HE-AI, HE-RA, HE-LDB, and HE-LDP, respectively.

The second experiment is regarding to test pure VLC for slice header. The CU-level ALF-on/off control parameters are all coded in VLC. There is no syntax modification for this part. The results are shown in Table 4. The anchor is HM-4.0-dev-miscs (Rev. 1423). No impact on coding efficiency is observed.

# Conclusions

MediaTek proposed to use pure VLC for SAO and ALF parameters. In this way, CABAC parsing is not required for APS and slice header. It is reported that 0-0.2% bit rate reduction and no bit rate change are observed when APS coding and slice header coding are changed from CABAC to VLC, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S. Wenger, J. Boyce, Y.-W. Huang, C.-Y. Tsai, P. Wu, and M. Li, “Adaptation parameter set (APS),” Document of Joint Collaborative Team on Video Coding, JCTVC-F747, July 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.

Table 3.  Results of using pure VLC for APS

Table 4.  Results of using pure VLC for slice header