| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G221 |
| --- | --- |

| Title: | Non-CE13: The maximum number of merging candidates in P-slice |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Shigeru Fukushima Hideki Takehara Motoharu Ueda JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@ jvckenwood.com fukushima.shigeru@ jvckenwood.com takehara.hideki@ jvckenwood.com motoharu.ueda@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In the HM 4.0, the maximum number of merging candidates is fixed to 5 as maxNumMergeCand in both P-slice and B-slice.

This contribution reports the results of investigation of the variable maxNumMergeCand in low delay P settings. The simulation results report that 0.1% BD-rate gain is obtained under the setting of maxNumMergeCand equal to 3.

# Introduction

In the current HEVC scheme, B-slice is used instead of P-slice under common test conditions. However, P-slice is still needed as coding structure for small amount of operation and memory access.

In the HM 4.0 [1], the maximum number of merging candidates is fixed to 5 as maxNumMergeCand, and maxNumMergeCand is set to the same value in both P-slice and B-slice as the result of experiments in case of B-slice structure. However, maxNumMergeCand in the HM 4.0 is too large in case of P-slice structure because the number of effective merging candidates in P-slice is smaller than that of in B-slice. Therefore, maxNumMergeCand in case of P-slice can be set smaller than B-slice.

This contribution reports the results of investigation of the variable maxNumMergeCand in low delay P settings and recommends applying best number of maxNumMergeCand for P-slice.

# Algorithm Description

## Current setting (HM4.0)

In the HM 4.0, maxNumMergeCand is fixed to 5 in both P-slice and B-slice.

## Proposed setting

This contribution recommends that maxNumMergeCand is set different value from current maxNumMergeCand in P-slice.

Truncated unary binarization is applied for merge_idx. The truncated unary binarization tables for merge_idx in case of maxNumMergeCand equals to 2, 3, 4 and 5 are presented in Table1.

Table 1 Truncated unary binarization table for merge_idx

| merge_idx | maxNumMergeCand |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2 | 3 | 4 | 5 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 10 | 10 | 10 |
| 2 | - | 11 | 110 | 110 |
| 3 | - | - | 111 | 1110 |
| 4 | - | - | - | 1111 |

## Implementation

### Software

The software provided by TI in CE13 [2] activity (Section3.2.1) is used for these experiments.

In this software, maxNumMergeCand is signaled in slice header and the truncated unary binarization tables used for merge_idx coding depend on the value of maxNumMergeCand as presented in Table1.

# Experiments

## Test conditions

The common test conditions [3] are used and low delay P settings (LP-HE, LP-LC) are simulated.

## Experimental environment

The experimental environment used is presented in Table 2.

Table 2 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 3, 4 and 5 present the simulation results of maxNumMergeCand equal to 4, 3 and 2 for low delay P, respectively.

Table 3 Simulation results of maxNumMergeCand equal to 4 for low delay P

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | -0.2% |
| Class E | -0.1% | 0.1% | -0.3% | 0.0% | -0.4% | -0.4% |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | -0.1% | -0.3% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

Table 4 Simulation results of maxNumMergeCand equal to 3 for low delay P

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | 0.1% | -0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.4% | -0.2% | -0.1% | 0.0% | -0.4% |
| Class E | -0.2% | -0.5% | -0.1% | -0.2% | -0.7% | -0.8% |
| Overall | -0.1% | -0.3% | 0.0% | -0.1% | -0.1% | -0.3% |
|  | -0.1% | -0.3% | -0.1% | -0.1% | -0.1% | -0.3% |
| Enc Time[%] | 96% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 5 Simulation results of maxNumMergeCand equal to 2 for low delay P

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.3% | 0.4% | 0.0% | -0.2% | -0.3% |
| Class C | 0.2% | 0.5% | 0.5% | 0.1% | 0.1% | 0.4% |
| Class D | 0.1% | 0.0% | 0.2% | 0.0% | -0.5% | -0.2% |
| Class E | 0.3% | -0.3% | -0.3% | -0.3% | -1.9% | -1.7% |
| Overall | 0.2% | 0.1% | 0.2% | 0.0% | -0.5% | -0.4% |
|  | 0.2% | 0.1% | 0.2% | 0.0% | -0.5% | -0.4% |
| Enc Time[%] | 92% |  |  | 92% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

The simulation results report that the setting which maxNumMergeCand is equal to 4 provides no gain and loss and the encoding time is 98% for low delay P.

The simulation results report that the setting which maxNumMergeCand is equal to 3 provides 0.1% BD-rate [4] gain and the encoding time is 96% for low delay P.

The simulation results report that the setting which maxNumMergeCand is equal to 2 provides 0.2% BD-rate loss (high efficiency condition) and no gain and loss (low complexity condition) and the encoding time is 92% for low delay P.

(See the attached excel sheet)

# Conclusions

We recommend as follows:

- maxNumMergeCand in P-slice should be set as a different value from maxNumMergeCand in B-slice

- maxNumMergeCand is set equal to 3 in P-slice

- maxNumMergeCand is set equal to 3 as common condition of LP-HE and LP-LC

# References

[1] “WD4: Working Draft 4 of High-Efficiency Video Coding”,

JCTVC-F803, Torino, July 2011

[2] “Description of Core Experiment CE13: Motion data parsing robustness and throughput”,

JCTVC-F913, Torino, July 2011

[3] “Common test conditions and software reference configurations”,

JCTVC-F900, Torino, July 2011

[4] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. or JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).