| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G222 |
| --- | --- |

| Title: | Non-CE8: Offset coding in SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com Marta Karczewicz +1-858-658-5673 {ichong, martak}@qualcomm.com |
| Source: | MediaTek Inc., Qualcomm Inc. |  |  |

_____________________________

# Abstract

The offset coding in SAO of HM-4.0 does not fit the offset distribution. In this contribution, a new codeword design can be used to better fit the offset distribution, or an offset prediction technique can be used to reduce offset information. Simulation results reportedly show that the luma bit rate and the chroma bit rate are reduced by 0-0.1%, and 0.1-0.5%, respectively, with unchanged run time.

# Introduction

In High Efficiency Video Coding test model version 4.0 (HM-4.0), sample adaptive offset (SAO) is applied to luma and chroma, and each component is independently processed. SAO can divide one picture into multiple LCU-aligned regions, and each region can select one SAO type among two band offset (BO) types, four edge offset (EO) types, and no processing (OFF). For each to-be-processed pixel, BO uses the pixel intensity to classify the pixel into a band. The pixel intensity range is equally divided into 32 bands, as shown in Figure 1. After pixel classification, one offset is derived for all pixels of each band, and the offsets of center 16 bands or outer 16 bands are selected and coded. As for EO, it uses two neighboring pixels of a to-be-processed pixel to classify the pixel into a category. Similarly, one offset is derived for all pixels of each category after pixel classification except for category 0, which is forced to use zero offset. Table 1 shows the EO pixel classification, where “c” denotes the pixel to be classified. Figure 2 shows four EO types with different selections of neighboring pixels.

Figure 1.  Pixel intensity range equally divided into 32 bands by BO

Table 1.  EO pixel classification

Figure 2.  Four EO types

In CAVLC, offsets are represented by signed exp-Golomb codes. In CABAC, singed offsets are mapped to unsigned indices for generating unary codes, where the mapping is the same as that used in signed exp-Golomb coding to convert a signed value to an unsigned index as an input of exp-Golomb codeword table. In both CABLC and CABAC, a zero offset has the shortest codeword, and the codeword length gets longer when the offset magnitude increases. However, the current codewords in HM-4.0 may not fit the offset distribution well. In this contribution, a new offset coding method is proposed. The rest of this contribution is organized as follows. The proposed offset coding in SAO will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed offset coding

The proposed offset coding has two parts. In the first part, we design new codewords to better fit the offset distribution. In the second part, we use a simple prediction technique to reduce offset information.

## Offset codeword design

The offset coding in BO is not changed in our design. Figure 2 and Figure 3 show the offset distribution of 10-bit pixels and that of 8-bit pixels, respectively, for different categories in EO. It can be seen that zero offset is not always most selected, and the current offset codeword design in EO of HM-4.0 is not proper. Fortunately, the real offset distribution of each category can be regarded as a shift of the distribution for signed exp-Golomb coding, and the proposed codeword design on the encoder side is as follows:

If ( ( offset – b ) < 0 ), index = – ( ( offset – b ) << 1 ) – a

Else, index = ( ( offset – b ) << 1 ) - 1 + a

The index is used as the input of exp-Golomb code table for CAVLC or as the input of unary code table for CABAC. The parameters, a and b, are shown in Table 2 for different EO categories, where b denotes the offset value of the highest probability and a controls the priorities of two offset values of the same magnitude.

Figure 2.  Offset distribution of 10-bit pixels for different categories in EO

Figure 3.  Offset distribution of 8-bit pixels for different categories in EO

Table 2.  Parameters of the codeword design

|  | 10-bit pixels | 8-bit pixels |
| --- | --- | --- |
| EO category 1 | b = 2, a = 1 | b = 1, a = 1 |
| EO category 2 | b = 1, a = 1 | b = 1, a = 0 |
| EO category 3 | b = -1, a = 0 | b = -1, a = 1 |
| EO category 4 | b = -2, a = 0 | b = -1, a = 0 |

## Offset prediction

To predict the offset value, we use the neighboring region information. If the current region and the left region in the same depth select EO, category 1 and category 2 of the current region will be predicted by corresponding offsets of the left region, and in the current region the category 3 and category 4 will be predicted by category 2 and category 1 offset magnitudes of opposite signs. If the current region applies BO, the offset of the second band will be predicted by that of the first band, and the offset of the 16th band will be predicted by that of the 15th band.

# Simulation results

The software is implemented on top of HM-4.0. High efficiency all intra (HE-AI), high efficiency random access (HE-RA), high efficiency low delay B (HE-LDB), low complexity all intra (LC-AI), low complexity random access (LC-RA), and low complexity low delay B (LC-LDB) common test conditions defined in JCTVC-F900 [1] were tested. Table 3 shows that the proposed method results in 0-0.1% luma bit rate reduction and 0.1-0.5% chroma bit rate reduction. The encoding time and the decoding time are unchanged on average.

# Conclusions

MediaTek and Qualcomm modified the offset coding in SAO. It is reported that the luma bit rate and the chroma bit rate are reduced by 0-0.1%, and 0.1-0.5%, respectively, with the same run time on average.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.

Table 3.  Results of the proposed offset coding in SAO