| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G223 |
| --- | --- |

| Title: | Non-CE9: Division-free MV scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Jian-Liang Lin, Ching-Yeh Chen, Yi-Wen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, jl.lin, chingyeh.chen, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HM-4.0 motion vector (MV) scaling for the derivation of spatial and temporal motion vector predictors (MVPs), a division operation is required to derive the scaling factor. In hardware and many DSP-based platforms, dividers are undesirable because of larger gate counts and more processing cycles. In this contribution, a division-free MV scaling is proposed to replace the general divider by a look-up table and simple arithmetic operations. Moreover, the effective scaling range is doubled to deal with reference pictures of longer temporal distances. Simulation results reportedly show that the proposed method has no bit rate increase in random access conditions and even 0.1% bit rate reduction in low delay conditions. The proposed design is also implemented in Verilog and synthesized with TSMC 40nm process. The synthesis results reportedly show that the gate count of the MV scaling module is reduced by 54-58%.

# Introduction

In the motion vector predictor (MVP) derivation of HM-4.0, when a MVP is derived from a MV pointing to a different reference picture, the MV is scaled to the target reference picture as the final MVP. In the MV scaling process, the scaling factor is defined by equ. (1).

ScalingFactor = ( POCcurr – PoCref )/( POCcol – POCcol_ref ) = tb/td,    	(1)

where td is the POC distance between the co-located picture and the reference picture for the co-located block, and tb is the POC distance between the current picture and the target reference picture. In HM-4.0, the scaling factor is calculated by the following equations:

X = ( 214 + | td/2 | ) / td  	(2)

ScalingFactor = clip ( -1024, 1023, ( tb × X + 32 ) >> 6 )	(3)

Then, the scaled MV is derived as follow:

ScaledMV = sign ( ScalingFactor × MV ) × ( ( abs ( ScalingFactor × MV )+127 ) ) >> 8 )	(4)

In equ. (2), a divider is required to derive the value of X. However, in hardware, the divider is undesirable due to the high area cost. Moreover, in HM-4.0, the MV scaling is the only thing that still needs the division operation. All other coding tools are division-free. In this contribution, to reduce the hardware complexity, a division-free MV scaling is proposed to replace the divider with a look-up table (LUT) and simple arithmetic operations. Besides, the range of scaling factor is doubled for the large tb in low delay conditions. The rest of this contribution is organized as follows. The algorithm will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Algorithm description

## MV scaling in HM-4.0

The MV scaling process in HM-4.0 is specified in equ. (2-4). Figure 1 shows the circuit diagram of HM-4.0 MV scaling hardware. In the divider, a 16-bit input is divided by an 8-bit input. The divider occupies over half of gate counts in the MV scaling module.

Figure 1.  The circuit diagram of HM-4.0 MV scaling hardware

In HM-4.0, the ScalingFactor is clipped in the range of [-1024, 1023], as shown in equ. (3). The ScalingFactor will be right shifted by eight bits as shown in equ. (4), which means the effective scaling range is limited in [-4, 4). In the 6th JCT-VC meeting in Torino, a reference picture selection method was adopted in JCTVC-F701 [1]. One nearest picture and three high quality pictures (pictures with lowest QPs) are used as reference pictures for low delay cases. In the common test conditions, the temporal distance between two high quality pictures is 4. The effective scaling range of [-4, 4) is not enough in low delay cases. In the simulation results reported in Section 3, enlarging the effective scaling range from [-4, 4) to [-8, 8) can contribute 0.1% coding efficiency gain in low delay cases.

## Proposed division-free MV scaling

In the proposed method, the divider is replaced by a 15-entry 8-bit LUT with simple arithmetic operations. The scaling factor is derived as follows:

If ( abs( td ) < 16 )

X = ( td >= 0 ) ? LUT_div_x[ abs( td ) ] : –LUT_div_x[ abs( td ) ]

Else

X = ( td >= 0 ) ? LUT_div_x[ abs( td ) >> ( n–1 ) ] : –LUT_div_x[ abs( td ) >> ( n–1 ) ]  	(5)

ScalingFactor = clip ( -1024, 1023, ( tb × X ) >> n ),	(6)

where the LUT_div_x is specified in Table 1, and the n represents the position of the MSB of abs( td ) minus 3 when abs( td ) is larger than 8. The n is specified in Table 2.

Then, the scaled MV is derived as follow:

ScaledMV = Sign (ScalingFactor × MV ) × ( ( abs( ScalingFactor × MV )+63 ) ) >> 7 )	(7)

As described in Section 2.1, the effective scaling range is enlarged to [-8, 8) in the proposed method. This can be verified by equ. (6) and equ. (7). The ScalingFactor is limited in the range of [-1024, 1023] in equ. (6), and then the ScalingFactor is right shifted by 7 in equ. (7).

Table 1.  Specification of LUT_div_x according to abs(td)

| abs(td) | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LUT_div_x[abs(td)] | 128 | 64 | 43 | 32 | 26 | 21 | 18 | 32 | 28 | 26 | 23 | 21 | 20 | 18 | 17 |

Table 2.  Specification of n according to abs(td)

| abs(td) | 1-7 | 8-15 | 16-31 | 32-63 | 64-127 |
| --- | --- | --- | --- | --- | --- |
| n | 0 | 1 | 2 | 3 | 4 |

Figure 2 shows the circuit diagram of the proposed MV scaling hardware. The divider is replaced by a LUT and simple arithmetic operations, such as Find MSB and shifter. Since n represents the position of the MSB of abs(td) minus 3, it can be implemented as several bits of comparators with if/else conditions. There is no need to store the Table 2 in hardware implementations. Only Table 1 needs to be stored.

Figure 2.  The circuit diagram of the proposed MV scaling hardware

# Simulation results

## Results in common test conditions

The proposed method is implemented on HM-4.0. All results were simulated according to the common test conditions defined in JCTVC-F900 [2]. The tested conditions are high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay B-frame (HE-LDB), low complexity low delay B-frame (LC-LDB), high efficiency low delay P-frame (HE-LDP), and low complexity low delay P-frame (LC-LDP).

Table 3 shows the results of the HM-4.0 with enlarged effective scaling range as described in Section 2.1. The effective scaling range is increased from [-4, 4) to [-8, 8), and the bit rate is reduced by 0.1% in low delay cases.

Table 4 shows the results of the proposed division-free MV scaling. The coding efficiency is almost no change in HE-RA and LC-RA. The bit rate is reduced by 0.1% in low delay cases. The 0.1% gain comes from enlarging the effective scaling range. Compared with the HM-4.0 with the enlarged effective scaling range in Table 3, no bit rate increase is observed. The proposed method can also achieve less run time due to replacing the complex division with LUT and other simple arithmetic operation.

Table 3.  Results of HM-4.0 with effective scaling range enlarged to [-8, 8)

Table 4.  Results of the division-free MV scaling

## Results in conditions with doubled GOP sizes

In order to test conditions with larger tb and td, the GOP sizes are doubled. For random access conditions, the GOP size is increased from eight to 16. For low delay conditions, the temporal distance of two high quality pictures and the RateGOPSize are increased from four to eight. Table 5 shows the results of the proposed division-free MV scaling in conditions with doubled GOP sizes. The anchor is HM-4.0 without enlarged effective scaling range with doubled GOP sizes. The bit rate in random access cases is not changed, and the bit rate is reduced by 0.2-0.3% in low delay cases. The coding gain comes from enlarged the effective scaling range. Since the complex division is avoided, the run time is reduced.

Table 5.  Results of the division-free MV scaling in conditions with doubled GOP sizes

## Hardware synthesis results

The MV scaling method in HM-4.0 and the proposed non-division MV scaling are implemented with Verilog HDL and synthesized TSMC 40 nm process library. The synthesizer is Synopsis Design Compiler ®. Two target frequencies, 200 MHz and 300 MHz, are evaluated. Table 6 and Table 7 show the synthesis results. The proposed division-free MV scaling reduces 58% and 54% of gate counts when target frequency is in 200 MHz and 300 MHz, respectively. Note that, in 300 MHz, the HM-4.0 MV scaling circuit cannot be synthesized within 3.3 ns (300 MHz), and one pipeline is required. The latency for HM-4.0 MV scaling in 300 MHz is one cycle, while the latency of proposed method is 0 cycle.

Table 6.  Synthesis results of the HM-4.0 MV scaling and the proposed MV scaling at 200 HMz

| 200 MHz | Gate counts | Reduction | Latency |
| --- | --- | --- | --- |
| HM-4.0 | 7,331 | – | 0 cycle |
| Proposed | 3,093 | 58% | 0 cycle |

Table 7.  Synthesis results of the HM-4.0 MV scaling and the proposed MV scaling at 300 HMz

| 300 MHz | Gate counts | Reduction | Latency |
| --- | --- | --- | --- |
| HM-4.0 (pipelined) | 7,250 | – | 1 cycle |
| Proposed | 3,349 | 54% | 0 cycle |

# Conclusions

MediaTek proposed a division-free MV scaling that replaces the division operation in MV scaling by a look-up table and simple arithmetic operations in order to reduce hardware complexity. Simulation results reportedly show that the proposed method has no impact on coding efficiency in random access conditions and has 0.1% bit rate reduction in low delay conditions. The HM-4.0 method and the proposed method are implemented with Verilog HDL and synthesized with TSMC 40nm process. The synthesis results reportedly show that 54-58% gate count is reduced by the proposed method.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Li, J. Xu, F. Wu, H. Li, “Encoding optimization to improve coding efficiency for low delay cases,” Document of Joint Collaborative Team on Video Coding, JCTVC-F701, July 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.