| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G224 |
| --- | --- |

| Title: | Non-CE13: Multiple-scaled merging candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@ jvckenwood.com fukushima.shigeru@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

This proposal is for the improvement of the merge mode. In the HM4.0, for the merge mode, some spatial merging candidates and a temporal merging candidate are added into a merging candidate list. After that, if needed, combined bi-predictive merging candidates, a non-scaled bi-predictive merging candidate and zero motion vector merging candidates are added. The non-scaled bi-predictive merging candidate is added under a random access condition. Therefore, the multiple-scaled merging candidate by extending the non-scaled merging candidate for low delay conditions is proposed in this contribution.
The simulation results report that the proposed techniques provide no gain for random access setting, 0.1-0.2% BD-rate gain for low delay B setting, and 0.1-0.2% BD-rate gain for low delay P setting.

# Introduction

This proposal is for the improvement of coding efficiency of the merge mode. In the HM4.0, for the merge mode, some spatial merging candidates and a temporal merging candidate are added into a merging candidate list at first. After that, if needed, combined bi-predictive merging candidates, a non-scaled bi-predictive merging candidate and zero motion vector merging candidates are added into the list. The non-scaled bi-predictive merging candidate is added under a random access condition. This contribution proposes multiple-scaled merge candidate by extending the non-scaled merging candidate for low delay conditions.

# Algorithm Description

## Current algorithm (HM4.0)

HM 4.0 has the following merging candidates:

- spatial merging candidates

- a temporal merging candidate

If needed, HM4.0 also has the following merging candidates:

- combined bi-predictive merging candidates

- a non-scaled bi-predictive merging candidate

- zero motion vector merging candidates

The non-scaled bi-predictive merging candidate is added into a merging candidate list under a random access condition.

## Proposed algorithm

Multiple-scaled merging candidates are proposed in this proposal. The candidates are made by scaling operation without division operator.

When the slice type is B-slice, multiple-scaled or non-scaled bi-predictive merging candidate is derived as shown in Figure 1 and Figure 2.

Figure 1 Multiple-scaled bi-predictive merging candidate

Figure 2 Non-scaled bi-predictive merging candidate

When the slice type is P-slice, multiple-scaled uni-predictive merging candidate is derived as shown in Figure 3.

Figure 3 Multiple-scaled uni-predictive merging candidate

### Propose 1: multiple-scaled bi-predictive merging candidate for B-slice

If (PicOrderCnt − RefPicOrderCnt( refIdxLXorigCand, LX) = = 1, multiple-scaled bi-predictive merging candidate is derived and the following applies.

- refIdxLYtmpCand is checked from 0 to num_ref_idx_lY_active_minus1.
RefPicOrderCnt( refIdxLXorigCand, LX ) != RefPicOrderCnt( refIdxLYtmpCand, LY ) 
refIdxLXscaleCandl = refIdxLXorigCand
refIdxLYscaleCandl = refIdxLYtmpCand

- ScaleFactor = PicOrderCnt − RefPicOrderCnt( refIdxLYscaleCandl, LY )
mvLXscaleCandl[ 0 ] = mvLXorigCand[ 0 ]
mvLXscaleCandl[ 1 ] = mvLXorigCand[ 1 ]
mvLYscaleCandl[ 0 ] = ScaleFactor * mvLXorigCand[ 0 ]
mvLYscaleCandl[ 1 ] = ScaleFactor * mvLXorigCand[ 1 ]

Otherwise, if (PicOrderCnt − RefPicOrderCnt( refIdxLXorigCand, LX) != 1) and ( abs( PicOrderCnt − RefPicOrderCnt( refIdxLXorigCand, LX ) ) = = 
abs( PicOrderCnt − RefPicOrderCnt( refIdxLXorigCand, LY ) )  ), non-scaled bi-predictive merging candidate are derived and the following applies.

- refIdxLXscaleCandl = refIdxLXorigCand
refIdxLYscaleCandl = refIdxLXorigCand

- mvLXscaleCandl[ 0 ] = mvLXorigCand[ 0 ]
mvLXscaleCandl[ 1 ] = mvLXorigCand[ 1 ]
mvLYscaleCandl[ 0 ] = − mvLXorigCand[ 0 ]
mvLYscaleCandl[ 1 ] = − mvLXorigCand[ 1 ]

### Propose 2: multiple-scaled uni-predictive merging candidate for P-slice

If (PicOrderCnt − RefPicOrderCnt( refIdxL0origCand, L0) = = 1, multiple-scaled uni-predictive merging candidate is derived and the following applies.

- refIdxL0tmpCand is checked from 0 to num_ref_idx_l0_active_minus1.
RefPicOrderCnt( refIdxL0origCand, L0 ) != RefPicOrderCnt( refIdxL0tmpCand, L0 )
refIdxL0scaleCandl = (refIdxL0origCand = = 0) ? 1 : 0

- ScaleFactor = PicOrderCnt − RefPicOrderCnt( refIdxL0scaleCandl, L0 )
mvL0scaleCandl[ 0 ] = ScaleFactor * mvL0origCand[ 0 ]
mvL0scaleCandl[ 1 ] = ScaleFactor * mvL0origCand[ 1 ]

## Implementation

### Software

These proposed techniques are implemented into HM4.0 software.

The proposed software includes MrgEncFix which is encoding bug fix proposed by Samsung in CE13 activities.

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

### Simulation results compared with HM4.0 anchor

Table 2, Table 3 and Table 4 present the simulation results which are compared with HM4.0 anchor for random access, low delay B and low delay P, respectively.

Table 2 Propose 1: Simulation results for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.2% | -0.1% | 0.0% | 0.1% |
| Class B | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% |
|  | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3 Propose 1: Simulation results for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.6% | -0.6% | -0.3% | -0.5% | -0.4% |
| Class C | -0.3% | -0.5% | -0.6% | -0.2% | -0.2% | -0.5% |
| Class D | -0.3% | 0.0% | -0.3% | -0.2% | -0.2% | -0.5% |
| Class E | -0.5% | 0.1% | 0.3% | -0.3% | -0.1% | -0.4% |
| Overall | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.5% |
|  | -0.3% | -0.3% | -0.4% | -0.3% | -0.3% | -0.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 4 Propose 2: Simulation results for low delay P

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.5% | -0.2% | -0.2% | -0.3% | -0.6% |
| Class C | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.3% |
| Class D | -0.1% | 0.0% | -0.6% | -0.1% | -0.3% | -0.5% |
| Class E | -0.3% | -0.4% | -0.6% | -0.3% | -0.6% | -0.6% |
| Overall | -0.2% | -0.3% | -0.4% | -0.2% | -0.3% | -0.5% |
|  | -0.2% | -0.3% | -0.4% | -0.2% | -0.3% | -0.5% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

The simulation results report that the proposed techniques provide 0.1% BD-rate [3] gain for random access setting, 0.3% BD-rate gain for low delay B setting, and 0.2% BD-rate gain for low delay P setting. (See the attached excel sheet)

### Simulation results compared with HM4.0 anchor including MrgEncFix

Table 5, Table 6 and Table 7 present the simulation results which are compared with HM4.0 anchor including MrgEncFix for random access, low delay B and low delay P, respectively.

Table 5 Propose 1: Simulation results for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 6 Propose 1: Simulation results for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.4% | -0.2% | -0.4% | -0.5% |
| Class C | -0.1% | -0.4% | -0.4% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | 0.2% | -0.1% | -0.2% | -0.4% | -0.1% |
| Class E | 0.0% | 0.1% | -0.3% | -0.2% | -0.1% | -0.2% |
| Overall | -0.1% | -0.1% | -0.3% | -0.2% | -0.3% | -0.3% |
|  | -0.1% | -0.1% | -0.3% | -0.2% | -0.3% | -0.3% |

Table 7 Propose 2: Simulation results for low delay P

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.5% | -0.3% | -0.2% | -0.4% | -0.6% |
| Class C | 0.0% | -0.2% | -0.2% | -0.1% | -0.2% | -0.4% |
| Class D | -0.1% | 0.4% | -0.5% | -0.1% | -0.2% | -0.4% |
| Class E | -0.1% | -0.6% | 0.2% | -0.3% | -0.3% | 0.1% |
| Overall | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% | -0.4% |
|  | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% |

The simulation results report that the proposed techniques provide no gain for random access setting, 0.1-0.2% BD-rate gain for low delay B setting, and 0.1-0.2% BD-rate gain for low delay P setting. (See the attached excel sheet)

# Conclusions

We recommend that the proposed techniques be adopted in next WD and HM.

# References

[1] “WD4: Working Draft 4 of High-Efficiency Video Coding”,

JCTVC-F803, Torino, July 2011

[2] “Common test conditions and software reference configurations”,

JCTVC-F900, Torino, July 2011

[3] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. or JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD changes

##### 8.4.2.1.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-1)

predFlagLXCol = availableFlagLXCol	(8-2)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList and the variable maxNumMergeCand is set to 5.

- When slice_type is equal to B, the following applies.

  - The derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

  - The derivation process for multiple-scaled and non-scaled bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0nscaleCandl and refIdxL1nscaleCandl, the prediction list utilization flags predFlagL0nscaleCandl and predFlagL1nscaleCandl, the motion vectors mvL0nscaleCandl and mvL1nscaleCandl of every new candidate nscaleCandl being added in mergeCandList. The number of candidates being added numNscaleMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numNscaleMergeCand is greater than 0, l ranges from 0 to numNscaleMergeCand – 1, inclusive.

- When slice_type is equal to P, the following applies.

  - The derivation process for non-scaled uni-predictive merging candidates specified in subclause 8.4.2.1.x is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0nscaleCandl and refIdxL1nscaleCandl, the prediction list utilization flags predFlagL0nscaleCandl and predFlagL1nscaleCandl, the motion vectors mvL0nscaleCandl and mvL1nscaleCandl of every new candidate nscaleCandl being added in mergeCandList. The number of candidates being added numNscaleMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numNscaleMergeCand is greater than 0, l ranges from 0 to numNscaleMergeCand – 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, the NumMergeCand and the MaxNumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand – 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-3)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-4)

refIdxLX = refIdxLXN	(8-5)

predFlagLX = predFlagLXN	(8-6)

##### 8.4.2.1.5	Derivation process for multiple-scaled and non-scaled bi-predictive merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process,

- the maximum number of elements maxNumMergeCand within the mergeCandList.

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0scaleCandl and refIdxL10scaleCandl of every new candidate scaleCandl being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0scaleCandl and predFlagL10scaleCandl of every new candidate scaleCandl being added in mergeCandList during the invokation of this process,

- motion vectors mvL0scaleCandl and mvL10scaleCandl of every new candidate scaleCandl being added in mergeCandList during the invokation of this process,

The function RefPicOrderCnt( refidx, LX ) specifies the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of the current picture with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”. The variable PicOrderCnt specifies the PicOrderCnt of the current picture.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

When numOrigMergeCand is greater than 0 and numMergeCand is less than maxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables origIdx and scaleCnt are set to 0, the variable scaleStop is set to false and the following steps are repeated until scaleStop is equal to true.

- The following assignments are made with origCand being the candidate at position origIdx in the merging candidate list mergeCandList ( origCand = mergeCandList[ origIdx ] ).

- The following is applied one time with X being 0 and Y being 1. When scaleCnt is less than 1 and numMergeCand is less than maxNumMergeCand, it is applied a second time with X being 1 and Y being 0.

When all of the following conditions are true,

      - refIdxLXorigCand >= 0

      - ( PicOrderCnt − RefPicOrderCnt( refIdxLXorigCand, LX ) ) = = 1

      - refIdxLYtmpCand is checked from 0 to num_ref_idx_lY_active_minus1.
RefPicOrderCnt( refIdxLXorigCand, LX ) !=  RefPicOrderCnt( refIdxLYtmpCand, LY )

the following applies.

      - When no candidate in mergeCandList has a listX motion vector equal to listX motion vector of origCand, a listY motion vector equal to the listX motion vector of origCand multiplied by ScaleFactor and listX reference indices equal to the listX reference index of origCand and listY reference indices equal to the refIdxLYtmpCand, the candidate scaleCandl with l equal to ( numMergeCand − numInputMergeCand )is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = scaleCandl ) and the reference indices, the prediction list utilization flags and the motion vectors of scaleCandl are dervied as follows and numMergeCand is incremented by 1.

refIdxLXscaleCandl = refIdxLXorigCand		(8-7)

refIdxLYscaleCandl = refIdxLYtmpCand		(8-8)

predFlagL0scaleCandl = 1		(8-9)

predFlagL1scaleCandl = 1		(8-10)

ScaleFactor = PicOrderCnt − RefPicOrderCnt( refIdxLYscaleCandl, LY ) 		(8-xx)

mvLXscaleCandl[ 0 ] = mvLXorigCand[ 0 ] 		(8-11)

mvLXscaleCandl[ 1 ] = mvLXorigCand[ 1 ] 		(8-12)

mvLYscaleCandl[ 0 ] = ScaleFactor * mvLXorigCand[ 0 ] 		(8-13)

mvLYscaleCandl[ 1 ] = ScaleFactor * mvLXorigCand[ 1 ] 		(8-14)

numMergeCand = numMergeCand + 1	(8-15)

      - The variable scaleCnt is incremented by 1.

When all of the following conditions are true,

      - refIdxLXorigCand >= 0

      - ( PicOrderCnt − RefPicOrderCnt( refIdxLXorigCand, LX ) ) ! = 1

      - RefPicOrderCnt( refIdxLXorigCand, LX ) != RefPicOrderCnt( refIdxorigCandLX, LY )

      - abs( PicOrderCnt − RefPicOrderCnt( refIdxLXorigCand, LX ) ) = = 
abs( PicOrderCnt − RefPicOrderCnt( refIdxLXorigCand, LY ) )

      - refIdxLXorigCand < num_ref_idx_lY_active_minus1 + 1

the following applies.

      - When no candidate in mergeCandList has a listX motion vector equal to listX motion vector of origCand, a listY motion vector equal to the listX motion vector of origCand multiplied by 1 and list0 and list1 reference indices equal to the listX reference index of origCand, the candidate scaleCandl with l equal to ( numMergeCand − numInputMergeCand )is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = scaleCandl ) and the reference indices, the prediction list utilization flags and the motion vectors of scaleCandl are dervied as follows and numMergeCand is incremented by 1.

refIdxL0scaleCandl = refIdxLXorigCand		(8-16)

refIdxL1scaleCandl = refIdxLXorigCand		(8-17)

predFlagL0scaleCandl = 1		(8-18)

predFlagL1scaleCandl = 1		(8-19)

mvL0scaleCandl[ 0 ] = mvLXorigCand[ 0 ] 		(8-20)

mvL0scaleCandl[ 1 ] = mvLXorigCand[ 1 ] 		(8-21)

mvL1scaleCandl[ 0 ] = − mvLXorigCand[ 0 ] 		(8-22)

mvL1scaleCandl[ 1 ] = − mvLXorigCand[ 1 ] 		(8-23)

numMergeCand = numMergeCand + 1	(8-24)

      - The variable scaleCnt is incremented by 1.

- The variable origIdx is incremented by 1.

- When origIdx is equal to numOrigMergeCand or numMergeCand is equal to maxNumMergeCand or scaleCnt is equal to 1, scaleStop is set to true.

##### 8.4.2.1.x	Derivation process for multiple-scaled uni-predictive merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process,

- the maximum number of elements maxNumMergeCand within the mergeCandList.

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0scaleCandl and refIdxL10scaleCandl of every new candidate scaleCandl being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0scaleCandl and predFlagL10scaleCandl of every new candidate scaleCandl being added in mergeCandList during the invokation of this process,

- motion vectors mvL0scaleCandl and mvL10scaleCandl of every new candidate scaleCandl being added in mergeCandList during the invokation of this process,

The function RefPicOrderCnt( refidx, LX ) specifies the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of the current picture with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”. The variable PicOrderCnt specifies the PicOrderCnt of the current picture.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

When numOrigMergeCand is greater than 0 and numMergeCand is less than maxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables origIdx and scaleCnt are set to 0, the variable scaleStop is set to false and the following steps are repeated until scaleStop is equal to true.

- The following assignments are made with origCand being the candidate at position origIdx in the merging candidate list mergeCandList ( origCand = mergeCandList[ origIdx ] ).

- The following is applied.

    - When all of the following conditions are true,

      - refIdxL0origCand >= 0

      - num_ref_idx_l0_active_minus1 >= 1

      - ( PicOrderCnt − RefPicOrderCnt( refIdxL0origCand, L0 ) ) = = 1

the following applies.

      - When no candidate in mergeCandList has a list0 motion vector equal to the list0 motion vector of origCand multiplied by ScaleFactor and list0 reference indices equal to the list0 reference index of origCand, the candidate scaleCandl with l equal to ( numMergeCand − numInputMergeCand )is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = scaleCandl ) and the reference indices, the prediction list utilization flags and the motion vectors of scaleCandl are dervied as follows and numMergeCand is incremented by 1.

refIdxL0scaleCandl = (refIdxL0origCand = = 0) ? 1 : 0		(8-25)

refIdxL1scaleCandl = -1		(8-26)

predFlagL0scaleCandl = 1		(8-27)

predFlagL1scaleCandl = 0		(8-28)

ScaleFactor = PicOrderCnt − RefPicOrderCnt( refIdxL0scaleCandl, L0 ) 		(8-xx)

mvL0scaleCandl[ 0 ] = ScaleFactor * mvL0origCand[ 0 ] 		(8-29)

mvL0scaleCandl[ 1 ] = ScaleFactor * mvL0origCand[ 1 ] 		(8-30)

mvL1scaleCandl[ 0 ] = 0 		(8-31)

mvL1scaleCandl[ 1 ] = 0 		(8-32)

numMergeCand = numMergeCand + 1	(8-33)

      - The variable scaleCnt is incremented by 1.

- The variable origIdx is incremented by 1.

- When origIdx is equal to numOrigMergeCand or numMergeCand is equal to maxNumMergeCand or scaleCnt is equal to 1, scaleStop is set to true.