## Scanning order array initialisation process

Input to this process is a block widthsize blkWidth and a block height blkHeight.

Output of this process is the array ScanOrder[ scanIdx ][ pos ][ comp ]. The array index scanIdx equal to 0 specifies a zig-zag scan as specified in subclause Error: Reference source not found with blkWidth and blkHeight as inputs, scanIdx equal to 1 specifies a horizontal scan, scanIdx equal to 2 specifies a vertical scan, and scanIdx equal to 3 specifies an up-right diagonal scan in subclause Error: Reference source not found. The array index pos specifies the scan position ranging from 0 to ( blkWidth * blkHeight ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. The array ScanOrder is derived as follows.

ScanOrder[0] = ZigZag

i  =  0
y  =  0
if (blkWidth < 16) {
  while( y  < blkHeight ) {
	x  =  0
	while( x  <  blkWidth ) {
		ScanOrder[ 1 ][ i ][ 0 ]  = x
		ScanOrder[ 1 ][ i ][ 1 ]  = y
		x++
		i++
	}
	y++
  }
}
else if (blkWidth ==16) {
    ScanOrder[1] = ScanOrderH16
} 
else if(blkWidth ==32) {
    ScanOrder[1] = ScanOrderH32
}

i  =  0
while( i  < blkWidth * blkHeight) {
    ScanOrder[ 2 ][ i ][ 0 ] = ScanOrder[ 1] [ i ]>>log2(blkSize) 
    ScanOrder[ 2 ][ i ][ 1 ] = ScanOrder[ 1] [ i ]%log2(blkSize)
}

ScanOrder[3] = DiagScan

Table 7-1 – Specification of ScanType[ log2TrafoSize − 2 ][ IntraPredMode ]

| IntraPredMode | log2TrafoSize − 2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 2 | 2 | 2 | 2 |
| 2-3 | 0 | 0 | 0 | 0 |
| 4-5 | 1 | 1 | 0 | 0 |
| 6 | 0 | 0 | 0 | 0 |
| 7-8 | 2 | 2 | 0 | 0 |
| 9-10 | 0 | 0 | 0 | 0 |
| 11-12 | 1 | 1 | 0 | 0 |
| 13-14 | 0 | 0 | 0 | 0 |
| 15-16 | 2 | 2 | 0 | 0 |
| 17-19 | 0 | 0 | 0 | 0 |
| 20 | 1 | 1 | 0 | 0 |
| 21-22 | 1 | 1 | 1 | 1 |
| 23 | 1 | 1 | 0 | 0 |
| 24-27 | 0 | 0 | 0 | 0 |
| 28 | 2 | 2 | 0 | 0 |
| 29-30 | 2 | 2 | 2 | 2 |
| 31 | 2 | 2 | 0 | 0 |
| 32-33 | 0 | 0 | 0 | 0 |
| 34 | 0 | 0 | 0 | 0 |
| 35 | 0 | 0 | 0 | 0 |