| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G227 |
| --- | --- |

| Title: | Non-CE9: Crosscheck for Samsung's proposals (JCTVC-G552) by JVC KENWOOD |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@ jvckenwood.com fukushima.shigeru@ jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results of Samsung's proposals. It is confirmed that the BD-rate results match those provided by the proponent.

We confirmed that the results completely matched with the results provided by the proponent.

# Introduction

This contribution reports crosscheck results of Samsung's proposals (JCTVC-G552 [1]).

# Algorithm Description

## Current algorithm (HM4.0)

In HM4.0, the reference indices of temporal merging candidates are derived by the majority decision of the reference indices of the three neighboring block.

## Proposed algorithm

1) Utilization of upper-left position instead of center position for Merge and AMVP.

In the proposed technique, upper-left position instead of center position is used for temporal merge candidates of Merge and AMVP.

2) Utilization of zero reference index for TMVP candidate of Merge

In the proposed technique, the reference indices of temporal merging candidates are set to 0.

## Implementation

### Software

1) Utilization of upper-left position instead of center position for Merge and AMVP.

This proposed technique is implemented into HM4.0 software.

2) Utilization of zero reference index for TMVP candidate of Merge

This proposed technique has already been implemented into HM4.0 software.

The macro MRG_TMP_REF_IDX in TypeDef.h is set to 0 as follows:

#define MRG_TMVP_REFIDX 0 ///< (JCTVC-E481 - D274 (2) ) refidx derivation for merge TMVP

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2 and Table 3 present the simulation results for random access, and low delay B, respectively, when upper-left position is used instead of center position as one of TMVP candidate position.

Table 2 Simulation result for random access
when upper-left position is used instead of center position as one of TMVP candidate position

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 3 Simulation result for low delay B
when upper-left position is used instead of center position as one of TMVP candidate position

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.2% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | -0.2% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E | 0.0% | 0.4% | 0.5% | 0.1% | -0.1% | 0.3% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

The simulation results report that the proposed technique provides no gain or loss for random access and low delay B settings. (See the attached excel sheet)

Table 4 and Table 5present the simulation results for random access, and low delay B, respectively, when reference index of TMVP of Merge set by zero.

Table 4 Simulation results for random access
when reference index of TMVP of Merge set by zero

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 5 Simulation results for low delay B
when reference index of TMVP of Merge set by zero

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.3% | 0.6% | 0.1% | 0.7% | 0.8% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.4% | 0.3% |
| Class D | 0.3% | 0.8% | 0.4% | 0.2% | 0.8% | 0.3% |
| Class E | 0.0% | 0.1% | 0.5% | 0.1% | 1.0% | 0.6% |
| Overall | 0.1% | 0.3% | 0.4% | 0.1% | 0.7% | 0.5% |
|  | 0.1% | 0.3% | 0.4% | 0.1% | 0.7% | 0.6% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

The simulation results report that the proposed technique provides no gain or loss for random access settings, and 0.1% [3] loss for low delay B settings. (See the attached excel sheet)

Table 6 and Table 7 present the simulation results for random access, and low delay B, respectively, when reference index of TMVP of Merge set by zero and upper-left position is used instead of center position as one of TMVP candidate.

Table 6 Simulation results for randomaccess
when reference index of TMVP of Merge set by zero and 
upper-left position is used instead of center position as one of TMVP candidate

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 7 Simulation results for low delay B
when reference index of TMVP of Merge set by zero and 
upper-left position is used instead of center position as one of TMVP candidate

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.4% | 0.1% | 0.6% | 1.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.3% | 0.1% |
| Class D | 0.2% | 0.7% | 0.6% | 0.3% | 0.5% | 0.8% |
| Class E | 0.0% | 0.0% | 0.6% | 0.0% | 0.9% | 0.9% |
| Overall | 0.1% | 0.3% | 0.4% | 0.1% | 0.6% | 0.7% |
|  | 0.1% | 0.2% | 0.3% | 0.1% | 0.6% | 0.7% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

The simulation results report that the proposed technique provides no gain or loss for random access settings, and 0.1% [3] loss for low delay B settings. (See the attached excel sheet)

# Conclusions

We confirmed that the results completely matched with the results provided by the proponent.

# References

[1] I.-K. Kim, N. Shlyankhov, J. H. Park “Simplification of temporal motion vector (TMVP) candidate derivation for Merge and AMVP”,
   JCTVC-F552, Geneva, November 2011

[2] F. Bossen “Common test conditions and software reference configurations”,
   JCTVC-F900, Torino, July 2011

[3] G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,
   VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. or JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).