| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G228 |
| --- | --- |

| Title: | CE12.3.2: Reducing pixel line buffers by modifying DF for horizontal LCU boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports results of CE12.3.2, which is based on JCTVC-F053 method2 to modify deblocking filter (DF) only for horizontal LCU boundaries and to keep DF unchanged for rest edges. Pixels above the first row of the upper side of the horizontal LCU boundary are not used in filtering decisions. Moreover, filtering operations are also modified without changing pixels above the first row of the upper side of the horizontal LCU boundary. In comparison with HM-4.0, the proposed method can remove all pixel line buffers dedicated for DF and reportedly causes 0.0-0.2% coding efficiency loss with roughly unchanged run time and similar visual quality in most cases.

# Introduction

In High-Efficiency Video Coding test model version 4.0 (HM-4.0), deblocking filter (DF) contains horizontal filtering across vertical edges and vertical filtering across horizontal edges. When the largest coding units (LCUs) are processed by DF in a raster scan order, each LCU needs pixels from its upper LCU. Therefore, pixel line buffers of sizes proportional to the picture width are required. In HM-4.0, for luma vertical filtering, four pre-DF pixels on each side of an edge are used for filtering decisions, which requires four luma line buffers; for chroma vertical filtering, two horizontally filtered pixels (i.e. pixels after horizontal filtering, denoted as H-DF pixels) on each side of an edge are used, which requires two chroma line buffers. In this contribution, vertical filtering based on JCTVC-F053 [1] method2 is proposed only for horizontal LCU boundaries in order to reduce line buffers, while the DF in HM-4.0 is applied for rest edges. The rest of this document is organized as follows. Details of the proposed method will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed method

In the following, Figure 1 can be rotated in the clockwise direction by 90 degrees for demonstrating a horizontal edge where p pixels and q pixels are above and below the edge, respectively. The proposed method is applied only for horizontal LCU boundaries, and the DF in HM-4.0 is applied for rest edges. In HM-4.0, filtering decisions are applied for luma only and are bypassed for chroma. Pre-DF pixels are used in filtering decisions, which contain two parts: filter usage (on/off) decision and filter strength (strong/weak) selection. The filter usage decision is according to:

d=p'22−2⋅p'12+p'02+q'22−2⋅q'12+q'02+p'25−2⋅p'15+p'05+q'25−2⋅q'15+q'05<β

If the above condition is true, the filtering is applied for the edge; otherwise, the filtering is bypassed for the edge. Next, a filter strength selection is performed for each of the eight columns, i=0,…,7. The filter strength selection is according to:

d<β>>2andp'3i−p'0i+q'0i−q'3i<β>>3andp'0i−q'0i<5⋅tc+1>>1

If this condition is true, strong filtering is applied for the column; otherwise, weak filtering is applied for the column. Please note that p’ and q’ pixels are pre-DF pixels.

Figure 1.  Pixels involved in filtering an edge

In the proposed method, pixels above the first row of the upper side of the horizontal LCU boundary (p’1-p’3) are not used in the filtering decisions. Therefore, the proposed filter usage decision is according to:

d=q'22−2⋅q'12+q'02+q'25−2⋅q'15+q'05<β2

The proposed filter strength selection is according to:

d<β>>3andq'3i−q'0i<β>>4andp'0i−q'0i<5⋅tc+1>>1

Moreover, the luma and chroma filtering operations are also modified without changing pixels above the first row of the upper side of the horizontal LCU boundary. The original strong and weak filters for luma are described as follows.

Original strong filter for luma:

p2i=Clip0,255,2⋅p3i+3⋅p2i+p1i+p0i+q0i+4>>3

p1i=Clip0,255,p2i+p1i+p0i+q0i+2>>2

p0i=Clip0,255,p2i+2⋅p1i+2⋅p0i+2⋅q0i+q1i+4>>3

q0i=Clip0,255,p1i+2⋅p0i+2⋅q0i+2⋅q1i+q2i+4>>3

q1i=Clip0,255,p0i+q0i+q1i+q2i+2>>2

q2i=Clip0,255,p0i+q0i+q1i+3⋅q2i+2⋅q3i+4>>3

Original weak filter for luma:

Δ=Clip−tc,tc,9⋅q0i−p0i−3⋅q1i−p1i+8>>4

p0i=Clip0,255,p0i+Δ.

q0i=Clip0,255,q0i−Δ

p0 and q0 are allowed to be modified if abs() < (thr1) ,where thr1 is set equal to 10*tc.

Δ1=Clip−tc2,tc2,(p0i+p2i+1)>>1−p1i+Δ>>1

p1i=Clip0,255,p1i+Δ1

p1 is allowed to be modified if abs() < (thr1) and dp=p'22−2⋅p'12+p'02+p'25−2⋅p'15+p'05<thr2

where thr2 is set equal to (β+(β>>1))>>3

Δ2=Clip−tc2,tc2,(q0i+q2i+1)>>1−q1i+Δ>>1

q1i=Clip0,255,q1i+Δ2

q1 is allowed to be modified if abs() < (thr1) anddq=q'22−2⋅q'12+q'02+q'25−2⋅q'15+q'05<thr2

where thr2 is set equal to (β+(β>>1))>>3

Now the luma filtering operations are modified as follows.

Proposed strong filter for luma:

p0i=Clip0,255,p0i+2⋅p0i+2⋅p0i+2⋅q0i+q1i+4>>3

q0i=Clip0,255,p0i+2⋅p0i+2⋅q0i+2⋅q1i+q2i+4>>3

q1i=Clip0,255,p0i+q0i+q1i+q2i+2>>2

q2i=Clip0,255,p0i+q0i+q1i+3⋅q2i+2⋅q3i+4>>3

Proposed weak filter for luma:

Δ=Clip−tc,tc,9⋅q0i−p0i−3⋅q1i−p0i+8>>4

p0i=Clip0,255,p0i+Δ.

q0i=Clip0,255,q0i−Δ

p0 and q0 are allowed to be modified if abs() < (thr1) ,where thr1 is set equal to 10*tc.

p1 is not allowed to be modified.

Δ2=Clip−tc2,tc2,(q0i+q2i+1)>>1−q1i+Δ>>1

q1i=Clip0,255,q1i+Δ2

q1 is allowed to be modified if abs() < (thr1) anddq=q'22−2⋅q'12+q'02+q'25−2⋅q'15+q'05<thr2

where thr2 is set equal to (β+(β>>1))>>3

The original filter for chroma is described below.

Δ=Clip−tc,tc,4⋅q0i−p0i+p1i−q1i+4>>3

p0i=Clip0,255,p0i+Δ

q0i=Clip0,255,q0i−Δ

Now the chroma filtering operations are modified as follows.

Δ=Clip−tc,tc,4⋅q0i−p0i+p0i−q1i+4>>3

p0i=Clip0,255,p0i+Δ

q0i=Clip0,255,q0i−Δ

where p and q pixels are H-DF pixels. The proposed filtering decisions only use p’0 and q’0-q’3, and the proposed filtering operations only use p0 and q0-q3. In the proposed DF with LCU-based processing, the vertical filtering of the lowest horizontal LCU boundary has to wait for the lower LCU. The horizontal filtering for the last luma pixel row and the last chroma pixel row can be delayed until the lower LCU comes. When the lower LCU comes, the horizontal filtering is first performed on pre-DF pixels including the p’0 above the horizontal LCU boundary. Next, vertical filtering is applied. One line of pre-DF pixels, p’0, is used in filtering decisions, and then the generated H-DF pixel, p0, is used in filtering operations. Therefore, only one luma line buffer and one chroma line buffer are required to store pre-DF pixels. Fortunately, these pixel line buffers can be shared with intra prediction, which means there is no dedicated pixel line buffer for the proposed DF.

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-F900 [2], and the software is based on HM-4.0. There are three experiments including chroma only modification, where the chroma part of the proposed method is tested, luma only modification, where the luma part of the proposed method is tested, and combined modification, where both luma and chroma parts of the proposed method are tested. Simulation results of the proposed method are shown in Table 1, Table 2 and Table 3, respectively. The chroma-only modification does not cause any noticeable change in terms of BD-rate, run time, and visual quality. The luma-only modification and combined modification causes 0.0-0.2% bitrate increase while run time and subjective quality are similar to those of HM-4.0 in most cases.

# Conclusions

In this contribution, vertical filtering method based on JCTVC-F053 method2 was proposed for horizontal LCU boundaries to reduce pixel line buffers required by DF while the DF in HM-4.0 was still used for rest edges. The proposed method can remove all pixel line buffers dedicated for DF. It uses fewer pre-DF pixels above the horizontal LCU boundary for the filtering decisions, and filtering operations are also modified. Simulation results reportedly shows 0.0-0.2% bitrate increases with roughly unchanged run time and similar subjective quality in most cases when compared with HM-4.0.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-W. Hsu, J. An, X. Guo, J.-L. Lin, Y.-W. Huang, S. Lei, “Deblocking filter with reduced pixel line buffers for LCU-based processing,” Document of Joint Collaborative Team on Video Coding, JCTVC-F053, July 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.

Table 1.  Results of modification for chroma only

Table 2.  Results of modification for luma only

Table 3.  Results of modification for both luma and chroma