| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G229 |
| --- | --- |

| Title: | CE12.3.3: Reducing motion data line buffers |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports results of CE12.3.3, which is based on the motion data compression method in JCTVC-F060. Simulation results reportedly show that the proposed method can reduce motion data line buffer size by 50% with the same coding efficiency, encoding time, and decoding time in comparison with HM-4.0. No undesirable visual artifact is observed due to the modified motion data for calculating boundary strengths in deblocking filter.

# Introduction

Since LCUs are processed in raster scan order, the motion data of bottom 4x4 blocks on one LCU row are required to be stored in a line buffer for motion vector prediction and boundary strength calculation in deblocking filter (DF). The line buffer size is proportional to the picture width and can be calculated as 1216 bits times the number of LCUs on one row. For 4Kx2K videos, 9.73K bytes of memory are needed for motion data. In JCTVC-F060 [1], we proposed a motion data compression method to reduce the line buffer. It had no coding efficiency loss and could reduce 50% motion data line buffer on HM-3.0. The proposed technique was then included in CE12 to verify again on top of HM-4.0. The rest of this contribution is organized as follows. The proposed algorithm will be described in Section 2, simulation results will be shown in Section 3, and finally conclusions will be given in Section 4.

# Algorithm description

Motion data of a block include inter prediction direction (2 bits), reference index (4 bits), and motion vector (MV, 16 bits for MV_x and 16 bits for MV_y). In order to reduce the line buffer storage and access, it is proposed to compress the motion data in the line buffer. For each PU that needs to access the motion data of the upper LCU row, only the compressed motion data in the line buffer can be accessed.

Figure 1 shows an example of the proposed 2:1 motion data compression of one line. For every four 4x4 blocks, the motion data of the first and the last 4x4 blocks are stored in the line buffer. The motion data of the first block represents the motion data of the first two 4x4 blocks. The motion data of the last block represents the motion data of the last two 4x8 blocks. For example, the motion data of block 0 represents the motion data of blocks 0 and 1; the motion data of block 3 represents the motion data of blocks 2 and 3. In this way, only half memory is required. The memory size of motion data line buffer for 4Kx2K videos is reduced from 9.73K bytes to 4.86K bytes.

Since the motion data in the line buffer are compressed, the boundary strengths of DF may be changed if the upper PU is in the upper LCU row and the block partition of the upper PU is 4x4 or 4x8. Please recall that in HM-4.0, no inter 4x4 block is used by default.

Figure 1.  The proposed 2:1 motion data compression of one line

# Simulation results

The proposed method is implemented on HM-4.0. All results were simulated according to the common test conditions defined in JCTVC-F900 [2]. The tested conditions are high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD). Table 1 shows the results of the proposed 2:1 motion data compression method. Almost no bit rate increase is observed. Encoding and decoding times are roughly unchanged.

Table 1.  Results of the 2:1 motion data compression.

Since the boundary strengths of DF may be changed by the proposed motion data compression, we carefully evaluated the subjective view quality. We did not observe any undesirable visual artifact caused by the proposed method. The visual quality of the proposed method is the same as that of HM-4.0.

# Conclusions

MediaTek proposed a 2:1 motion data compression method to reduce the motion data line buffer size. Simulation results reportedly show that the 2:1 motion data compression can reduce 50% motion data line buffer size without coding efficiency loss. Encoding time, decoding time, and subjective quality are unchanged.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei, “Reducing line buffers for motion data and CABAC,” Document of Joint Collaborative Team on Video Coding, JCTVC-F060, July 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.