| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G230 |
| --- | --- |

| Title: | Non-CE12.3: Reducing pixel line buffers by modifying DF to R3W2 for horizontal LCU boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Masaru Ikeda, Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610 JAPAN Seungwook Park, Byeongmoon Jeon 221, Yangjae-Dong, Seocho-Gu, Seoul, Korea | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com Teruhiko Suzuki +81-50-3750-2740 {Masaru.Ikeda, teruhikos}@jp.sony.com Byeongmoon Jeon +82-10-8203-1126 {seungwook.park, bm.joen}@lge.com |
| Source: | MediaTek Inc., Sony Corporation, LG |  |  |

_____________________________

# Abstract

In this contribution, modified vertical filtering with reading 3 pixels and writing 2 pixles (R3W2) for deblocking filter (DF) is proposed for horizontal LCU boundaries to reduce pixel line buffers required by DF, while the DF in HM-4.0 is still used for rest edges. The proposed method can save one pixel line buffer for DF compared with the current HM-4.0. It uses fewer pre-DF pixels above the horizontal LCU boundary for the filtering decisions, and filtering operations are also modified. Simulation results reportedly show 0% bit rate increase with unchanged run time and similar subjective quality in comparison with HM-4.0.

# Introduction

Deblocking filter (DF) contains horizontal filtering across vertical edges and vertical filtering across horizontal edges. When the largest coding units (LCUs) are processed by DF in a raster scan order, each LCU needs pixels from its upper LCU. Therefore, pixel line buffers of sizes proportional to the picture width are required. In HM-4.0, for luma vertical filtering, four reconstructed pixels on each side of an edge are read for filtering decisions, and at most three pixels are filtered and written back, which requires four luma line buffers. Because reading four pixels and writing three pixels are required above LCU horizontal boundaries in HM-4.0, we call it “Read 4 and Write 3” case, which is R4W3 in short.

In this contribution, in order to reduce luma line buffers, modified DF with R3W2 is proposed to deal with the pixels above LCU horizontal boundaries, while the DF in HM-4.0 is applied for rest edges. The rest of this document is organized as follows. Details of the proposed method will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed method

In the following, Figure 1 can be rotated in the clockwise direction by 90 degrees for demonstrating a horizontal edge where p pixels and q pixels are above and below the edge, respectively. The proposed method is applied only for horizontal LCU boundaries, and the DF in HM-4.0 is applied for rest edges. In HM-4.0, filtering decisions are applied for luma only and are bypassed for chroma. Pre-DF pixels are used in filtering decisions, which contain two parts: filter usage (on/off) decision and filter strength (strong/weak) selection. The filter usage decision is according to:

d=p'22−2⋅p'12+p'02+q'22−2⋅q'12+q'02+p'25−2⋅p'15+p'05+q'25−2⋅q'15+q'05<β

If the above condition is true, the filtering is applied for the edge; otherwise, the filtering is bypassed for the edge. Next, a filter strength selection is performed for each of the eight columns, i=0,…,7. The filter strength selection is according to:

d<β>>2andp'3i−p'0i+q'0i−q'3i<β>>3andp'0i−q'0i<5⋅tc+1>>1

If this condition is true, strong filtering is applied for the column; otherwise, weak filtering is applied for the column. Please note that p’ and q’ pixels are pre-DF pixels.

In HM-4.0, four pixels (p’0-p’3) above horizontal LCU boundaries are required for filtering decisions.

Figure 1.  Pixels involved in filtering an edge

In the proposed R3W2 DF, only three pixels (p’0-p’2) above the horizontal LCU boundary are allowed to be used in the filtering decisions. Therefore, the proposed filter usage decision is according to:

d=p'22−2⋅p'12+p'02+q'22−2⋅q'12+q'02+p'25−2⋅p'15+p'05+q'25−2⋅q'15+q'05<βThe proposed filter strength selection is according to:

d<β>>2andp'2i−p'0i+q'0i−q'3i<β>>3andp'0i−q'0i<5⋅tc+1>>1

Moreover, the luma filtering operations are also modified with changing at most two pixels above the first row of the upper side of the horizontal LCU boundary. The original strong and weak filters for luma are described as follows.

Original strong filter for luma:

p2i=Clip0,255,2⋅p3i+3⋅p2i+p1i+p0i+q0i+4>>3

p1i=Clip0,255,p2i+p1i+p0i+q0i+2>>2

p0i=Clip0,255,p2i+2⋅p1i+2⋅p0i+2⋅q0i+q1i+4>>3

q0i=Clip0,255,p1i+2⋅p0i+2⋅q0i+2⋅q1i+q2i+4>>3

q1i=Clip0,255,p0i+q0i+q1i+q2i+2>>2

q2i=Clip0,255,p0i+q0i+q1i+3⋅q2i+2⋅q3i+4>>3

Original weak filter for luma:

Δ=Clip−tc,tc,9⋅q0i−p0i−3⋅q1i−p1i+8>>4

p0i=Clip0,255,p0i+Δ.

q0i=Clip0,255,q0i−Δ

p0 and q0 are allowed to be modified if abs() < (thr1) ,where thr1 is set equal to 10*tc.

Δ1=Clip−tc2,tc2,(p0i+p2i+1)>>1−p1i+Δ>>1

p1i=Clip0,255,p1i+Δ1

p1 is allowed to be modified if abs() < (thr1) and dp=p'22−2⋅p'12+p'02+p'25−2⋅p'15+p'05<thr2

where thr2 is set equal to (β+(β>>1))>>3

Δ2=Clip−tc2,tc2,(q0i+q2i+1)>>1−q1i+Δ>>1

q1i=Clip0,255,q1i+Δ2

q1 is allowed to be modified if abs() < (thr1) anddq=q'22−2⋅q'12+q'02+q'25−2⋅q'15+q'05<thr2

where thr2 is set equal to (β+(β>>1))>>3

Figure 2 shows waveform of both pre-deblocking (black waveform with blue pixels) and post-deblocking (green waveform with green pixels) in strong filtering. When simple waveform (p2i=p1i=p0i, q0i=q1i=q2i) is considered now, pixel p2i values changed by strong filter are as follows.

p’2i - p2i 	= (2*p3i + 3*p2i + p1i + p0i + q0i + 4)>>3 - p2i

= (2*p3i - 5*p2i + p1i + p0i + q0i + 4)>>3

= (-p1i + q0i + 4)>>3				(p2i=p1i=p0i, q0i=q1i=q2i)

= (-D + 4)*1/8					(D= p0i - q0i)

Pixel value of p1i, p0i, q0i, q1i and q1i is also changed in the same way.

p’1i – p1i 	= (-2*D + 4)*1/8

p’0i – p0i 	= (-3*D + 4)*1/8

q’0i – q0i 	= (3*D + 4) *1/8

q’1i – q1i 	= (2*D + 4) *1/8

q’2i – q2i 	= (D + 4) *1/8

The waveform (p2i-p1’i-p0’i) is not smooth compared with the waveform between (p2’i-p1’i-p0’i) because p2i is not changed in case of R3W2. If p1i is changed into central position p1”i (red pixel in Figure 2) between p2i and p0’i, the waveform (p2i-p1”i-p0’i) becomes smoother than the waveform (p2i-p1’i-p0’i) . Proposed filtering operation of p1i is derived as follows.

p’’1i – p1i 	= (-3*D + 2*4) *1/16

= (-3*( p0i - q0i) + 8)*1/16

= (13* p0i + 3*q0i + 8)>>4 – p1i

= (4* p2i + 5*p1i + 4*p0i + 3*q0i + 8)>>4 – p1i		(a)

The each coefficient in (a) is decided based on both the same pixels as the original filtering operation of p1i and the weight of central position p1i.

Figure 2.  Pixels performed by strong filter

Now the luma filtering operations are modified as follows.

Proposed strong filter for luma:

p1i=Clip0,255,4⋅p2i+5⋅p1i+4⋅p0i+3⋅q0i+8>>4

p0i=Clip0,255,p2i+2⋅p1i+2⋅p0i+2⋅q0i+q1i+4>>3

q0i=Clip0,255,p1i+2⋅p0i+2⋅q0i+2⋅q1i+q2i+4>>3

q1i=Clip0,255,p0i+q0i+q1i+q2i+2>>2

q2i=Clip0,255,p0i+q0i+q1i+3⋅q2i+2⋅q3i+4>>3

Since two pixels above horizontal LCU boundaries are allowed to be filtered and changed, the proposed weak filter for luma is the same as that in HM-4.0.

Please not that p and q pixels are pixels after horizontal deblocking (denoted as H-DF pixels). The proposed filtering decisions only use p’0-p’2 and q’0-q’3, and the proposed filtering operations only use p0-p2 and q0-q3. The horizontal filtering for the last three luma pixel rows can be delayed until the lower LCU comes. When the lower LCU comes, the horizontal filtering is first performed on pre-DF pixels including p0-p2. Next, vertical filtering is applied. Three lines of pre-DF pixels, p’0-p’2, are used in filtering decisions, and then the generated H-DF pixel, p0-p2, are used in filtering operations. Therefore, only three luma line buffers are required to store pre-DF pixels, which saves one luma line compared with HM-4.0.

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-F900 [1], and the software is based on HM-4.0. Simulation results of the proposed method are shown in Table 1. The modification does not cause any noticeable change in terms of BD-rate, run time, and visual quality.

# Conclusions

MediaTek, Sony and LG proposed R3W2 DF for horizontal LCU boundaries to reduce pixel line buffers required by DF, while the DF in HM-4.0 is still used for rest edges. The proposed method can save one pixel line buffer for DF compared with HM-4.0. It uses fewer pre-DF pixels above the horizontal LCU boundary for the filtering decisions, and filtering operations are also modified. Simulation results reportedly shows 0% bit rate increase with unchanged run time and similar subjective quality in comparison with HM-4.0.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.

Table 1.  Results of the proposed R3W2 DF