| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G231 |
| --- | --- |

| Title: | CE13: Results of tests 1, 3d, and 3e in section 3.1 on replacing redundant MVPs and its combination with adaptive MVP list size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { jl.lin, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE13 section 3.1 tests 1, 3d, and 3e, which are based on JCTVC-F052. In test 1, redundant or empty MVPs are replaced by truncating the first available MVP to integer precision or by adding a constant value to the first available MVP. In test 3d, test 1 is combined with adaptive MVP list size by neighboring Merge indices. In test 3e, test 1 is combined with adaptive MVP list size by current CU size. In comparison with HM-4.0 under JCTVC-F900 common test conditions, it is reported that test 1 together with a bug-fix achieves 0.2-0.5% coding efficiency gain with 100-103% encoding time and 98-100% decoding time, test 3d together with a bug-fix achieves 0.0-0.4% coding efficiency gain with 98-103% encoding time and 99-102% decoding time, and test 3e together with a bug-fix achieves 0.1-0.5% coding efficiency gain with 100-102% encoding time and 100-101% decoding time, where the bug-fix alone achieves 0.1% coding efficiency gain and no run time difference.

# Introduction

In HM-4.0, to decouple MVP list construction and MVP index parsing, the MVP list size is fixed to five and two for Merge and AMVP, respectively. In order to compensate the coding efficiency loss caused by the fixed MVP list size, additional MVPs are used to replace redundant MVPs or to fill empty positions in the MVP list. As described in CE13 [1], the goal of this contribution is to evaluate the performance of proposed replacing MVPs and its combination with various methods of adaptive list size. The rest of this contribution is organized as follows. Proposed methods will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed methods

Two categories are proposed: generating additional MVPs and its combination with adaptive list size.

## Generating additional MVPs – test 1

In this category, we test additional MVPs for Merge mode. In test 1, if the number of available MVPs is smaller than the list size, two additional MVPs are derived by modifying the first available MVP in the MVP list according to the following procedure until the MVP list size has reached its maximum or both additional MVPs have been tested:

If the first available MVP in the MVP list has a list0 MV

- The first additional MVP is derived by truncating the list0 MV to integer precision. This additional MVP is added to the MVP list if it is not redundant.

- The second additional MVP is derived by adding one integer offset to the list 0 MV in the horizontal dimension. This additional MVP is added to the MVP list if it is not redundant.

Else (if the first available MVP in the MVP list has a list1 MV)

- The first additional MVP is derived by truncating the list1 MV to integer precision. This additional MVP is added to the MVP list if it is not redundant.

- The second additional MVP is derived by adding one integer offset to the list1 MV in the horizontal dimension. This additional MVP is added to the MVP list if it is not redundant.

## Combination of additional MVPs and adaptive list size – test 3d and test 3e

In this category, we test two combinations of additional MVPs (test 1) with two methods of adaptive list size for Merge mode. In CE13 section 3.1 tests 2d and 2e, two tools to determine the MVP list size adaptively are proposed [2]. In test 2d, the MVP list size of each PU is adaptively determined by merge indices of neighboring PUs. In test 2e, the MVP list size is determined by the current CU size. The combination of test 1 with test 2d and test 2e are described in the followings.

Test 3d, additional MVPs combined with adaptive list size by neighboring Merge indices:

In this test, the MVP list size of a current PU for Merge mode is determined according to the final coded MVP indices of its neighboring PUs. The MVP list size, S, is decided by the maximum value of the coded MVP indices of neighboring PUs plus a constant value, N, which is coded in the slice header. The MVP list size can be calculated using the following equation:

,

S = max( Idxleft , Idxabove ) + N

where Idxleft and Idxabove represent the Merge index of the left PU and that of the above PU, respectively.

If the number of available MVPs is smaller than the list size determined by the Merge indices, two additional MVPs proposed in test 1 are added to the MVP list until the MVP list size has reached its adaptive list size or both additional MVPs have been tested.

Test 3e, additional MVP combined with adaptive list size by the current CU size:

In this test, the MVP list size of a current PU for Merge is determined according to the current CU size. When the current CU size is smaller than or equal to 8x8, the list size is reduced from 5 to 4; otherwise the list size is 5. If the number of available MVPs is smaller than the list size determined by the CU size, two additional MVPs proposed in test1 are added to reach its MVP list size.

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-F900 [2]. As shown in Table 1, there are totally three experiments including test 1, test 3d and test 3e.

The software is based on HM-4.0 with the merge estimation bug fixed in the encoder. In the PU merge process of HM-4.0, the encoder only considers distortion to decide the best merge MVP. The bug fix takes rate into consideration. As shown in Table 2, the bug fix can result in coding efficiency gains as 0.1%, 0.1%, 0.2%, 0.1%, 0.1% and 0.0% for HE-RA, LC-RA, HE-LDB, LC-LDB, HE-LDP, and LC-LDP, respectively, with unchanged encoding and decoding times.

Results of the bug fix and test 1 are shown in Table 3. Coding efficiency gains are observed as 0.2%, 0.2%, 0.4%, 0.3%, 0.3% and 0.5% for HE-RA, LC-RA, HE-LDB, LC-LDB, HE-LDP, and LC-LDP, respectively, with 100-103% encoding time and 98-100% decoding time. Because we add integer MVPs that do not need interpolation, the decoding time can be reduced.

Results of the bug fix and test 3d (test 1 + test 2d) are shown in Table 4. Coding efficiency gains are observed as 0.0%, 0.0%, 0.2%, 0.2%, 0.2%, and 0.4% for HE-RA, LC-RA, HE-LD, LC-LD, HE-LDP, and LC-LDP, respectively, with 98-103% encoding time and 99-102% decoding time.

Results of the bug fix and test 3e (test 1 + test 2e) are shown in Table 5. Coding efficiency gains are observed as 0.1%, 0.2%, 0.3%, 0.2%, 0.3%, and 0.5% for HE-RA, LC-RA, HE-LD, LC-LD, HE-LDP, and LC-LDP, respectively, with 100-102% encoding time and 100-101% decoding time.

# Conclusions

MediaTek proposed three tests in this contribution. Test 1 replaces redundant or empty MVPs by truncating the first available MVP to integer precision or by adding a constant value to the first available MVP. Test 3d combines test1 with the adaptive MVP list size determined by merge indices. Test 3e combines test 1 with the adaptive MVP list size according to CU size. It is reported that test l achieves 0.1-0.5% coding efficiency gain with 100-103% encoding time and 98-100% decoding time, test 3d achieves -0.1-0.4% coding efficiency gain with 98-103% encoding time and 99-102% decoding time, and test 3e achieves 0.0-0.5% coding efficiency gain with 100-102% encoding time and 100-101% decoding time.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Jung, F. Bossen, “Description of Core Experiment CE13: Motion data parsing robustness and throughput,” Document of Joint Collaborative Team on Video Coding, JCTVC-F913, July 2011.

- J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei, “CE13: Results of tests 2d and 2e in section 3.1 on adaptive MVP list size,” Document of Joint Collaborative Team on Video Coding, JCTVC-G232, November 2011.

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.

- J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei, “CE9: Results of experiment ROB04,” Document of Joint Collaborative Team on Video Coding, JCTVC-F052, July 2011.

Table 1.  Descriptions of tests

|  | Redundancy Replacement | Fixed List Size | Adaptive List Size by Merge Index | Adaptive List Size by CU Size |
| --- | --- | --- | --- | --- |
| HM-4.0 | Off | (5,2) | Off | Off |
| Test 1 | On | (5,2) | Off | Off |
| Test 3d | On | Off | On | Off |
| Test 3e | On | Off | Off | On |

Table 2.  Results of the bug-fix

Table 3.  Results of the bug-fix and test 1

Table 4.  Results of the bug-fix and test 3d (bug-fix + test1 + test2d)

Table 5.  Results of the bug-fix and test 3e (bug-fix + test1 + test2e)