##### WD for Test1

##### 8.4.2.1.1 Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-1)

predFlagLXCol = availableFlagLXCol	(8-2)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList and the variable maxNumMergeCand is set to 5.

- When slice_type is equal to B, the following applies.

  - The derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

  - The derivation process for non-scaled bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0nscaleCandl and refIdxL1nscaleCandl, the prediction list utilization flags predFlagL0nscaleCandl and predFlagL1nscaleCandl, the motion vectors mvL0nscaleCandl and mvL1nscaleCandl of every new candidate nscaleCandl being added in mergeCandList. The number of candidates being added numNscaleMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numNscaleMergeCand is greater than 0, l ranges from 0 to numNscaleMergeCand – 1, inclusive.

- The derivation process for additional motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, the NumMergeCand and the MaxNumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0AddCandm and refIdxL1AddCandm, the prediction list utilization flags predFlagL0AddCandm and predFlagL1AddCandm, the motion vectors mvL0AddCandm and mvL1AddCandm of every new candidate AddCandm being added in mergeCandList. The number of candidates being added numAddMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numAddMergeCand is greater than 0, m ranges from 0 to numAddMergeCand – 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, the NumMergeCand and the MaxNumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand – numAddMergeCand). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand – 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-3)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-4)

refIdxLX = refIdxLXN	(8-5)

predFlagLX = predFlagLXN	(8-6)

##### 8.4.2.1.x  Derivation process for additional merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process,

- the maximum number of elements maxNumMergeCand within the mergeCandList.

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0AddCandl and refIdxL1AddCandl of every new candidate AddCandl being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0AddCandl and predFlagL1AddCandl of every new candidate AddCandl being added in mergeCandList during the invokation of this process,

- motion vectors mvL0AddCandl and mvL1AddCandl of every new candidate AddCandl being added in mergeCandList during the invokation of this process,

The function RefPicOrderCnt( refidx, LX ) specifies the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of the current picture with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”. The variable PicOrderCnt specifies the PicOrderCnt of the current picture.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

The variable AddStop is initialized to true. When numOrigMergeCand is greater than 0 and numMergeCand is less than maxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables origIdx and nAddCnt are set to 0, the variable AddStop is set to false and the following steps are repeated until AddStop is equal to true.

- The following assignments are made with origCand being the candidate at position 0 in the merging candidate list mergeCandList ( origCand = mergeCandList[ 0 ] ).

- If refIdxL0origCand >= 0

    - When AddCnt is less than 1, numMergeCand is less than maxNumMergeCand, the following applies.

      - refIdxL0AddCandl = refIdxL0origCand		(8-7)

      - refIdxL1AddCandl = refIdxL1origCand		(8-8)

      - predFlagL0AddCandl = predFlagL0origCand		(8-9)

      - predFlagL1AddCandl = predFlagL1origCand		(8-10)

      - mvL0AddCandl[ 0 ] = ( mvL0origCand[ 0 ] >>2) <<2	(8-11)

      - mvL0AddCandl[ 1 ] = ( mvL0origCand[ 1 ]>>2) <<2 	(8-12)

      - mvL1AddCandl[ 0 ] =  mvL1origCand[ 0 ]                              (8-13)

      - mvL1AddCandl[ 1 ] =  mvL1origCand[ 1 ]                     	(8-14)

      - numMergeCand = numMergeCand + 1	(8-15)

      - The variable AddCnt is incremented by 1.

- Else if refIdxL1origCand >= 0

    - When nTruncatedCnt is less than 1, numMergeCand is less than maxNumMergeCand, and, the following applies.

      - refIdxL0AddCandl = refIdxL0origCand		(8-16)

      - refIdxL1AddCandl = refIdxL1origCand		(8-17)

      - predFlagL0AddCandl = predFlagL0origCand		(8-18)

      - predFlagL1AddCandl = predFlagL1origCand		(8-19)

      - mvL0AddCandl[ 0 ] = mvL0origCand[ 0 ]                  	(8-20)

      - mvL0AddCandl[ 1 ] = mvL0origCand[ 1 ]                  	(8-21)

      - mvL1AddCandl[ 0 ] =  ( mvL1origCand[ 0 ] >>2) <<2                              (8-22)

      - mvL1AddCandl[ 1 ] =  ( mvL1origCand[ 0 ] >>2) <<2                     	(8-23)

      - numMergeCand = numMergeCand + 1	(8-24)

      - The variable AddCnt is incremented by 1.

- If refIdxL0origCand >= 0

    - When AddCnt is less than 1, numMergeCand is less than maxNumMergeCand, the following applies.

      - refIdxL0AddCandl = refIdxL0origCand		(8-25)

      - refIdxL1AddCandl = refIdxL1origCand		(8-26)

      - predFlagL0AddCandl = predFlagL0origCand		(8-27)

      - predFlagL1AddCandl = predFlagL1origCand		(8-28)

      - mvL0AddCandl[ 0 ] = mvL0origCand[ 0 ] + 4        	(8-29)

      - mvL0AddCandl[ 1 ] = mvL0origCand[ 1 ]                  	(8-30)

      - mvL1AddCandl[ 0 ] =  mvL1origCand[ 0 ]                              (8-31)

      - mvL1AddCandl[ 1 ] =  mvL1origCand[ 1 ]                     	(8-32)

      - numMergeCand = numMergeCand + 1	(8-33)

      - The variable AddCnt is incremented by 1.

- Else if refIdxL1origCand >= 0

    - When nTruncatedCnt is less than 1, numMergeCand is less than maxNumMergeCand, and, the following applies.

      - refIdxL0AddCandl = refIdxL0origCand		(8-34)

      - refIdxL1AddCandl = refIdxL1origCand		(8-35)

      - predFlagL0AddCandl = predFlagL0origCand		(8-36)

      - predFlagL1AddCandl = predFlagL1origCand		(8-37)

      - mvL0AddCandl[ 0 ] = mvL0origCand[ 0 ]                  	(8-38)

      - mvL0AddCandl[ 1 ] = mvL0origCand[ 1 ]                  	(8-39)

      - mvL1AddCandl[ 0 ] =  mvL1origCand[ 0 ] + 4                              (8-40)

      - mvL1AddCandl[ 1 ] =  mvL1origCand[ 0 ]                     	(8-41)

      - numMergeCand = numMergeCand + 1	(8-42)

- The variable AddCnt is incremented by 1.

- When origIdx is equal to numOrigMergeCand or numMergeCand is equal to maxNumMergeCand or AddCnt is equal to 2, AddStop is set to true.

#### WD for Test2d

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| pcm_loop_filter_disable_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| inter_4x4_enabled_flag | u(1) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) | ue(v) |
| mrg_list_size[i] | ue(v) |
| rbsp_trailing_bits( ) |  |
| } |  |

##### 8.4.2.1.2 Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-43)

predFlagLXCol = availableFlagLXCol	(8-44)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- max_mrg_idx=Max(merge_idx[( xP - 1,  yP + nPSH )][ yP + nPSH ], merge_idx[xP + nPSW][ yP - 1 ])

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList and the variable maxNumMergeCand is set to mrg_list_size[max_mrg_idx] .

#### WD for Test2e

##### 8.4.2.1.2 Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-45)

predFlagLXCol = availableFlagLXCol	(8-46)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList. If nPSW is larger than 8 and nPSH is larger than 8, the variable maxNumMergeCand is set to 5, otherwise,  the variable maxNumMergeCand is set to 4.