| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G233 |
| --- | --- |

| Title: | CE1: Subtest 13, Disabling CABAC probability model update |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Test Report |  |  |
| Author(s) or Contact(s): | T. Davies 14 Waterside Drive Langley, SL3 6EZ, UK | Email: | thdavies@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This contributions presents the results of Subtest 13 of Core Experiment 1 on disabling PMU in CABAC. With RDOQ on, BD-Rate differences of 2.8%-6.2% are observed. With RDOQ off, BD-Rate differences of 4.0%-7.3% are observed. It is noted that BD-Rate differences are very much higher for Class F sequences, and that this is likely due to unsuitable initialization tables.

# Introduction

This contribution presents the results of Subtest 13 of Core Experiment 1 [1]. This experiment considers the effect of turning off PMU in CABAC.

Two experiments have been conducted. Both use CABAC on low-complexity configurations. In the first experiment, CABAC is compared against CABAC with PMU disabled. This produces a BD-Rate difference of 2.8% (AI) to 6.2%(RA and LD-B).

In the second experiment, CABAC without RDOQ is compared against CABAC with PMU disabled and also without RDOQ. This produces a BD-Rate difference of 4.0% (AI) to 7.3% (LD-B).

# Results

Results are presented in the following tables. Although the average over Class F sequences is shown in each case, the Overall averages reported do not include Class F.

## Subtest 1: CABAC versus CABAC, no PMU

Software: HM4.0 (anchor) and HM4.0 modified with CABAC PMU disabled

Configurations tested: All LC common conditions for 4.0 [2].

Simulation parameters: --SymbolMode=1 on LC settings

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 3.1% | -1.0% | -1.4% |
| Class B | 3.4% | 1.3% | 0.2% |
| Class C | 1.9% | 2.1% | 2.0% |
| Class D | 1.1% | 1.3% | 1.5% |
| Class E | 4.7% | -6.3% | -5.1% |
| Class F | 5.3% | 6.1% | 6.6% |
| Overall | 2.8% | -0.2% | -0.3% |
|  | 2.7% | -0.1% | -0.2% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Class A | 7.2% | 7.4% | 6.6% |
| Class B | 7.0% | 8.9% | 7.1% |
| Class C | 5.4% | 9.0% | 8.8% |
| Class D | 5.1% | 7.0% | 7.0% |
| Class E |  |  |  |
| Class F | 9.2% | 10.9% | 11.4% |
| Overall | 6.2% | 8.1% | 7.4% |
|  | 6.5% | 8.3% | 7.5% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 7.2% | 13.8% | 11.1% |
| Class C | 4.9% | 11.5% | 11.5% |
| Class D | 5.4% | 10.1% | 10.6% |
| Class E | 7.2% | 2.7% | 0.6% |
| Class F | 11.7% | 14.9% | 15.2% |
| Overall | 6.2% | 10.2% | 9.1% |
|  | 6.1% | 10.2% | 9.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Low delay P LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 6.4% | 8.4% | 5.6% |
| Class C | 3.8% | 7.3% | 7.1% |
| Class D | 4.1% | 5.0% | 6.4% |
| Class E | 7.0% | -1.0% | -2.1% |
| Class F | 10.8% | 12.6% | 13.6% |
| Overall | 5.3% | 5.5% | 4.7% |
|  | 5.2% | 5.6% | 4.9% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |

## Subtest 2: CABAC no RDOQ versus CABAC, no PMU, no RDOQ

Software: modified HM4.0 with sub-LCU PMU disabled for CABAC (disabled by default for CAVLC)

Configurations tested: All LC common conditions for 4.0 [2].

Simulation parameters: --SymbolMode=0 --RDOQ=0 versus --SymbolMode=1 --RDOQ on LC settings

|  | All Intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 3.9% | -0.3% | -0.3% |
| Class B | 5.1% | 2.2% | 1.5% |
| Class C | 3.6% | 2.2% | 2.0% |
| Class D | 2.3% | 1.6% | 1.6% |
| Class E | 4.9% | -4.2% | -3.8% |
| Class F | 6.1% | 4.5% | 4.6% |
| Overall | 4.0% | 0.6% | 0.4% |
|  | 4.0% | 0.7% | 0.5% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access LC |  |  |
|  | Y | U | V |
| Class A | 7.4% | 3.6% | 3.2% |
| Class B | 7.6% | 6.2% | 4.9% |
| Class C | 5.8% | 6.8% | 6.8% |
| Class D | 5.5% | 5.4% | 5.2% |
| Class E |  |  |  |
| Class F | 9.0% | 8.3% | 8.4% |
| Overall | 6.6% | 5.5% | 5.0% |
|  | 6.5% | 5.7% | 5.1% |
| Enc Time[%] | 92% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 8.3% | 12.5% | 10.5% |
| Class C | 6.0% | 10.2% | 10.1% |
| Class D | 6.6% | 7.9% | 8.1% |
| Class E | 8.4% | 0.7% | -1.1% |
| Class F | 11.3% | 11.7% | 12.6% |
| Overall | 7.3% | 8.6% | 7.6% |
|  | 7.3% | 8.7% | 7.5% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay P LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 7.3% | 7.5% | 5.5% |
| Class C | 4.8% | 6.4% | 5.9% |
| Class D | 5.1% | 3.7% | 3.4% |
| Class E | 8.3% | -1.3% | -2.8% |
| Class F | 10.4% | 10.3% | 10.5% |
| Overall | 6.3% | 4.6% | 3.5% |
|  | 6.3% | 4.7% | 3.6% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 101% |  |  |

# Conclusions

Disabling PMU with and without RDOQ provides losses up to 6.2% (with RDOQ) or 7.3% (without RDOQ). It is interesting to observe that RDOQ is to some extent compensating for the loss of PMU. The RDOQ process in HM4.0 uses probability estimates without modeling adaption and so is well-matched to the PMU-disabled case.

Class F results have not been included in the overall averages, and results are substantially worse in this case. For SlideEditing with LD-B configuration in Experiment 1, the measured BD-R difference is 19.6%. This is no doubt because CABAC context initialization has not been optimised for screen content of this sort. It is likely that use of a non-adaptive design will require multiple initialization tables.

# References

[1] JCTVC-F901, “Description of Core Experiment 1: Entropy Coding”, R. Joshi (Qualcomm), E. Alshina (Samsung), H. Sasai (Panasonic), H. Kirchhoffer (HHI), J. Lainema (Nokia),  JCT-VC 6th Meeting, Turin, Italy, 14-22 July

[2]	JCTVC-900, “Common test conditions and software reference configurations”, F. Bossen, JCT-VC 6th Meeting, Turin, Italy, 14-22 July 2011

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).