| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G235 |
| --- | --- |

| Title: | CE8.h: CU-based ALF with non-local means filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaaki Matsumura Seishi Takamura Hirohisa Jozawa 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-3382 jozawa.hirohisa@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This contribution reports the performance of a technique that utilizes a denosing filter as the in-loop filter of HM codec. In the proposed method, a denoising filter called non-local means filter is unified into CU-based adaptive loop filter of HM4.0.

Compared to the anchor of HM4.0, the average BD-rate gains were –0.1, –0.2, –0.3, and –0.5% for Intra, Random Access, Low Delay B, and Low Delay P, respectively. The average decoding time increased 2 to 5%. The maximum gain was –1.4% in Low Delay P for the sequence “BasketballDrive”.

# Introduction

The in-loop filter that supports to reduce the distortion of a decoded picture is one of important coding tools for improving coding efficiency. HM4.0 adopts three kinds of filters; deblocking filter, sample adaptive offset and adaptive loop filter (ALF) [1]. Among them, CU-based ALF is implemented as one of the ALF modes, and selects a filter pattern whether to apply ALF for each CU.

This contribution proposes the unification of ALF and non-local means (NLM) approach [2, 3]. The proposed method selects the filter pattern from ALF and NLM filter for each CU (Figure 1). NLM filter derives the filter coefficients of each neighboring samples on-the-fly. By derivation of the filter coefficients of NLM for each sample, favorable denoised result was obtained [4], since the filter coefficients are locally adapted.

Figure 1: Comparison of conventional CU-based ALF and proposed method.

In this contribution, Section 2 contains syntax and semantics. The decoding description is provided in section 3. In Section 4, describes experimental results with objective/subjective evaluations are described. The conclusion is contained in Section 5, references in Section 6.

# Syntax and semantics

In this section, modified syntax and semantics are described. The modified syntax is specified in Table 1. The difference between conventional ALF and proposal set highlighted in yellow. The modified semantics used in the syntax are specified below:

alf_cu_flag[i] specifies the filter pattern. If the alf_cu_flag is equal to 1, ALF is applied to a CU. Otherwise, NLM filter is applied to the CU.

alf_nlm_coefficient specifies the strength of NLM filter. The value of alf_nlm_coefficient shall be in the range of 0 to 255, inclusive. If alf_nlm_coefficient is equal to 0, NLM filter does not apply to the luma comportment.

alf_nlm_threshold_1 specifies the threshold that identifies whether NLM filter applies or not. The local activity (Activity) between a sample and neighboring samples is derived. If the Activity is greater than alf_nlm_threshold_1, NLM filter with pixel template applies to the sample. Otherwise, NLM filter does not apply to the sample. The value of alf_nlm_threshold_1 shall be in the range of 0 to 216 – 1, inclusive.

alf_nlm_threshold_2 specifies the threshold to switch the pattern of NLM filter. alf_nlm_threshold_2 takes greater than alf_nlm_threshold_1, constantly. If the Activity is greater than alf_nlm_threshold_2, NLM filter with cross-shaped template applies to the sample. The value of alf_nlm_threshold_2 shall be in the range of 0 to 216 – 1, inclusive.

Table 1: Adaptive loop filter coding unit control parameter syntax.

| alf_cu_control_param() { | C | Descriptor |
| --- | --- | --- |
| if( adaptive_loop_filter_flag ) { |  |  |
| alf_cu_control_flag | 2 | u(1) |
| if( alf_cu_control_flag ) { |  |  |
| alf_cu_control_max_depth | 2 | ue(v) |
| alf_length_cu_control_info | 2 | se(v) |
| for( i = 0; i < NumAlfCuFlag; i++ ) |  |  |
| alf_cu_flag[ i ] | 2 | u(1) ¦ ae(v) |
| alf_nlm_coefficient | 2 | f(8) ¦ ae(v) |
| if( alf_nlm_coefficient ){ |  |  |
| alf_nlm_threshold_1 | 2 | f(16) ¦ ae(v) |
| alf_nlm_threshold_2 | 2 | f(16) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

# Decoding description

In this section, the decoding process of proposed in-loop filter is described. The difference between filter allocations of conventional ALF and proposed one set highlighted in yellow.

When interpreting the luma samples of the coding unit as to be filtered, depending on pcm_flag, pcm_loop_filter_disable_flag and alf_cu_control_flag, the following applies.

- If pcm_loop_filter_disable_flag is equal to 1 and pcm_flag is equal to 1, the luma samples are not filtered.

- Otherwise, if alf_cu_control_flag is equal to 0, the luma samples are filtered.

- Otherwise (alf_cu_control_flag is equal to 1), if AlfCuFlag[ xC ][ yC ] is equal to 1 where xC and yC are the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the luma samples are filtered.

- Otherwise (AlfCuFlag[ xC ][ yC ] is not equal to 1), if alf_nlm_coefficient is not equal to 0, where xC and yC are the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the luma samples are filtered by NLM filter.

- Otherwise (alf_nlm_coefficient is equal to 0), the luma samples are not filtered.

When alf_cu_control_flag is equal to 1, if alf_nlm_coefficient is not equal to 0, the derivation process for a half-resized padded sample array as specified in subclause 3.1 is invoked with a padded luma sample array s” as the input and the output is half-resized padded luma sample array s2”, and the derivation process for a filter strength for NLM filter as specified in subclause 3.2 is invoked with alf_nlm_coefficient as the input and the outputs are CoefNlmPel and CoefNlmCrs.

When alf_cu_control_flag is equal to 1, if AlfCuFlag[ xC ][ yC ] is not equal to 1, the derivation process for denoised block as specified in subclause 3.3 is invoked with the location (xC, yC) of a coding unit, the size of the coding unit, a padded luma sample s”, a half-resized padded luma sample s2”, and two filter strengths CoefNlmPel and CoefNlmCrs as the inputs and the output is the filtered reconstruction of luma picture recFiltPictureL.

## Derivation process for a half-resized padded sample array

Input to this process is padded luma sample array s”.

Output of this process is half-resized padded luma sample array s2”.

The sample values of the half-resized padded luma sample array s2” are derived by

x = x2 << 1
y = y2 << 1

s2”[ x2 ][ y2 ] = s”[ x ][ y ] + s”[ x + 1 ][ y ] + s”[ x ][ y + 1 ] + s”[ x + 1 ][ y + 1 ]		(3-1)

## Derivation process for a filter strength for NLM filter

Input to this process is alf_nlm_coefficient.

Outputs of this process are:

- CoefNlmPel derived by 681.5744  (alf_nlm_coefficient)2,

- CoefNlmCrs derived by 157.2864  (alf_nlm_coefficient)2.

## Derivation process for denoised block

Inputs to this process are:

- a location ( xC, yC ) specifying the top-left coordinate of a coding unit,

- the unit size of a coding unit,

- a padded luma sample array s”,

- a half-resized padded luma sample array s2”,

- two filter strengths CoefNlmPel and CoefNlmCrs.

Output of this process is the filtered reconstruction of luma picture recFiltPictureL.

The input coding unit is split into 2x2 sub-blocks. For each 2x2 sub-block the following steps apply:

- The derivation process for Activity as specified in subclause 3.3.1 is invoked with s2” and a location ( xS, yS ) specifying the top-left coordinate of the 2x2 sub-block as the inputs and the output is a local activity (Activity).

- If the Activity is less than or equal to alf_nlm_threshold_1, the luma samples of the sub-block are not filtered.

- Otherwise, the following steps apply:

- The derivation process for the index of an edge direction EdgeIdx as specified in subclause 3.3.2 is invoked with s2” and the location ( xS, yS ) as the inputs and the output is the EdgeIdx derived by sobel filtering.

- If the EdgeIdx is equal to 0 or Activity is less than or equal to nlm_threshold_2, for each sample ( xP, yP ) of 2x2 sub-block, the derivation process for a denoised value by NlmFlterPel as specified in subclause 3.3.3 is invoked with the location ( xP, yP ), s”, CoefNlmPel, and the EdgeIdx as the inputs and the output is recFiltPictureL[ xP ][ yP ].

- Otherwise, for each sample ( xP, yP ) of 2x2 sub-block, the derivation process for a denoised value by NlmFlterCrs as specified in subclause 3.3.4 is invoked with the location ( xP, yP ), s”, CoefNlmCrs, and the EdgeIdx as the inputs and the output is recFiltPictureL[ xP ][ yP ].

### Derivation process for local activity

Inputs to this process are:

- a half-resized padded luma sample array s2”,

- a location (xS, yS) specifying the top-left coordinate of a 2x2 sub-block.

Output of this process is a local activity (Activity).

Activity between a sample and neighboring samples as is derived by

x2 = xS >> 1
y2 = yS >> 1

Activity = 2 * (Abs(s2”[ x2 – 1 ][ y2    ] – s2”[ x2 ][ y2 ])
+ Abs(s2”[ x2 + 1 ][ y2    ] – s2”[ x2 ][ y2 ])
+ Abs(s2”[ x2    ][ y2 – 1 ] – s2”[ x2 ][ y2 ])
+ Abs(s2”[ x2    ][ y2 + 1 ] – s2”[ x2 ][ y2 ]))
+ Abs(s2”[ x2 – 1 ][ y2 – 1 ] – s2”[ x2 ][ y2 ])
+ Abs(s2”[ x2 + 1 ][ y2 – 1 ] – s2”[ x2 ][ y2 ])
+ Abs(s2”[ x2 – 1 ][ y2 + 1 ] – s2”[ x2 ][ y2 ])
+ Abs(s2”[ x2 + 1 ][ y2 + 1 ] – s2”[ x2 ][ y2 ])					(3-2)

### Derivation process for the index of edge direction

Inputs to this process are:

- a half-resized padded luma sample array s2”,

- a location ( xS, yS ) specifying the top-left coordinate of a 2x2 sub-block.

Output of this process is the index of edge direction (EdgeIdx) derived by sobel filtering. Figure 2 illustrates the relation between the EdgeIdx and edge direction.

For the derivation of an EdgeIdx, the following ordered steps apply:

- dx and dy are derived by

x2 = xS >> 1
y2 = yS >> 1

dx =  s2”[ x2 + 1 ][ y2 – 1 ] – s2”[ x2 – 1 ][ y2 – 1 ]
+  s2”[ x2 + 1 ][ y2 + 1 ] – s2”[ x2 – 1 ][ y2 + 1 ]
+ ((s2”[ x2 + 1 ][ y2    ] – s2”[ x2 – 1 ][ y2    ]) << 1) 				(3-3)

dy =  s2”[ x2 – 1 ][ y2 + 1 ] – s2”[ x2 – 1 ][ y2 – 1 ]
+  s2”[ x2 + 1 ][ y2 + 1 ] – s2”[ x2 + 1 ][ y2 – 1 ]
+ ((s2”[ x2    ][ y2 + 1 ] – s2”[ x2    ][ y2 – 1 ]) << 1) 				(3-4)

- If Abs(dx) + Abs(dy) is less than (3 << (BitDepth – 3)), EdgeIdx is assigned to 0.

- Otherwise, if dy is equal to 0, EdgeIdx is assigned to 6.

- Otherwise, a gradient (Grad) is assigned to dx  dy, and EdgeIdx is derived by

EdgeIdx =  6;Grad<−5.0 7;Grad<−2.0 8;Grad<−1.0 9;Grad<−0.510;Grad<−0.2 1;Grad< 0.2 2;Grad< 0.5 3;Grad< 1.0 4;Grad< 2.0 5;Grad< 5.0 6;otherwise								(3-5)

Figure 2: The mapping of the EdgeIdx according to edge direction.

### Derivation process for a denoised value by NlmFlterPel

Inputs to this process are:

- a location (xP, yP),

- a padded luma sample array s”,

- a filter strength CoefNlmPel,

- the index of edge direction EdgeIdx.

Output of this process is recFiltPictureL[ xP ][ yP ].

For the derivation of recFiltPictureL[ xP ][ yP ], the following steps apply:

- Initially, SumCoefs is assigned to 215 – 1, and SumSamples is assigned to (215 – 1) * s”[ xP ][ yP ].

- For each tap element (the relative location of tap element (xP2, yP2) and the location (xP, yP) are illustrated in Figure 3) related to the EdgeIdx, a SD (Square Difference) between s”[ xP ][ yP ] and s”[ xP2 ][ yP2 ] is derived. SumCoefs and SumSamples are derived as follows.

Coef = ExpLUT[Floor(Clip3(0, CoefNlmPel * SD, 999))]				(3-6)

SumCoefs   += Coef									(3-7)

SumSamples += Coef * s”[ xP2 ][ yP2 ] 						(3-8)

- The output is derived by

recFiltPictureL[ xP ][ yP ] = SumSamples + (SumCoefs >> 1) / SumCoefs			(3-9)

NOTE –ExpLUT[i] (i = 0..999) is assigned by

ExpLUT[i]= 0;i=999Floor((215− 1)*exp(−i÷142.85073)+0.5);otherwise		(3-10)

Figure 3: The mapping of the tap elements according to EdgeIdx

### Derivation process for a denoised value by NlmFlterCrs

Inputs to this process are:

- a location (xP, yP),

- a padded luma sample array s”,

- a filter strength CoefNlmPel,

- the index of edge direction EdgeIdx.

Output of this process is recFiltPictureL[ xP ][ yP ].

For the derivation of recFiltPictureL[ xP ][ yP ], the following steps apply:

- Initially, SumCoefs is assigned to 215 – 1, and SumSamples is assigned to (215 – 1) * s”[ xP ][ yP ].

- For each tap element related to the EdgeIdx (Figure 3), a SSD (Sum of Square Difference) between all the sample values of cross-shaped template around the location ( xP, yP ) and all the sample values of cross-shaped template around the location ( xP2, yP2 ) is derived (Figure 8). SumCoefs and SumSamples are derived as follows.

Coef = ExpLUT[Floor(Clip3(0, CoefNlmCrs * SSD, 999))]				(3-11)

SumCoefs   += Coef									(3-12)

SumSamples += Coef * s”[ xP2 ][ yP2 ] 						(3-13)

- The output is derived by

recFiltPictureL[ xP ][ yP ] = SumSamples + (SumCoefs >> 1) / SumCoefs			(3-14)

Figure 4: Derivation example for SSD between cross-shaped template around the location (xP, yP) and cross-shaped template around the location (xP2, yP2).

# Experimental results

In this section, objective/subjective evaluations are described.

## Objective evaluation

Table 2 presents coding efficiencies and computational complexities based on common condition [6] compared to HM4.0-r1354 anchors. The source code was compiled on 64-bit Windows 7 environment by using Microsoft Visual Studio 2010 Professional edition, and the experiments were performed on the cluster of PCs with 64-bit Windows 7, Intel Core i7 2600 (3.4GHz) and 16GB RAM.

Table 2: Experimental results for proposed ALF compared to the anchors.

|  | All Intra HE |  |  |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  | Y | U | V |
| Class A | -0.1% | 0.1% | 0.1% |  | Class A |  |  |  |
| Class B | -0.1% | 0.1% | 0.1% |  | Class B | -0.3% | 0.0% | 0.0% |
| Class C | -0.2% | 0.2% | 0.2% |  | Class C | -0.5% | 0.2% | 0.2% |
| Class D | -0.1% | 0.1% | 0.1% |  | Class D | -0.3% | 0.5% | 0.5% |
| Class E | -0.2% | 0.2% | 0.2% |  | Class E | -0.3% | 0.0% | 0.6% |
| Overall | -0.1% | 0.1% | 0.1% |  | Overall | -0.3% | 0.2% | 0.3% |
|  | -0.1% | 0.1% | 0.1% |  |  | -0.3% | 0.1% | 0.2% |
| Enc Time[%] | 112% |  |  |  | Enc Time[%] | 104% |  |  |
| Dec Time[%] | 104% |  |  |  | Dec Time[%] | 104% |  |  |
|  | Random Access HE |  |  |  |  | Low delay P HE |  |  |
|  | Y | U | V |  |  | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% |  | Class A |  |  |  |
| Class B | -0.3% | 0.1% | 0.2% |  | Class B | -0.5% | 0.0% | -0.1% |
| Class C | -0.4% | 0.1% | 0.2% |  | Class C | -0.6% | 0.2% | 0.1% |
| Class D | -0.1% | 0.2% | 0.1% |  | Class D | -0.4% | 0.2% | -0.4% |
| Class E |  |  |  |  | Class E | -0.7% | 0.1% | -0.1% |
| Overall | -0.2% | 0.1% | 0.1% |  | Overall | -0.5% | 0.1% | -0.1% |
|  | -0.2% | 0.1% | 0.1% |  |  | -0.5% | 0.0% | -0.2% |
| Enc Time[%] | 105% |  |  |  | Enc Time[%] | 107% |  |  |
| Dec Time[%] | 102% |  |  |  | Dec Time[%] | 105% |  |  |

Compared to the anchor of HM4.0, the maximum gain was –1.4% in Low Delay P for the sequence “BasketballDrive”.

## Subjective evaluation

Figure 5 and Figure 6 present the comparison between HM anchor and the proposed method from the viewpoint of subjective quality.

Figure 5: BasketballDrive (class A), LD-P, BD-Y: −1.4%, 
QP=37, POC #499, +128bits, PSNR-Y: +0.093dB, PSNR-U: −0.046dB, PSNR-V: −0.095dB

Figure 6: vidyo1 (class E), LD-P, BD-Y: −1.3%, 
QP=32, POC #594, +40bits, PSNR-Y: +0.117dB, PSNR-U: +0.013dB, PSNR-V: +0.001dB

# Conclusion

In this contribution, a CU-based adaptive loop filter with non-local means filter was proposed. Compared to the anchor of HM4.0, the average BD-rate gains were –0.1, –0.2, –0.3, and –0.5% for Intra, Random Access, Low Delay B, and Low Delay P, respectively. The average decoding time increased 2 to 5%. The maximum gain was –1.4% in Low Delay P for the sequence “BasketballDrive”.

Thanks to Intel’s and MediaTek’s cross-verification [6, 7] for confirming the coding results.

# References

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCTVC-F803_d5.doc ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.

- Masaaki Matsumura, Yukihiro Bandoh, Seishi Takamura, Hirohisa Jozawa, “In-loop filter based on non-local means filter (E206),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011.

- Masaaki Matsumura, Yukihiro Bandoh, Seishi Takamura, Hirohisa Jozawa, “Modifications of in-loop filter based on non-local means filter (F047),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.

- A. Buades, B. Coll, and J. M. Morel, “A Non Local Algorithm for Image Denoising,” Proc. IEEE Int. Conf. on Computer Vision and Pattern Recognition, vol. 2, pp. 60-65, June 2005.

- Frank Bossen, “Common test conditions and software reference configurations (F900),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.

- Y. Chiu, L. Xu, “CE8 Subtest h: Cross verification of NTT’s CU-based ALF with NLM filter (JCTVC-G235) by Intel (JCTVC-G299),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011.

- C.-Y. Tsai, Y.-W. Huang, “CE8.h.1: Crosscheck for NTT's CU-based ALF with non-local means filter in JCTVC-G235 (G482),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011.

# Patent rights declaration(s)

NTT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## Thresholds setting

The parameters alf_nlm_threshold_1 and alf_nlm_threshold_2 affect coding efficiency and computational complexities. When these parameters are assigned to small value, more noises of luma samples in CU are removed, since the number of luma samples to which NLM filter is applied increases (Figure 7),. Table 3 presents a simulation results with these parameters assigned to small values ignoring computational complexity.

Figure 7: Example of template allocation according to setting of thresholds

Table 3: Experimental results with small thresholds compared to the anchors.

|  | All Intra HE |  |  |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  | Y | U | V |
| Class A | -0.1% | 0.1% | 0.1% |  | Class A |  |  |  |
| Class B | -0.1% | 0.1% | 0.1% |  | Class B | -0.4% | -0.1% | 0.1% |
| Class C | -0.3% | 0.3% | 0.3% |  | Class C | -0.6% | 0.0% | 0.4% |
| Class D | -0.3% | 0.2% | 0.2% |  | Class D | -0.4% | 0.6% | -0.1% |
| Class E | -0.2% | 0.3% | 0.3% |  | Class E | -0.3% | 0.2% | 0.5% |
| Overall | -0.2% | 0.2% | 0.2% |  | Overall | -0.4% | 0.1% | 0.2% |
|  | -0.2% | 0.2% | 0.2% |  |  | -0.4% | 0.1% | 0.1% |
| Enc Time[%] | 116% |  |  |  | Enc Time[%] | 106% |  |  |
| Dec Time[%] | 107% |  |  |  | Dec Time[%] | 106% |  |  |
|  | Random Access HE |  |  |  |  | Low delay P HE |  |  |
|  | Y | U | V |  |  | Y | U | V |
| Class A | -0.2% | 0.0% | 0.0% |  | Class A |  |  |  |
| Class B | -0.4% | 0.1% | 0.2% |  | Class B | -0.6% | 0.2% | 0.3% |
| Class C | -0.5% | 0.2% | 0.1% |  | Class C | -0.9% | 0.2% | 0.1% |
| Class D | -0.4% | 0.2% | 0.1% |  | Class D | -0.7% | 0.4% | 0.1% |
| Class E |  |  |  |  | Class E | -0.7% | -0.1% | 0.4% |
| Overall | -0.4% | 0.1% | 0.1% |  | Overall | -0.7% | 0.2% | 0.2% |
|  | -0.4% | 0.1% | 0.1% |  |  | -0.7% | 0.2% | 0.2% |
| Enc Time[%] | 107% |  |  |  | Enc Time[%] | 109% |  |  |
| Dec Time[%] | 104% |  |  |  | Dec Time[%] | 109% |  |  |