| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G237 |
| --- | --- |

| Title: | CE12 Subtest 5: Cross-verification of Panasonic’s proposal JCTVC-Gxxx |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shuo Lu Masaru Ikeda Teruhiko Suzuki Sony City Osaki, 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2820 Shuo.Lu@jp.sony.com Masaru.Ikeda@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross verification results of Panasonic’s proposal on deblocking filter using adaptive weighting factors (JCTVC-Gxxx[1]). The source code was provided by Panasonic, and the performance has been evaluated under common test conditions as defined in JCTVC-F900[2]. No mismatch was found comparing to the results reported by Panasonic. Detailed results can be found in the attached excel file.

# Introduction

Core Experiment 12 is aiming at improving deblocking filtering techniques in HEVC(JCTVC-F912[3]). The proposal in JCTVC-F405 belongs to the subtest 5 of CE 12, in which weighting factors are introduced into the deblocking filter.

The BD-rate, the encoding/decoding time, and subjective evaluation are shown as below.

# Results

## Test condition

The experiments are executed based on HM4.0. The configurations and common test conditions specified in JCTVC-F900[2] are used. Both encoding and decoding time is measured on the same machine as anchor, and decoding time is gathered without outputting decoded YUV data.

## BD-rate and encoding/decoding time

### Results of basic version proposal

Summary of the results generated with the proposalJCTVC-F405 [1] are shown in the following tables in terms of BD-rate. Complete data can be found in the attached excel file.

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% |
| Class B | -0.6% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% |
| Class C | -0.3% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Class D | -0.2% | 0.1% | 0.1% | -0.2% | 0.0% | 0.0% |
| Class E | -0.3% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| All | -0.4% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table2-1: Results for Intra

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4% | 0.0% | -0.2% | -0.2% | 0.1% | 0.0% |
| Class B | -0.4% | 0.0% | 0.0% | -0.2% | 0.0% | 0.1% |
| Class C | -0.3% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Class D | -0.2% | 0.1% | -0.1% | -0.1% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| All | -0.3% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table2-2: Results for RandomAccess

|  | Low delay |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | -0.2% | -0.3% | -0.3% | 0.0% | 0.1% |
| Class C | -0.4% | -0.3% | -0.1% | -0.2% | 0.0% | -0.3% |
| Class D | -0.3% | 0.2% | 0.1% | -0.2% | 0.1% | -0.1% |
| Class E | -0.6% | 0.3% | 0.7% | -0.3% | -0.6% | 0.1% |
| All | -0.5% | 0.0% | 0.0% | -0.2% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table2-3: Results for LowDelay

### Results of modified proposal

In addition, an easy subjective criterion in the estimation process is introduced into this proposal (JCTVC-F405 [1]), which is enabled in the encoder by the macro:

#define PANASONIC_ADAPTIVE_DEBLOCKING_SUBJECTIVE_ESTIMATION         1

Summary of results generated with the proposal in combination with introduced estimation method are shown in the following tables in terms of BD-rate. Complete data can be found in the attached excel file.

TBD.

## Subjective quality

TBD

# Conclusion

In this contribution, the cross verified results of deblocking filter proposed by Panasonic are reported. It is confirmed that the results match perfectly with those reported by Panasonic.

# References

[1] JCT-VC, “Deblocking filter using adaptive weighting factor”, JCTVC-Gxxx, 7th. JCT-VC Meeting, Geneva, CH, Nov. 2011

[2]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900, 6th. JCT-VC Meeting, Torino, IT, July 2011

[3]JCT-VC, “Description of Core Experiment 12: Deblocking Filtering”, JCTVC-F912, 6th. JCT-VC Meeting, Torino, IT, July 2011

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).