- Working draft for the proposed modification

- 8.4.2.1.1 Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction in subclause  is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol, the scaled flag scaleFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col                                                           (8-1)

predFlagLXCol = availableFlagLXCol                                                                                                (8-2)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1 and if scaleFlagLXCol is equal to 0

- When merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList and the variable maxNumMergeCand is set to 5.

- When slice_type is equal to B, the following applies.

  - The derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

  - The derivation process for non-scaled bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand, numOrigMergeCand and maxNumMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0nscaleCandl and refIdxL1nscaleCandl, the prediction list utilization flags predFlagL0nscaleCandl and predFlagL1nscaleCandl, the motion vectors mvL0nscaleCandl and mvL1nscaleCandl of every new candidate nscaleCandl being added in mergeCandList. The number of candidates being added numNscaleMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand ). When numNscaleMergeCand is greater than 0, l ranges from 0 to numNscaleMergeCand – 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, the NumMergeCand and the MaxNumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand – 1, inclusive.

- if availableFlagCol is equal to 1 and if scaleFlagLXCol is equal to 1 and if  the Col candidate has not the same motion vectors and not the same reference indices as all candidates in the list mergeCandList, all candidates from position numOrigMergeCand to position numMergeCandare are incremented in the list. numMergeCand is set equal to numMergeCand +1. The Col candidate is inserted in the list at numOrigMergeCand position.

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]								(8-88)

mvLX[ 1 ] = mvLXN[ 1 ]	           							(8-89)

refIdxLX = refIdxLXN								(8-90)

predFlagLX = predFlagLXN							(8-91)

- Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

- the scaled flag scaleFlagLXCol

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8-3)

yPRb = yP + nPSH	(8-4)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1	(8-5)

yPCtr = ( yP + ( nPSH >> 1 ) – 1	(8-6)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvCol and refIdxCol are derived as follows,

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following assignments are made.

      - If all PicOrderCnts of pictures in all reference picture lists are smaller than PicOrderCnt( currPic ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of RefIdxLX.

      - Otherwise (one of PicOrderCnt of pictures in all reference picture lists are larger than PicOrderCnt( currPic ) ), the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvLN[ xPCol ][ yPCol ], RefIdxColLN and LN, respectively, with N being value of collocated_from_l0_flag.

and the variable availableFlagLXCol is set equal to 1, and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ), the variable scaleFlagLXCol is set equal to 0,

mvLXCol = mvCol	(8-7)

  - Otherwise, the variable scaleFlagLXCol is set equal to 1, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-8)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-9)

mvLXCol = ClipMv( Sign( DistScaleFactor * mvLXA ) * 			
				( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8-10)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )	(8-11)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-12)