| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G241 |
| --- | --- |

| Title: | Non-CE9: On parallel derivation of the temporal predictor for Merge/Skip modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume LAROCHE Tangi POIRIER Patrice ONNO Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution presents a simplification of motion vector derivation process for the Merge/Skip modes. The aim of the proposed scheme is to reduce the number of cycles in hardware needed for this derivation process for the worst case. The proposed scheme consists in scaling the temporal candidates in parallel to the cascaded predictors and related pruning process. Based on the complexity analysis from Texas Instruments in JCTVC-F088 and JCTVC-F068, it is reported that 3 cycles are saved for the Merge/Skip mode derivation process for the worst case hardware implementation with less than 0.1% BDR loss.

# Introduction

The complexity related to the worst case of the HM4.0 Merge/Skip motion vector predictors derivation has significantly increased compared to the HM3.0. This comes from a cascade of motion vector predictors derivation and from an increase of the amount of comparisons. Figure 1 shows the diagram of the motion vector predictor derivation process for the Merge/Skip modes of the current HM4.0. In this figure, an estimation of the number of cycles for each step of the derivation process is reported. This estimation is based on the complexity analysis of JCTVC-F068 [1] and JCTVC-F088 [2]. Of course, these numbers of cycles depend on the hardware architecture.

In JCTVC-F068 [1], it was mentioned that on a specific hardware architecture (266 MHz frequency, 45 nm technology) the availability check consumes 1 cycle, the MVP scaling consumes 3 cycles and the list pruning needs one 1 cycle when 5 predictors are used. With 5 predictors in the list, 10 comparisons are needed in the worst case. When the number of comparisons increases, more cycles are needed.

Error: Reference source not found gives, for the worst case, the number of comparisons and the number of cycles needed for each step of Figure 1. For the spatial checking, one comparison for each possible spatial predictor is needed for a non 2Nx2N Merge mode. Indeed, for the second PUs, the “avoid Merge check” compares the value of each spatial candidate to the value of the selected candidates of the first PU. In the current implementation, 5 spatial predictors are checked. For the temporal candidates, both H and the centered predictor are checked. All these steps can be processed in parallel as explained in JCTVC-F088 [2]. Therefore, only one cycle is needed. In the worst case, a scaling is applied on the temporal predictor. This step 3 consumes 3 cycles as explained in [2]. When all spatial and temporal predictors are available, the first pruning process on a list of 5 candidates is applied. This step 4 needs 10 comparisons in the worst case and consumes one cycle (see [2]). Then, the additional cascade predictors are generated (steps 5, 6, 7). In step 5, 5 combined candidates are generated based on the previous non redundant candidates. In this step, 12 possible bidirectional predictors are generated in the worst case. For each predictor, the motion information of L0 and L1 are compared in order to eliminate the predictors which have exactly the same motion information in both lists. Consequently, 12 comparisons are needed. This step can take 1 to 2 cycles. In the same way, the scaled candidate is derived from candidates of the first list by scaling the motion vector from the first list to the second list. The related scaling only inverses the vector value. Consequently, step 6 takes only one cycle. The zero motion predictors do not need any specific operation. Then, a second pruning process is applied. This process needs 32 comparisons in the worst case. This happens when the 8 possible candidates are compared to 4 predictors in the first list. This step 8 needs 2 cycles. Consequently, we can consider that the full Merge MVP derivation process needs 8 cycles.

1

2

5

3

4

Scale

1

2

5

3

4

Final MVP pruning

1

5

1st MVP pruning

1

2

3

4

5

MVP combinations

1

2

4

3

1 cycle

3 cycles

1 cycle

2 cycles

1 cycle

Total

8 cycles

Scaled MVP

5

6

Zero MVP

7

8

1

2

4

3

5

6

7

8

3

1

2

4

5

6

7

8

Figure 1 Diagram of HM4.0 Merge/Skip MVP derivation process.

Table 1 Number of comparisons and related number of cycles for each step of the Merge/Skip mode MVP derivation process for the worst case.

| Step number | Step name | Maximum number of comparisons | Number of cycles |
| --- | --- | --- | --- |
| 1 | Spatial Checking | 5 | 1 |
| 2 | temporal checking | 0 | >1 |
| 3 | TMVP scaling | 0 | 3 |
| 4 | 1st MVP pruning | 10 | 1 |
| 5 | MVP combinations | 12 | 1 |
| 6 | Scaled MVP | 0 | 1 |
| 7 | Zero MVP | 0 | >1 |
| 8 | Final pruning | 32 | 2 |

In this contribution, we propose a parallel derivation of the temporal motion vector predictor scaling in order to reduce the throughput of the Merge MVP derivation process.

# Proposed scheme

The proposed simplification consists in scaling the temporal predictor in parallel to the cascade of candidate derivation and related pruning process. As a consequence, the Merge/Skip modes MVP derivation process needs only 5 cycles for the worst case instead of 8 in the current HM4.0.

Figure 2 is the diagram corresponding to the proposed scheme. As in the current Merge/Skip mode derivation process, the spatial and temporal predictors are checked. If the temporal predictor does not need any scaling, the first pruning process is applied in the same way as in HM4.0. In that case, the same motion vector candidate list is generated as in HM4.0. Otherwise, the temporal predictor is scaled and this scaling is processed in parallel to the pruning processes and to the cascade of MV derivations. So, the temporal predictor is not tested in the first pruning process. The combined and scaled predictors are generated and a second pruning process is applied. This second pruning process only compares the combined predictors. Eventually, the final pruning process is applied on the last three cascade candidates and on the temporal predictors. In addition, in this final pruning process, if the temporal predictor is not a duplicate compared to the candidates already in the list, it is added after spatial predictors and before combined predictors (Its position after the first pruning process) .This corresponds to the same position as the temporal candidate would be if it was not scaled.  If the temporal predictor is a duplicate predictor, the derivation process is ended. As depicted in Figure 2, only 5 cycles are consumed for the worst case with the proposed scheme.

Please note that when the temporal predictor needs to be scaled, the first pruning process (step 4) needs only 6 comparisons. Moreover, the checking of the temporal predictor at the final pruning process needs 5 comparisons in the worst case (compared to a list which contains at most 5 predictors). So the proposed scheme requires only 1 additional comparison. The combination of the second and the final pruning processes (step 8 and 9) is the same as the final pruning process (step 9) of Figure 1, except that the temporal predictor is tested. So, each pruning process consumes 1 cycle if we consider that the final pruning of Figure 1 consumes 2 cycles.

Final Pruning

1

2

5

3

4

Scale

1

2

5

3

4

2d  MVP pruning

1

2

1st MVP pruning

1

2

3

4

MVP combinations

1

2

4

3

1 cycle

1 cycle

1 cycle

Total

5 cycles

Scaled MVP

5

6

Zero MVP

7

8

1

2

4

3

5

6

7

8

3

1

2

4

5

6

7

8

1

2

3

1 cycle

1 cycle

9

Figure 2 Diagram of the proposed scheme.

# Experimental results

The experiments were conducted in accordance with the 6 usual Inter configurations using the common conditions [3]. Please note that the presented results include the encoder bug fix on the Merge mode selection process ENC_MRG_FIX. This encoder bug fix improves of the current HM4.0 of 0.1%.

Error: Reference source not found provides the results of the simplification + ENC_MRG_FIX over the HM4.0 Anchors + ENC_MRG_FIX. The complete results are presented in the attached Excel “Results_HM4.0+bugfix_Anchor_Tempo_parallel+bugfix.xls” Only 0.1% loss is observed for the RAHE, RALC and LDLC and 0.2% for LDHE. For P configuration LDPHE and LDPLC, the proposed scheme doesn’t impact the coding efficiency (0.0%).

Table 2 Simulation results of the proposed scheme + ENC_MRG_FIX over HM4.0 Anchors + ENC_MGR_FIX

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class B | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% |
|  | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | -0.1% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.4% |
| Class D | 0.2% | 0.2% | 0.0% | 0.2% | 0.2% | 0.2% |
| Class E | 0.2% | 0.1% | 0.1% | 0.0% | 0.3% | 0.1% |
| Overall | 0.2% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% |
|  | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 97% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% |
| Class D | 0.0% | 0.3% | -0.2% | 0.0% | 0.0% | 0.1% |
| Class E | 0.1% | 0.1% | 0.4% | 0.0% | -0.2% | -0.1% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

The same result is observed when the simplification is compared to HM4.0 Anchors, as reported in the attached Excel “Results_HM4.0_Anchor_Tempo_parallel.xls”. Moreover, when the simplification + ENC_MRG_FIX is compared to the HM4.0 Anchors coding gains are observed for LDHE and LDPHE and no loss for other configurations, as reported in the attached Excel “Results_HM4.0_Anchor_Tempo_parallel+bugfix.xls”.

# Conclusion and recommendations

This contribution presents a simplification of the Merge mode motion vector derivation process. The aim of this simplification is to reduce the throughput of the current HM4.0 MVP derivation process. The proposed method consists in scaling the temporal predictor in parallel to the cascade of candidate derivation and related pruning process. The simplification has a minor impact on the coding efficiency with a loss of 0.09% in average for all Inter configurations. This simplification saves 3 cycles for hardware implementation as explained in JCTVC-F068. Consequently, only 5 cycles are needed for the current Merge mode derivation process compared to the 8 cycles of the current HM4.0.

According to the minor impact on the coding efficiency and the worst case reduction for hardware implementation, it is recommended to adopt this simplification into the HEVC working draft.

# References

- M. Zhou, V. Sze, Y. Matsuba, “A study on HM3.0 parsing throughput issue”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F068, 6th Meeting: Torino, Italy, July, 2011.

- M. Zhou, M. Sinangil, V. Sze, S.Park, J. Park, B. Jeon, “CE9: Simplified AMVP design (SP06S1, SP06S2)”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F088, 6th Meeting: Torino, Italy, July, 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, Italy, July, 2011.

# Working draft

Under editing

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).