# Draft Changes of proposal JCTVC-G243

### Prediction unit syntax

| prediction_unit( x0, y0 , log2CUSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2CUSize >= Log2MinIPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| else if (log2PUWidth != 6) { |  |
| mprm_pred_flag [ x0 ][ y0 ] | u(1) ¦ ae(1) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| } else { |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| } |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### Prediction unit semantics

mvp_idx_l0[ x0 ][ y0 ] specifies the motion vector predictor index of list 0 where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When mvp_idx_l0[ x0 ][ y0 ] is not present, it shall be inferred to be equal to 0.

mvp_idx_l1[ x0 ][ y0 ] has the same semantics as mvp_idx_l0, with l0 and list 0 replaced by l1 and list 1, respectively.

mvp_idx_lc[ x0 ][ y0 ] has the same semantics as mvp_idx_l0, with l0 and list 0 replaced by lc and list combination, respectively.

prev_intra_luma_pred_flag[ x0 ][ y0 ], mpm_idx[ x0 ][ y0 ], mprm_pred_flag [ x0 ][ y0 ] and rem_intra_luma_pred_mode[ x0 ][ y0 ] specify the intra prediction mode for luma samples. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When prev_intra_luma_pred_flag[ x0 ][ y0 ] is equal to 1, the intra prediction mode is inferred from a neighbouring intra-predicted prediction unit according to subclause 8. When mprm_pred_flag [ x0 ][ y0 ] is equal to 1, the intra prediction mode is derived from a neighbouring intra-predicted prediction unit according to subclause 8.

### Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2PUSize specifying the size of the current prediction unit,

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

–	a variable array intraModeTable

–	a variable array intraModeCounterTable

–	a variable array intraRankTable

Output of this process is the variable IntraPredMode[ xB ][ yB ], value-updated intraModeTable, value-updated intraRankTable and value-updated intraModeCounterTable.

Table 8-1 specifies the value for the intra prediction mode and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Planar |
| 1 | Intra_Vertical |
| 2 | Intra_Horizontal |
| 3 | Intra_DC |
| Otherwise (4..34) | Intra_Angular |
| 35 | Intra_FromLuma (used only for chroma) |

Table 8-2 specifies the number of luma intra prediction modes intraPredModeNum depending on log2PUSize.

Table 8-2 – Specification of intraPredModeNum

| log2PUSize | intraPredModeNum |
| --- | --- |
| 2 | 19 |
| 3 | 35 |
| 4 | 35 |
| 5 | 35 |
| 6 | 4 |

Table 8-3 specifies the mapping table used for converting the number of intra prediction modes.

Table 8-3 – Specification of mapIntraPredMode4 and mapIntraPredMode10

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mapIntraPredMode4[ value ] | 0 | 1 | 2 | 3 | 3 | 3 | 1 | 3 | 2 | 2 | 3 | 3 | 1 | 1 | 3 | 3 | 2 |  |
| mapIntraPredMode10[ value ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |  |
| value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| mapIntraPredMode4[ value ] | 2 | 3 | 3 | 3 | 3 | 1 | 1 | 1 | 3 | 3 | 3 | 3 | 2 | 2 | 2 | 2 | 3 | 3 |
| mapIntraPredMode10[ value ] | 3 | 3 | 4 | 5 | 5 | 1 | 1 | 6 | 6 | 7 | 4 | 8 | 8 | 2 | 2 | 9 | 9 | 3 |

IntraPredMode[ xB ][ yB ] labelled 0, 1, 2, .., 34 represents directions of predictions as illustrated in Figure 8-1.

Figure 8-1 – Intra prediction mode directions (informative)

Table 8-x – Specification of mprmTable

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mprmTable[0][value] | 2 | 13 | 16 | 2 | 11 | 12 | 14 | 10 | 15 | 17 | 18 | 5 | 1 | 6 | 7 | 4 | 8 | 2 | 9 |
| mprmTable[1][value] | 2 | 23 | 30 | 2 | 19 | 21 | 25 | 10 | 28 | 32 | 34 | 20 | 22 | 24 | 26 | 27 | 29 | 31 | 33 |

| value | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mprmTable[0][value] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| mprmTable[1][value] | 11 | 5 | 12 | 1 | 13 | 6 | 14 | 7 | 4 | 15 | 8 | 16 | 2 | 17 | 9 | 18 |

IntraPredMode[ xB ][ yB ] is derived as the following ordered steps. [Ed. (WJ): proponent suggests to move this part to the syntax since the other syntax elements utilize IntraPredMode. But it seems too complex to move all the following process to the syntax table. Maybe it’s better to move this part to the semantics section or simply avoid the use of IntraPredMode to parse the syntax item]

- The derivation process for neighbouring treeblocks specified in subclause XXX with ( xB,  yB ) given as input and the output is assigned to tbAddrA and tbAddrB specifying the treeblock addresses of treeblocks covering ( xBA,  yBA ) and ( xBB, yBB ) respectively where ( xBA,  yBA ) is set equal to ( xB-1,  yB ) and ( xBB,  yBB ) is set equal to ( xB,  yB-1 ).

- For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If the treeblock with address tbAddrN is not available, intraPredModeN is set equal to -1

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to Intra_DC,

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

- For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

- If intraPredModeN is greater than or equal to intraPredModeNum

- If intraPredModeNum is equal to 4 then candIntraPredModeN is set equal to mapIntraPredMode4[ intraPredModeN ]

- Otherwise candIntraPredModeN is set equal to mapIntraPredMode10[ intraPredModeN ].

- Otherwise, candIntraPredModeN is set equal to intraPredModeN

- The candModeList[x] and NumMPMCand are derived as follows:

- NumMPMCand is set equal to 2

- If both candIntraPredModeN are not available, then the value 3 is assigned to candModeList[ 0 ] and the value 0 is assigned to candModeList[ 1 ]

- Otherwise, if only one candIntraPredModeN is available or if both candIntraPredModeN are the same, candModeList[ x ] is derived by applying the following procedure:

  - candIntraPredModeN is assigned to candModeList[ 0 ]

  - If candIntraPredModeN is not equal to 0, the value 0 is assigned to candModeList[ 1 ]

  - Otherwise, the value 3 is assigned to candModeList[ 1 ]

- Otherwise, both candIntraPredModeN are assigned to the candidate modes list with the smaller of the two candidates at candModeList[ 0 ] and the larger at candModeList[ 1 ].

- MPRM is derived as follows:

- If none of candModeList[ 0 ] and candModeList[ 1 ] is equal to 0, MPRM is set equal to 0

- Otherwise if candModeList[ 0 ] or candModeList[ 1 ] is equal to 3, MPRM is set equal to 1

- Otherwise, the following applies.

  - if candModeList[ 1 ] is equal to 0, angularMPM is set equal to candModeList[ 0 ], otherwise, angularMPM is set equal to candModeList[ 1 ]

  - if log2PUSize is equal to 2, k is set equal to 0, otherwise k is set equal to 1

  - MPRM is set equal to mprmTable[ k ][ angularMPM ]

- IntraPredMode[ xB ][ yB ] is derived by applying the following procedure:

- If prev_intra_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[mpm_idx[ xB ][ yB ]]

- Otherwise if mprm_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  MPRM[ xB ][ yB ]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following ordered steps:

  - IntraPredMode[ xB ][ yB ] = rem_intra_luma_pred_mode

  - If log2PUSize is not equal to 6, the following applies:

    - for x=0…1, candModeList[ x ] = intraRankTable[ k ] [ candModeList[ x ] ]

    - MPRM = intraRankTable[ k ] [ MPRM ]

- The candMPMAllList[x] and NumMPMAllCand are derived as follows:

  - The value of 3 is assigned to NumMPMAllCand.

  - The modes in candModeList [0], candModeList [1] and MPRM are assigned to candMPMTmpList[x], x = 0…2

  - If log2PUSize is not equal to 6, the following applies:

    - For x=0..2, candMPMTmpList[ x ] is modified by the following pseudo-code:

      - rank = 0

      - while( intraModeTable[ k ][ rank ] ! = candMPMTmpList[ x ] )  rank ++

      - candMPMTmpList[ x ] = rank

  - The available modes in candMPMAllTmpList[x] are sorted in increasing order and are assigned to candMPMAllList[x], with x = 0…2

- for (cIdx = 0; cIdx < 2; cIdx++) {

if (IntraPredMode[ xB ][ yB ]>= candMPMAllList [cIdx]) IntraPredMode[ xB ][ yB ]++}

- If log2PUSize is not equal to 6, IntraPredMode[ xB ][ yB ] is set equal to intraModeTable[ k ][ IntraPredMode[ xB ][ yB ] ].

  - If candModeList[ 0 ] is greater than candModeList[ 1 ], then two values are swapped. [Ed. (WJ): may be better expression needed]

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 0 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

  - When IntraPredMode[ xB ][ yB ] is equal or greater than candModeList[ 1 ], the value of IntraPredMode[ xB ][ yB ] is increased by one

- If log2PUSize is not equal to 6 the following procedure applies:

- If entropy_coding_mode_flag is equal to 0, the variable array intraModeTable[ k ] is updated by invoking process in subclause Error: Reference source not found with intraModeTable[ k ], rem_intra_luma_pred_mode and 0 as inputs.

- Otherwise (entropy_coding_mode_flag is equal to 1), the variable array intraModeTable[ k ] is updated by invoking process in subclause Error: Reference source not found with intraModeTable[ k ], rem_intra_luma_pred_mode, intraPredModeNum,  intraModeCounterTable[ k ] and 0 as inputs.

### Parsing process for VLC codes

In tables 9-4 and 9-5, codeword is the concatenation of a prefix code and a suffix code. The prefix code depends on the codeNum range. The suffix code is generated using FL binarization by invoking section 9.3.2.6 with cMax as input, where cMax depends on the codeNum range.

Note: the following tables correspond to the 2 MPMs configuration

Table 9-4 – codeNum and codeword with vlcNum equal to 20

| codeNum range | prefix | suffix | Codeword |
| --- | --- | --- | --- |
| 0-3 | 00 | xx - FL, cMax=4 | 00xx |
| 4-7 | 010 | xx - FL, cMax=4 | 010xx |
| 8-14 | 011 | xxx - FL, cMax=8 | 011xxx |
| 15-16 | 011111 | x - FL, cMax=2 | 011111x |

Table 9-5 – codeNum and codeword with vlcNum equal to 21

| codeNum range | prefix | suffix | Codeword |
| --- | --- | --- | --- |
| 0-3 | 00 | xx - FL, cMax=4 | 00xx |
| 4-11 | 010 | xxx - FL, cMax=8 | 010xxx |
| 12-19 | 0110 | xxx - FL, cMax=8 | 0110xxx |
| 20-32 | 0111 | xxxx - FL, cMax=16 | 0111xxxx |

Note:

- the following tables correspond to the 2 MPMs + 1 MPRM configuration

- More regular tables could be derived, as illustrated in JCTVC-G243

Table 9-6 – codeNum and codeword with vlcNum equal to 20

| codeNum range | prefix | suffix | Codeword |
| --- | --- | --- | --- |
| 0-1 | 010 | x - FL, cMax=2 | 010x |
| 2-3 | 0010 | x - FL, cMax=2 | 0010x |
| 4-7 | 0100 | xx - FL, cMax=4 | 0100xx |
| 8-15 | 0000 | xx - FL, cMax=4 | 0000xx |

Table 9-7 – codeNum and codeword with vlcNum equal to 21

| codeNum range | prefix | suffix | Codeword |
| --- | --- | --- | --- |
| 0-1 | 0101 | x - FL, cMax=2 | 0101x |
| 2-3 | 0000 | x - FL, cMax=2 | 0000x |
| 4-7 | 0001 | xx - FL, cMax=4 | 0001xx |
| 8 | 001000 |  |  |
| 9-10 | 001001 | x - FL, cMax=2 | 001001x |
| 11-14 | 00101 | xx - FL, cMax=4 | 00101xx |
| 15-30 | 0011 | xxxx - FL, cMax=16 | 0011xxxx |

#### Initialisation process for intraModeTable and intraModeCounterTable

Outputs of this process are initial values of the variable array intraModeTable.

The variable array intraModeTable is initialized as follows specified in Table 9-8.

–	For idx=0..18, intraModeTable[ 0 ][ idx ] is set equal to idx.

–	For idx=0..34, intraModeTable[ 1 ][ idx ] is set equal to idx.

Table 9-8 – Specification of intraModeTable[k][codeNum]

|  | - codeNum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - k | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 | - 16 |
| - 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| - 1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |

|  | - codeNum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - k | - 17 | - 18 | - 19 | - 20 | - 21 | - 22 | - 23 | - 24 | - 25 | - 26 | - 27 | - 28 | - 29 | - 30 | - 31 | - 32 | - 33 | - 34 |
| - 0 | 17 | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - 1 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |

### Codeword index mapping update process

Inputs to this process are a VLC codeword index mapping table indexMappingTable that provides the mapping between VLC codeword indexes and syntax element values, a codeword index value codeNum, and a value counterNum that specifies the number of counters used. If counterNum is not equal to 0, inputs additionally include an array counterArray indexed from 0 to (counterNum-1) that saves the value of counters, a value counterSum that saves the number of all counter increments since its previous reset.

Outputs of this process are indexMappingTable, and if counterNum is not equal to 0 counterArray and counterSum with updated values.

The process is specified in the following ordered steps:

- Syntax element value syntaxVal is set equal to indexMappingTable [codeNum].

- If codeNum is not less than 1, codeNumPre is set equal to (codeNum-1). Otherwise, codeNumPre is set equal to codeNum.  Syntax element value syntaxValPre is set equal to indexMappingTable [codeNumPre].

- If codeNum is greater or equal to counterNum, the values of indexMappingTable[codeNumPre] and indexMappingTable[codeNum] are exchanged with each other. No further step is carried out.

- Otherwise the following procedure applies:

  - When counterArray[codeNum] is equal to 255, for i = 0…(counterNum-1),  counterArray[i] is set equal to (counterArray[i]>>1).

  - The value of counterArray[codeNum] is increased by 1.

  - If counterArray[codeNum] >= counterArray[codeNumPre], the counter values of counterArray[codeNum] and counterArray[codeNumPre] are swapped, and the values of indexMappingTable [codeNumPre] and indexMappingTable [codeNum] are also swapped.

  - If counterSum is not less than 15, the value of counterSum is reset to 0. For i = 0…(counterNum-1),  counterArray[i] is set equal to  (counterArray[i]>>1).

  - Otherwise, the value of counterSum is increased by 1.

#### 9.2.4.6		Parsing process for rem_intra_luma_pred_mode

This process is invoked when entropy_coding_mode_flag is equal to 0 for parsing syntax element rem_intra_luma_pred_mode in subclause 2.

Inputs to this process are bits from slice data, a variable log2CUSize specifying the size of the current coding unit and a variable array intraModeTable.

Outputs of this process are the syntax element rem_intra_luma_pred_mode and value-updated intraModeTable.

The variable log2PUSize is derived as follows.

–	If PartMode is equal to PART_NxN, log2PUSize is set equal to log2CUSize – 1,

–	Otherwise, log2PUSize is set equal to log2CUSize.

The value of rem_intra_luma_pred_mode is derived as follows.

–	Based on log2PUSize, the value of a variable intraPredModeNum indicating the number of intra prediction modes for the given size of prediction unit is obtained according to Table 8-2.

–	If intraPredModeNum is equal to 4, read one bit and assign its value to rem_intra_luma_pred_mode. No further step is carried out. Othewise,

–	The values of variable k and vlcNum areis obtained as follows

  - If intraPredModeNum is equal to 1819, set vlcNum and k equal to 20 and 0, respectively.

  - If intraPredModeNum is equal to 35, set vlcNum and k equal to 21 and 1, respectively.

  - The parsing process described in subclause 9 is invoked with vlcNum as input and the variable codeNumrem_intra_luma_pred_mode as output.

  - The value of rem_intra_luma_pred_mode is set equal to intraModeTable[k][codeNum].

  - The value of a variable counterNum is set to 0. The variable array intraModeTable[k] is updated by invoking process in subclause Error: Reference source not found with intraModeTable[k], codeNum and counterNum as inputs.

### 9.3.2 Binarization process

| Table 9-9 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| prev_intra_luma_pred_flag | I | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |
| mprm_pred_flag | all | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
| rem_intra_luma_pred_mode | I | as specified in subclause 9.3.2.9 | 0 | Error: Reference source not found | 0 |
|  | P |  | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 2 |

#### Binarization process for rem_intra_luma_pred_mode

Input to this process is a request for the syntax element rem_intra_luma_pred_mode and a variable log2PUSize specifying the size of the current prediction unit.  cNumBins.

Output of this process is the binarization of the syntax element.

The binarization for rem_intra_luma_pred_mode is performed as follows:given by Table 9-1048.

- If log2PUSize is equal to 6, FL binarization is performed by invoking section 9.3.2.6 with cMax=2 as input.

- Otherwise if log2PUSize is greater than 2, bin string is derived as follows:

  - if log2PUSize is greater than 2, codeword is derived from table 9-5 with codeNum set to rem_intra_luma_pred_mode

  - otherwise (log2PUSize is equal to 2) codeword is derived from table 9-6 with codeNum set to rem_intra_luma_pred_mode

  - bin string corresponds to codeword bins ordered in reverse order

Table 9-10 – Binarization for rem_intra_luma_pred_mode

| Value of rem_intra_luma_pred_mode | Bin string |
| --- | --- |
| less than 32 | FL, cMax = cNumBins |
| 32 | 111110 |
| 33 | 111111 |