| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G243 |
| --- | --- |

| Title: | CE6b: Intra mode coding with 4 MPMs and mode ranking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Francois, S. Pautet (Canon), Joonyoung Park, Byeongmoon Jeon (LG), Tzu-Der Chuang, Ching-Yeh Chen, Mei Guo, Xun Guo, Yu-Wen Huang, Shawmin Lei (MediaTek), Wei-Jung Chien (Qualcomm), Ehsan Maani, Ali Tabatabai (Sony) | Tel: Email: | edouard.francois@crf.canon.fr jy.park@lge.com peter.chuang@mediatek.com wchien@qualcomm.com ali.tabatabai@am.sony.com |
| Source: | Canon, LG, MediaTek, Qualcomm, Sony |  |  |

_____________________________

# Abstract

This contribution proposes an intra mode coding solution combining proposals JCTVC-F062 and JCTVC-F459 relating to the use of additional MPMs, and JCTVC-F091, JCTVC-F106 and JCTVC-F269 relating to the use of mode ranking for remaining mode coding.

This combined solution uses 4 MPMs (2 usual MPMs + 2 MPRMs) and mode ranking for coding remaining modes bins. The same VLC and binarization tables are unified for LC and HE configurations. In HE, bypass coding is used instead of CABAC for the MPRMs and remaining modes bins. Chroma coding is also modified by considering in the candidate sets the 4 MPMs from luma. Finally, a 19th mode is added to the set of 4x4 block modes to get a more coherent set of modes.

Average BD-rate gains of 0.5% Y, 0.4% U, 0.5% V for AIHE configuration, and of 0.4% Y, 0.7% U, 0.7% V for AILC configuration, are reported. Encoding and decoding runtime are reportedly not significantly impacted.

# Introduction

CE6b explores solutions to improve intra mode coding. Two main technical solutions have been explored:

- Most Probable Modes (MPM) derivation, studied in JCTVC-F062 and JCTVC-F459

- Remaining mode coding, studied in JCTVC-F091, JCTVC-F106 and JCTVC-F269

It is proposed in CE6b description (JCTVC-F906, section 4.2.3) to test each one of these solutions, and also to explore combinations of these solutions. This proposal aims at developing a unified solution, taking benefit from both technical tracks.

The unified solution includes the following modifications:

- 2 additional MPMs (MPRMs) are added to the 2 MPMs already used in the current HM design. The encoding algorithm is adapted accordingly.

- Coding of remaining mode is based on mode ranking, applied to the complete set of modes. In addition,

  - unified binarization and VLC tables, derived from exp-Golomb codes, are used for HE and LC configurations;

  - CABAC coding is simplified by using bypass coding for MPRM index and remaining mode bins.

- Chroma coding is improved thanks to the use of 6 candidates, including the luma MPMs candidates.

- Mode 18 is added to the set of 4x4 blocks modes, resulting in 19 modes instead of 18.

These modifications are explained in more details in the following sections.

# Proposal description

## Addition of 2 MPRMs

2 MPMs, named MPRMs, are systematically added to the set of 2 MPMs (noted MPM1 and MPM2 in the sequel) already derived in the current HM design. To generate these two MPRMs, Table 1 (for Intra_4x4) and Table 2 (for Intra_8x8, Intra_16x16, and Intra_32x32) are used. Table 1 and Table 2 are composed of two closest adjacent modes of MPM. These 2 MPRMs are derived as follows.

Two closest adjacent modes of MPM1 (M1[0] and M1[1]) and two closest adjacent modes of MPM2 (M2[0] and M2[1]) are derived from Table 1 or Table 2 according to PU size.

The derivation of the 3rd MPM (MPM3) is as following process:

- If MPM1 is equal to Planar, MPM3 is set to equal to M2[1]

- Otherwise, MPM3 is set equal to Planar

The derivation of the 4rd MPM (MPM4) is as following process:

- If MPM1 is equal to Planar, MPM4 is set to equal to M2[0]

- Otherwise if MPM1 is not equal to M2[1], MPM4 is set equal to M2[1]

- Otherwise, MPM4 is set equal to M1[1]

Figure 1 shows the coding flow of adding two additional MPRMs. The intra mode coding is changed from one level in HM-4.0 to two levels. The first level is the same with the HM-4.0, and one more level of MPRM coding is introduced. If current intra mode (C) is not equal to one of the MPM1 or MPM2, the prev_pred_mode_flag will be set to zero, and the MPRM coding will be used. If current intra mode is equal to one of the MPM3 or MPM4, the mprm_pred_flag will be set to one and one mprm_idx will be use to indicate which MPM current mode is. Otherwise, the mprm_pred_flag will be set to zero and rem_intra_luma_pred_mode will be derived as shown in Sec. 2.2.

Figure 1.  The flow of adding two additional MPRMs

Table 1. Two closest adjacent modes of MPM for PU size equal to 4x4

| Mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPM_Minus_1 | 1 | 12 | 17 | 1 | 15 | 11 | 13 | 14 | 16 | 18 | 7 | 4 | 5 | 1 | 6 | 8 | 2 | 9 | 10 |
| MPM_Plus_1 | 2 | 13 | 16 | 2 | 11 | 12 | 14 | 10 | 15 | 17 | 18 | 5 | 1 | 6 | 7 | 4 | 8 | 2 | 9 |

Table 2. Two closest adjacent modes of MPM for PU size equal to 8x8, 16x16, or 32x32

| Mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPM_Minus_1 | 1 | 22 | 31 | 1 | 27 | 20 | 24 | 26 | 29 | 33 | 7 | 19 | 21 | 23 | 25 | 28 | 30 | 32 | 34 |
| MPM_Plus_1 | 2 | 23 | 30 | 2 | 19 | 21 | 25 | 10 | 28 | 32 | 34 | 20 | 22 | 24 | 26 | 27 | 29 | 31 | 33 |
| Mode | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |  |  |
| MPM_Minus_1 | 4 | 11 | 5 | 12 | 1 | 13 | 6 | 14 | 15 | 8 | 16 | 2 | 17 | 9 | 18 | 10 |  |  |  |
| MPM_Plus_1 | 11 | 5 | 12 | 1 | 13 | 6 | 14 | 7 | 4 | 15 | 8 | 16 | 2 | 17 | 9 | 18 |  |  |  |

The encoding process is modified to better exploit the usage of 4 MPMs. The MPMs are systematically inserted in the set of candidates that are checked using the full RDO algorithm.

## Mode ranking for remaining mode bins coding

Mode ranking is used to represent the distribution of all intra prediction modes. Mode ranking concept used in current CAVLC HM design is generalized for CABAC. Two ranking tables are used, one for 4x4 CUs and one for 8x8 to 32x32 CUs. The size of each table is equal to the number N of possible modes (N depending on the block size). Each entry of this table is a mode number, and the table gives its rank. The modes are initially ranked according to their values. For remaining mode decoding, instead of decoding a mode value as currently done in HM-4.0, its rank is decoded and possibly incremented depending on the MPMs/MPRMs ranks. Once the mode is decoded, it is swapped in the ranking table with its preceding mode, as already done in CAVLC.

The decoding process applies as described in Figure 1.

Figure 1: proposed intra mode decoding process.

## Remaining modes bins coding

### Unified binarization/VLC tables

As proposed in JCTVC-F091, the same binarization and VLC tables (provided in annex of this document), are used for both CABAC and CAVLC. These tables are derived from tables with limited size specifying for different rank ranges the codewords prefix and suffix. Depending on the rank range, different prefixes are used. The prefix consists of truncated unary (for 4x4 PUs case) or fixed length (for 8x8 to 32x32 PUs case) codes. Mode ranks within a specific range use the same prefix. Their suffix parts consist of fixed length codes, the length of which depends on the rank range. Table 3 and Table 4 specify, for respectively 4x4 PUs and 8x8 to 32x32 PUs, the prefix and FL suffix length for the different rank ranges.

In the most general case, the decoder needs to know the maximum symbol value for the prefix part (that uses truncated unary codes or unary codes), and the number of bits to parse for each of the symbols. For example, the maximum symbol for the unary prefix is 2 for 4x4 PU sizes and 3 for the rest of the PU sizes. And in the case of 4x4 PU sizes, the number of bits to parse is 2, 2, and 3 for prefix symbol of 0, 2, and 3, respectively. This ensures that both the decoder and the encoder do not rely on large VLC tables and only require a small amount of storage space to be able to entropy code/decode the Intra modes.

Table 3. Unified binarization/VLC table for 4x4 PUs.

| Prefix symbol | Rank range | prefix | suffix | codeword |
| --- | --- | --- | --- | --- |
| 0 | 0-3 | 0 | xx | 0xx |
| 1 | 4-7 | 10 | xx | 10xx |
| 2 | 8-14 | 11 | xxx | 11xx |

Table 4. Unified binarization/VLC table for 8x8 to 32x32 PUs.

| Prefix symbol | Rank range | prefix | suffix | codeword |
| --- | --- | --- | --- | --- |
| 0 | 0-1 | 00 | x | 00 x |
| 1 | 2-5 | 01 | xx | 01 xx |
| 2 | 6-13 | 10 | xxx | 10 xxx |
| 3 | 14-28 | 11 | xxxx | 11 xxxx |
| 3 | 29 | 11 | 11110 | 11 11110 |
| 3 | 30 | 11 | 11111 | 11 11111 |

### CABAC replaced by bypass coding

In addition, for CABAC, MPRMs index and remaining mode bins coding is simplified, similarly as in JCTVC-F376 [5], using bypass coding. Bin of MPRMs index and all bins of remaining mode are coded by bypass coding.

## Extension to 19 modes for 4x4 CUs

In the current HM design, 4x4 CUs make use of 18 modes, excluding mode 18, inserted between modes 9 and 10 (see Figure 2). Using 18 modes in current HM design enables to code the 16 (18-2) remaining modes with 4 bins FLC. When non-FLC binarization table is used, this limitation is no more useful. So in this proposal, mode 18 is added to the set of 4x4 modes.

Figure 2: intra coding modes.

## Improved chroma coding

In the proposed method, there are six available chroma intra prediction modes (IntraPredModeC) that can be signaled. Chroma predicted from luma signal (LM) and reuse of the luma intra prediction mode (DM) remain unchanged. When the luma intra prediction mode (luma_pred_mode) is not planar mode or DC mode, the derivation of remaining chroma intra prediction modes are modified as follow:

- Planar mode is assigned to intra_chroma_pred_mode[0],

- MPM_Minus_1[luma_pred_mode] is assigned to intra_chroma_pred_mode[1],

- MPM_Plus_1 [luma_pred_mode] is assigned to intra_chroma_pred_mode[2],

- PredMode_P [luma_pred_mode] is assigned to intra_chroma_pred_mode[3].

In the version cross-checked by Sharp (JCTVC-G311), PredMode_P[x] is derived from the following tables.

Table 5. Perpendicular modes of intra_pred_modes for PU size equal to 4x4

| Mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PredMode_P | 0 | 2 | 1 | 0 | 7 | 9 | 8 | 4 | 6 | 5 | 4 | 9 | 17 | 16 | 15 | 14 | 13 | 12 | 5 |

Table 6. Perpendicular modes of intra_pred_modes for PU size equal to 8x8, 16x16, or 32x32

| Mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PredMode_P | 0 | 2 | 1 | 0 | 7 | 9 | 8 | 4 | 6 | 5 | 4 | 9 | 17 | 16 | 15 | 14 | 13 | 12 | 11 |
| Mode | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |  |  |
| PredMode_P | 34 | 33 | 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 |  |  |  |

In these tables, mode 18 is not handled in the same way for 4x4 and 8x8 to 32x32 cases. The following unified table should be preferably used for both cases. This new table gives similar BD-rate performance as the cross-checked version.

Table 7. Perpendicular modes of intra_pred_modes

| Mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PredMode_P | 0 | 2 | 1 | 0 | 7 | 9 | 8 | 4 | 6 | 5 | 4 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 |
| Mode | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |  |  |  |
| PredMode_P | 34 | 33 | 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 |  |  |  |

# Results

The unified solution has been evaluated using test conditions of JCTVC-F900 [7]. HM4.0 with LM bug fix has been used as reference. The following table summarizes the BD-rate performance on Intra only configuration. It should be mentioned that the version cross-checked by Sharp (JCTVC-G311) uses slightly less regular binarization/VLC tables than those described in section 2.3.1. The more regular version given in Table 3 and Table 4 leads to same BD-rate.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | 0.0% | -0.1% | -0.3% | -0.3% | -0.3% |
| Class B | -0.4% | -0.5% | -0.5% | -0.4% | -0.7% | -0.8% |
| Class C | -0.5% | -0.8% | -0.8% | -0.5% | -1.0% | -0.9% |
| Class D | -0.4% | -0.5% | -0.6% | -0.3% | -0.6% | -0.7% |
| Class E | -0.6% | -0.3% | -0.5% | -0.6% | -0.8% | -0.8% |
| Overall | -0.5% | -0.4% | -0.5% | -0.4% | -0.7% | -0.7% |
|  | -0.5% | -0.4% | -0.5% | -0.4% | -0.7% | -0.7% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Full results are provided in attached xls files. Three files are provided:

- JCTVC-G243_CE6b_HM4.0_UnifIMC.xls: HM4.0 as reference; version cross-checked by Sharp

- JCTVC-G243_CE6b_HM4.0WithLMBugFix_UnifIMC.xls: LM Bug fix is used in HM4.0 and proposal; version cross-checked by Sharp

- JCTVC-G243_CE6b_HM4.0WithLMBugFix_UnifIMC_v1.xls: LM Bug fix is used in HM4.0 and proposal; more regular VLC/bin tables and unified chroma mode table are used.

It can be observed that very similar results are obtained in the three cases.

# Comments and conclusion

This proposal aims at combining two tools for intra mode coding, use of more than 2 MPMs, and use of mode ranking. A deeper analysis of the independent gain of each tool shows that the gains are partly cumulative:

- Using 2 MPMs only (without any of the other modifications described in this paper) brings around 0.3% Y gain.

- Using mode ranking only leads to around 0.2% Y gain (see JCTVC-G242).

- Using the improved encoding process gives 0.05% gain.

- Chroma coding improvement is orthogonal.

- The use of a 19th mode for 4x4 block brings minor gain, but enables a more coherent design.

Even if these different gains are not fully additive, the total gain from the unified solution is around 0.4-0.5%, showing a mostly additive benefit from this combination.

# References

- T.-D. Chuang, C.-Y. Chen, M. Guo, X. Guo, Y.-W. Huang, S. Lei, “Luma Intra Prediction Mode Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F062, 6th Meeting: Torino, Italy, July, 2011.

- E. Maani, A. Tabatabai, “Unifying binarizations of Intra modes in HE and LC”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F091, 6th Meeting: Torino, Italy, July, 2011.

- J. Park, B. Jeon, “CAVLC coding for intra prediction mode”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F106, 6th Meeting: Torino, Italy, July, 2011.

- E. Francois, S. Pautet, C. Gisquet, “Modified Intra Mode Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F269, 6th Meeting: Torino, Italy, July, 2011.

- V. Seregin, I.-K Kim, “Utilisation of CABAC equal probability mode for intra mode coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F376, 6th Meeting: Torino, Italy, July, 2011.

- W.-J. Chien, X. Wang, M.Karczewicz, “Parsing friendly intra mode coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F459, 6th Meeting: Torino, Italy, July, 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, Italy, July, 2011.

# Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).